3GPP TSG-RAN5 Meeting #2020-TTCN email	R5s210274

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4591 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_EUTRA_ReturnMpdcchPNB |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test case 7.2.3.15 fails for Cat-M1 UE when configured with 5MHz bandwidth due to conflict between scheduling data using NB#1 at subframe#9 and MIB scheduling (Table 7.3.3.9.1-1, TS36.523-3). MIB is allowed to be repeated in subframe#9 for FDD, TS36.331,cl.5.2.1.2. Hence, conflict between NB#1 and repeated MIB araises in subframe#9. For Bandwidth more than 5MHz(ie 10MHz), this is not an issue since NB1 does not conflict with MIB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | For 7.2.3.15, when bandwidth is 5MHz mpdcch_NarrowbandsToMonitor value is changed from 2 to 4 i.e from NB#1 to NB#3 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant cat-M UE will fail the test case with BW 5Mhz |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.15 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_EUTRA_ReturnMpdcchPNB |
| --- | --- |
| Reason for change | Test case 7.2.3.15 fails for Cat-M1 UE when configured with 5MHz bandwidth due to conflict between scheduling data using NB#1 at subframe#9 and MIB scheduling (Table 7.3.3.9.1-1, TS36.523-3). MIB is allowed to be repeated in subframe#9 for FDD, TS36.331,cl.5.2.1.2. Hence, conflict between NB#1 and repeated MIB araises in subframe#9. For Bandwidth more than 5MHz(ie 10MHz), this is not an issue since NB1 does not conflict with MIB. |
| Summary of change | For 7.2.3.15, when bandwidth is 5MHz mpdcch_NarrowbandsToMonitor value is changed from 2 to 4 i.e from NB#1 to NB#3 |
| TTCN module | Common\EUTRA\EUTRA_CellInfoInit.ttcn |
| MCC160 Comment | Postponed Test model changes to TS 36.523-3 clause 7.3.3.9 must first be discussed at RAN5 TTCN Workshop #53. |

Before change:

…

function f_EUTRA_ReturnMpdcchPNB(Dl_Bandwidth_Type p_DL_ChBandwidth,

SysinfoType_Type p_SysinfoType,

PhysCellId p_PhysCellId)

return Mpdcch_NarrowbandsToMonitor_r13_Type

{ //@sic R5-171566: added p_PhysCellId sic@

//Acc to 36.336 mpdcch-NarrowbandsToMonitor => Narrowbands to monitor for MPDCCH for RAR, see TS 36.213 [23, 6.2]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21].

var Mpdcch_NarrowbandsToMonitor_r13_Type v_Mpdcch_NarrowbandsToMonitor;

if (p_SysinfoType == sysinfoAndBR) { // Initiatile parameters for CE Mode A or B

select(p_DL_ChBandwidth) {

case (n25) { //5MHz

// 36.523-3 Table 7.3.3.9.1-2: Narrow band allocation per cell (5 MHz)

v_Mpdcch_NarrowbandsToMonitor := {2} //2 => NB1 @sic R5s170090 change 1.3 sic@

}

case (n50) { //10MHz

//36.523-3 Table 7.3.3.9.1-4: Narrow band allocation per cell (10 MHz)

select ( (p_PhysCellId mod 6)) {

case (0, 2, 3, 5) { v_Mpdcch_NarrowbandsToMonitor := {2} } //2 => NB1 @sic R5s170090 sic@

case (1, 4) { v_Mpdcch_NarrowbandsToMonitor := {3} }   //3 => NB2 @sic R5s170090 change 1.3 sic@

case else { FatalError(__FILE__, __LINE__, "invalid cellId for eMTC");}

}

}

case (n100) {//20MHz

//36.523-3 Table 7.3.3.9.1-6: Narrow band allocation per cell (20 MHz)

v_Mpdcch_NarrowbandsToMonitor := {8}  //8 => NB7 @sic R5s170090 change 1.3 sic@

}

case else { FatalError(__FILE__, __LINE__, "invalid value for ChannelBandwidth");}

}

}

return v_Mpdcch_NarrowbandsToMonitor;

}

....

After change:

…

function f_EUTRA_ReturnMpdcchPNB(Dl_Bandwidth_Type p_DL_ChBandwidth,

SysinfoType_Type p_SysinfoType,

PhysCellId p_PhysCellId)

return Mpdcch_NarrowbandsToMonitor_r13_Type

{ //@sic R5-171566: added p_PhysCellId sic@

//Acc to 36.336 mpdcch-NarrowbandsToMonitor => Narrowbands to monitor for MPDCCH for RAR, see TS 36.213 [23, 6.2]. Field values (1..maxAvailNarrowBands-r13) correspond to narrowband indices (0..[maxAvailNarrowBands-r13-1]) as specified in TS 36.211 [21].

var Mpdcch_NarrowbandsToMonitor_r13_Type v_Mpdcch_NarrowbandsToMonitor;

var charstring v_Testcase;  // Anritsu TTCN WA for 7.2.3.15

if (p_SysinfoType == sysinfoAndBR) { // Initiatile parameters for CE Mode A or B

select(p_DL_ChBandwidth) {

case (n25) { //5MHz

// 36.523-3 Table 7.3.3.9.1-2: Narrow band allocation per cell (5 MHz)

v_Testcase := testcasename(); // Anritsu TTCN WA for 7.2.3.15

if (v_Testcase == "TC_7_2_3_15" and px_ePrimaryBandChannelBandwidth == n25) {

v_Mpdcch_NarrowbandsToMonitor := {4} // Anritsu TTCN WA for 7.2.3.15

}

else{

v_Mpdcch_NarrowbandsToMonitor := {2} //2 => NB1 @sic R5s170090 change 1.3 sic@

}

}

case (n50) { //10MHz

//36.523-3 Table 7.3.3.9.1-4: Narrow band allocation per cell (10 MHz)

select ( (p_PhysCellId mod 6)) {

case (0, 2, 3, 5) { v_Mpdcch_NarrowbandsToMonitor := {2} } //2 => NB1 @sic R5s170090 sic@

case (1, 4) { v_Mpdcch_NarrowbandsToMonitor := {3} }   //3 => NB2 @sic R5s170090 change 1.3 sic@

case else { FatalError(__FILE__, __LINE__, "invalid cellId for eMTC");}

}

}

case (n100) {//20MHz

//36.523-3 Table 7.3.3.9.1-6: Narrow band allocation per cell (20 MHz)

v_Mpdcch_NarrowbandsToMonitor := {8}  //8 => NB7 @sic R5s170090 change 1.3 sic@

}

case else { FatalError(__FILE__, __LINE__, "invalid value for ChannelBandwidth");}

}

}

return v_Mpdcch_NarrowbandsToMonitor;

}