3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210281

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0672 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IMS test case 9.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. From 34.229-5, TP of 9.3 indicates storing and sending 3 segments of concatenated SMS as SIP MESSAGE request. 2. If only one stored message, use corresponding AT command from cs_AT_SEND_STORED_ONEMESSAGE. If three stored messages, use corresponding AT command from cs_AT_SEND_STORED_3_MESSAGES. 3. Distinguish AT command to be used for one stored SMS and 3 stored SMS. 4. Currently, AT+CSCA parameter is hardcoded to 2222222222 for setting service center address. Change AT+CSCA parameter according to PIXIT indication. Otherwise rp_DestinationAddress will be a mismatch. According to 34.229-1 A.7.7, default value of RP-DestinationAddress should be 112233445566778. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Add input parameter 3 to f_IMS_SMS_AT_Trigger_MO_SMS |  |  |  |  |  |  |  |  |
|  |  | 2. Add condition to check number of stored SMs messages. 3. Define new function f_IMS_3_SMS_AT_CMSS 4. Change input parameter of f_IMS_SMS_AT_CSCA to px_SMS_SMSC_InternationalNumber |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1 Change 1	3

2.2 Change 2	4

2.3 Change 3	5

2.4 Change 4	5

2.5 Change 5	6

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_IMS_NR5GC_MobileOriginatingConcatenatedSMS |
| --- | --- |
| Reason for change | From 34.229-5, TP of 9.3 indicates storing and sending 3 segments of concatenated SMS as SIP MESSAGE request. See related change in change 2 for function f_IMS_SMS_AT_Trigger_MO_SMS. |
| Summary of change | Add input parameter 3 to f_IMS_SMS_AT_Trigger_MO_SMS |
| TTCN module | IMS_NR5GC_CC_TestcaseImplementation_9.ttcn |
| MCC160 Comment | Rejected. The present changes 1-3 propose to send multiple SMS instead of 1 concatenated SMS. See TC 11.1.5 of 36.523-1 for example. The prose of TC 11.1.5 of 36.523-1 has been modified in the past: it had originally spoken of “concatenated SMS” and this was changed to “multiple SMS”. The TPDU structure is different. See also additional change from TF160 below. |

Before change:

function f_IMS_NR5GC_MobileOriginatingConcatenatedSMS() runs on IMS_PTC

{

var O1_Type v_MsgRef; // take value chosen by the UE

var integer v_SN_Num := 1; // initially (first segment) 1

var template RP_DATA_ul_Type v_SMS_RP_DATA_ul_SUBMIT_IMS;

// configure the SMS environment using the necessary AT commands

f_IMS_SMS_AT_Config();

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

// UE is made to send a Concatenated SMS over IP (The length of SMS text is determined so that the amount of segments of the concatenated SMS is three).

// trigger one overlong SMS of 3 segments of 134 octets / 153 7bit characters each

f_IMS_SMS_AT_Trigger_MO_SMS (113, 415, tsc_FoxShort & tsc_FoxShort & tsc_FoxShort);

…

}

After change:

function f_IMS_NR5GC_MobileOriginatingConcatenatedSMS() runs on IMS_PTC

{

var O1_Type v_MsgRef; // take value chosen by the UE

var integer v_SN_Num := 1; // initially (first segment) 1

var template RP_DATA_ul_Type v_SMS_RP_DATA_ul_SUBMIT_IMS;

// configure the SMS environment using the necessary AT commands

f_IMS_SMS_AT_Config();

f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling);

f_IMS_TestBody_Set(true);

// @siclog "Step 1" siclog@

// UE is made to send a Concatenated SMS over IP (The length of SMS text is determined so that the amount of segments of the concatenated SMS is three).

// trigger one overlong SMS of 3 segments of 134 octets / 153 7bit characters each

f_IMS_SMS_AT_Trigger_MO_SMS (113, 415, tsc_FoxShort & tsc_FoxShort & tsc_FoxShort, 3);

…

}

## 2.2 Change 2

| Function name | f_IMS_SMS_AT_Trigger_MO_SMS |
| --- | --- |
| Reason for change | If only one stored message, use corresponding AT command from cs_AT_SEND_STORED_ONEMESSAGE. If three stored messages, use corresponding AT command from cs_AT_SEND_STORED_3_MESSAGES. See related change in change 3 for new function f_IMS_3_SMS_AT_CMSS. |
| Summary of change | Add condition to check number of stored SMs messages. |
| TTCN module | IMS_SMS_Messages.ttcn |
| MCC160 Comment | Not needed. See change 1 |

Before change:

function f_IMS_SMS_AT_Trigger_MO_SMS (integer p_CSMP,

integer p_PDU_len,

charstring p_SMS_text) runs on IMS_PTC

{

// Write message to storage. Takes next index automatically.

f_IMS_SMS_AT_Write_MO_SMS(p_CSMP, p_PDU_len, p_SMS_text);

// Send message from storageSMS_AT_relatedFunctionsSMS_AT_relatedFunctions.

f_IMS_SMS_AT_CMSS(px_SMS_IndexOffset);

}

After change:

function f_IMS_SMS_AT_Trigger_MO_SMS (integer p_CSMP,

integer p_PDU_len,

charstring p_SMS_text,

integer p_NumOfSMS) runs on IMS_PTC

{

// Write message to storage. Takes next index automatically.

f_IMS_SMS_AT_Write_MO_SMS(p_CSMP, p_PDU_len, p_SMS_text);

// Send message from storageSMS_AT_relatedFunctionsSMS_AT_relatedFunctions.

if(p_NumOfSMS == 1) {f_IMS_SMS_AT_CMSS(px_SMS_IndexOffset);}

else if(p_NumOfSMS == 3) {f_IMS_3_SMS_AT_CMSS(px_SMS_IndexOffset);}

}

## 2.3 Change 3

| Function name | f_IMS_3_SMS_AT_CMSS |
| --- | --- |
| Reason for change | Distinguish AT command to be used for one stored SMS and 3 stored SMS. |
| Summary of change | Define new function f_IMS_3_SMS_AT_CMSS |
| TTCN module | IMS_SMS_Messages.ttcn |
| MCC160 Comment | Not needed. See change 1 |

New function:

// Anritsu TTCN CR required for 9.3

function f_IMS_3_SMS_AT_CMSS(integer p_Index) runs on IMS_PTC

{ // @sic R5s110091 sic@

f_UT_SendStored_3_Messages(MMI, p_Index);

}

## 2.4 Change 4

| Function name | f_IMS_SMS_AT_Config |
| --- | --- |
| Reason for change | Currently, AT+CSCA parameter is hardcoded to 2222222222 for setting service center address. Change AT+CSCA parameter according to PIXIT indication. Otherwise rp_DestinationAddress will be a mismatch. According to 34.229-1 A.7.7, default value of RP-DestinationAddress should be 112233445566778. |
| Summary of change | Change input parameter of f_IMS_SMS_AT_CSCA to px_SMS_SMSC_InternationalNumber |
| TTCN module | IMS_SMS_Messages.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_IMS_SMS_AT_Config() runs on IMS_PTC

{

// Set message service as indicated in the PIXIT.

f_IMS_SMS_AT_CSMS(px_SMS_Service);

// Set Preferrred memory to preferred memories as indicated in the PIXIT.

f_IMS_SMS_AT_CPMS(px_SMS_PrefMem1, px_SMS_PrefMem2, px_SMS_PrefMem3);

// Set message format to value given as PIXIT parameter

f_IMS_SMS_AT_CMGF(px_SMS_MsgFrmt);  // @sic R5s110751 sic@

// Set Character Set "GSM"

f_IMS_SMS_AT_CSCS("""GSM""");

// Set the UE to delete all messages from Preferred message store

f_IMS_SMS_AT_CMGD_ALL();

// Set service center address

f_IMS_SMS_AT_CSCA("""2222222222""", 129);

}

After change:

function f_IMS_SMS_AT_Config() runs on IMS_PTC

{

// Set message service as indicated in the PIXIT.

f_IMS_SMS_AT_CSMS(px_SMS_Service);

// Set Preferrred memory to preferred memories as indicated in the PIXIT.

f_IMS_SMS_AT_CPMS(px_SMS_PrefMem1, px_SMS_PrefMem2, px_SMS_PrefMem3);

// Set message format to value given as PIXIT parameter

f_IMS_SMS_AT_CMGF(px_SMS_MsgFrmt);  // @sic R5s110751 sic@

// Set Character Set "GSM"

f_IMS_SMS_AT_CSCS("""GSM""");

// Set the UE to delete all messages from Preferred message store

f_IMS_SMS_AT_CMGD_ALL();

// Set service center address

f_IMS_SMS_AT_CSCA(px_SMS_SMSC_InternationalNumber, 129);

}

## 2.5 Change 5

| Function name | f_NG_SMS_AT_Config |
| --- | --- |
| Reason for change | Currently, AT+CSCA parameter is hardcoded to 2222222222 for setting service center address. Change AT+CSCA parameter according to PIXIT indication. Otherwise rp_DestinationAddress will be a mismatch. According to 34.229-1 A.7.7, default value of RP-DestinationAddress should be 112233445566778. |
| Summary of change | Change input parameter of f_IMS_SMS_AT_CSCA to px_SMS_SMSC_InternationalNumber |
| TTCN module | SMSoverNAS_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before change:

function f_NG_SMS_AT_Config() runs on NR_BASE_PTC

{

// Set message service as indicated in the PIXIT.

f_NG_SMS_AT_CSMS(px_SMS_Service);

// Set Preferrred memory to preferred memories as indicated in the PIXIT.

f_NG_SMS_AT_CPMS(px_SMS_PrefMem1, px_SMS_PrefMem2, px_SMS_PrefMem3);

// Set message format to value given as PIXIT parameter

f_NG_SMS_AT_CMGF(px_SMS_MsgFrmt);  // @sic R5s110751 sic@

// Set Character Set "GSM"

f_NG_SMS_AT_CSCS("""GSM""");

// Set the UE to delete all messages from Preferred message store

f_NG_SMS_AT_CMGD_ALL();

// Set service center address

f_NG_SMS_AT_CSCA("""2222222222""", 129);

}

After change:

function f_NG_SMS_AT_Config() runs on NR_BASE_PTC

{

// Set message service as indicated in the PIXIT.

f_NG_SMS_AT_CSMS(px_SMS_Service);

// Set Preferrred memory to preferred memories as indicated in the PIXIT.

f_NG_SMS_AT_CPMS(px_SMS_PrefMem1, px_SMS_PrefMem2, px_SMS_PrefMem3);

// Set message format to value given as PIXIT parameter

f_NG_SMS_AT_CMGF(px_SMS_MsgFrmt);  // @sic R5s110751 sic@

// Set Character Set "GSM"

f_NG_SMS_AT_CSCS("""GSM""");

// Set the UE to delete all messages from Preferred message store

f_NG_SMS_AT_CMGD_ALL();

// Set service center address

f_NG_SMS_AT_CSCA(px_SMS_SMSC_InternationalNumber, 129);

}

Additional change from MCC160

/*

* @desc      Transmission of AT Command series to trigger 3 multiple short messages p_Index, +1, +2

* @param     p_Index

* @status    APPROVED (LTE)

*/

function f_SMS_AT_3_SMS_Multiple(integer p_Index) runs on EUTRA_PTC

{

f_UT_SendStored_3_Messages(UT, p_Index);

}

…

function fl_TC_11_1_5_Body() runs on EUTRA_PTC

{

var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get();

var SRB_COMMON_IND v_ReceivedAsp;

var NAS_MSG_Indication_Type v_NasInd;

var NAS_MessageContainer v_NAS_RcvMsgContainer_SMS;

var template (value) NAS_MessageContainer v_NAS_TxMessageContainer_SMS;

var CP_PDU_Type v_RcvSMS_CP_PDU;

var template CP_PDU_Type v_ExpSMS_CP_PDU;

var bitstring v_NAS_RcvBitsMsgContainer_SMS;

var B3_Type v_TI_Value;

var TransactionIdentifier_Type v_MO_TI_FromSS;

var TransactionIdentifier_Type v_MO_TI_FromUE;

var TransactionIdentifier_Type v_MO_TI_FromUE_Previous;

var O1_Type v_RpMsgRefRcvd;

var O4_Type v_NasCountUL;

var EUTRA_SecurityParams_Type v_AuthParams := f_EUTRA_Security_Get();

var template (value) EUTRA_FDD_TDD_CellInfo_Type  v_EUTRA_FDD_TDD_Info;

var template (value) PhysicalConfigDedicated_AntennaInfo_Type  v_AntennaInfo;

//@siclog "Step 1" siclog@

// Sending of 3 MO SMS as multiple SMS is initiated at the UE via MMI or AT command

if (px_SMS_MsgFrmt=="1") {

// Set text mode parameters to trigger status report.

// bit 7(RP) 6(UDHI) 5(SRR) 4 3(VPF) 2(RD) 1 0(MIT)

//     0     0       1      1 0      0     0 1

f_SMS_AT_CSMP(49, 167, 0);  // @sic R5s100773 sic@

// Write message with index 0 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

// Write message with index 1 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

// Write message with index 2 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

}

else { // PDU mode

// Write message with index 0 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

// Write message with index 1 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

// Write message with index 2 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

}

// Send 3 multiple messages from storage. Numbered px_SMS_IndexOffset, +1 , +2

f_SMS_AT_3_SMS_Multiple(px_SMS_IndexOffset);

//@siclog "Step 2" siclog@

…

function fl_TC_11_1_6_Body() runs on EUTRA_PTC

{

var SRB_COMMON_IND v_ReceivedAsp;

var NAS_MessageContainer v_NAS_RcvMsgContainer_SMS;

var template (value) NAS_MessageContainer v_NAS_TxMessageContainer_SMS;

var CP_PDU_Type v_RcvSMS_CP_PDU;

var template CP_PDU_Type v_ExpSMS_CP_PDU;

var bitstring v_NAS_RcvBitsMsgContainer_SMS;

var B3_Type v_TI_Value;

var TransactionIdentifier_Type v_MO_TI_FromSS;

var TransactionIdentifier_Type v_MO_TI_FromUE;

var TransactionIdentifier_Type v_MO_TI_FromUE_Previous;

var O1_Type v_RpMsgRefRcvd;

//@siclog "Step 1" siclog@

// Sending of 3 MO SMS as multiple SMS is initiated at the UE via MMI or AT command

if (px_SMS_MsgFrmt=="1") {

// Set text mode parameters to trigger status report.

// bit 7(RP) 6(UDHI) 5(SRR) 4 3(VPF) 2(RD) 1 0(MIT)

//     0     0       1      1 0      0     0 1

f_SMS_AT_CSMP(49, 167, 0);  // @sic R5s100773 sic@

// Write message with index 0 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

// Write message with index 1 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

// Write message with index 2 to memory storage

f_SMS_AT_CMGW_Text("""1111111111""", 129, tsc_Fox);

}

else { // PDU mode

// Write message with index 0 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

// Write message with index 1 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

// Write message with index 2 to memory storage

f_SMS_AT_CMGW_PDU(153, tsc_Fox_PDU);

}

// Send 3 multiple messages from storage. Numbered px_SMS_IndexOffset, +1 , +2

f_SMS_AT_3_SMS_Multiple(px_SMS_IndexOffset);

//@siclog "Step 2" siclog@

…