3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210285

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4595 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NBIOT_InitFrequency_f1Tof3_MFBI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-02-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | When the testcase 22.2.10 is run with px_MFBI_FrequencyBand=2 and px_OverlappingNotSupportedFrequencyBand_MFBI=25, the TTCN assigns Ncell 3 with EARFCN as 8689 (DL) and 26689 (UL). This doesn’t meet the pre-test conditions specified in 36.523-1 clause 22.2.10.3.1 which says “Ncell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.” The EARFCNs 8689 and 26689 are legitimate for Band 25, but not for Band 2. The maximum EARFCN that can be used is 8639 (DL) and 26639 (UL) which overlaps both Bands 2 and 25. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Ncell 3 is now assigned EARFCNs 8639 (DL) and 26639 (UL) when the testcase is run with IXITs setting px_MFBI_FrequencyBand=2 and px_OverlappingNotSupportedFrequencyBand_MFBI=25. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not in line with Prose and thus a conformant UE may fail the testcase |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.2.10 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS36.508 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D20wk50 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | f_NBIOT_InitFrequency_f1Tof3_MFBI |
| --- | --- |
| Reason for change | When the testcase 22.2.10 is run with px_MFBI_FrequencyBand=2 and px_OverlappingNotSupportedFrequencyBand_MFBI=25, the TTCN assigns Ncell 3 with EARFCN as 8689 (DL) and 26689 (UL). This doesn’t meet the pre-test conditions specified in 36.523-1 clause 22.2.10.3.1 which says “Ncell 3 belongs to another absolute centre frequency which again overlaps between bands controlled by IXITs px_MFBI_FrequencyBand and px_OverlappingNotSupportedFrequencyBandMFBI.” The EARFCNs 8689 and 26689 are legitimate for Band 25, but not for Band 2. |
| Summary of change | Ncell 3 is now assigned EARFCNs 8639 (DL) and 26639 (UL) when the testcase is run with IXITs setting px_MFBI_FrequencyBand=2 and px_OverlappingNotSupportedFrequencyBand_MFBI=25. |
| TTCN module | NBIOT_Idle_CellReselection.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#90-e |

Before Change:

function f_NBIOT_InitFrequency_f1Tof3_MFBI(integer p_Band, integer p_OverlapingBand, NB_OperationMode_Type p_NB_OperationMode) return NBIOT_Frequency_fList_Type { var NBIOT_FrequencyLowMidHigh_Type v_FreqLMH; var template(value) NBIOT_Frequency_fList_Type v_Freq := cs_NBIOT_FrequencyInitDummy; //Initialise with Dummy value // Initialise Frequencies Low/Mid/High for the primary Band v_FreqLMH:= f_NBIOT_InitFrequencyLowMidHigh(p_Band, p_NB_OperationMode ); --------Skipped----- case (19) { // Band 19 select (p_OverlapingBand) { case (5, 26) { } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case (25) { // Band 25 select (p_OverlapingBand) { case (2) { } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case (26) { // Band 26 select (p_OverlapingBand) { case (5, 18, 19) { } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case else { FatalError(__FILE__, __LINE__, "invalid Band"); } } return valueof(v_Freq); }

After Change:

function f_NBIOT_InitFrequency_f1Tof3_MFBI(integer p_Band, integer p_OverlapingBand, NB_OperationMode_Type p_NB_OperationMode) return NBIOT_Frequency_fList_Type { var NBIOT_FrequencyLowMidHigh_Type v_FreqLMH; var template(value) NBIOT_Frequency_fList_Type v_Freq := cs_NBIOT_FrequencyInitDummy; //Initialise with Dummy value // Initialise Frequencies Low/Mid/High for the primary Band v_FreqLMH:= f_NBIOT_InitFrequencyLowMidHigh(p_Band, p_NB_OperationMode ); --------Skipped----- case (19) { // Band 19 select (p_OverlapingBand) { case (5, 26) { } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case (25) { // Band 25 select (p_OverlapingBand) { case (2) { v_Freq.f2 := cs_NB_CarrierFreq (8639, v_0dot5, 26639, v0); } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case (26) { // Band 26 select (p_OverlapingBand) { case (5, 18, 19) { } case else { FatalError(__FILE__, __LINE__, "invalid overlaping Band"); } } } case else { FatalError(__FILE__, __LINE__, "invalid Band"); } } return valueof(v_Freq); }