3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210296

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4597 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NBIOT testcase 22.5.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, NB_IOT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-03 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 23 of NBIOT 22.5.17 TC SS transmits a Tracking Area Update Accept message, for that purpose different templates are used. In cs_TAU_Accept_Common p_AdditionalUpdateResult input parameter, variable v_NasInd is used where was latest updated in step 2-4b1, this means that we are not using the updated input parameter required, as this input parameter should be based in Tracking Area Update Request message received at step 22 of the test case. This can be solved using latest response received in step 22 for cs_TAU_Accept_Common as p_AdditionalUpdateResult input parameter,. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Used as p_AdditionalUpdateResult for TAU Accept, value based in TAU Request received in step 22. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 22.5.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function fl_TC_22_5_17_Body	4

# Corrections required

## Correction to function fl_TC_22_5_17_Body

| Function name | fl_TC_22_5_17_Body |
| --- | --- |
| Reason for change | In cs_TAU_Accept_Common p_AdditionalUpdateResult input parameter, variable v_NasInd is used where was latest updated in step 2-4b1, this means that we are not using the updated input parameter required, as this input parameter should be based in Tracking Area Update Request message received at step 22 of the test case. |
| Summary of change | Used as p_AdditionalUpdateResult for TAU Accept, value based in TAU Request received in step 22. |
| TTCN module | NBIOT/22_5/NBIOT_NAS_Attach.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 22"); v_EPS_ContextStatus := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Send this back in the Accept //@siclog "Step 23" siclog@ // The SS transmits a TRACKING AREA UPDATE ACCEPT message including GUTI-1. SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_TAU_Accept_Common(tsc_EpsUpdate_TaUpdate, f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti_Params1), cds_TAIListNonConsecutive_tlv(v_PLMN, { bit2oct(v_Tac) }), v_EPS_ContextStatus, omit, omit, f_GetNBIOTAdditionalUpdateResult(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), -, -, -, f_GetNBIOTNetworkFeatureSupport(CONTROL_PLANE), -, -, cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B), // @sic R5-199012 sic@ v_T3324Value1Min)))); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> f_NBIOT_PreliminaryPass(__FILE__, __LINE__, "Step 22"); v_EPS_ContextStatus := v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; // Send this back in the Accept //@siclog "Step 23" siclog@ // The SS transmits a TRACKING AREA UPDATE ACCEPT message including GUTI-1. SRB.send(cas_NB_SRB_NasPdu_REQ(nbiot_Cell1, tsc_SRB1bis, cs_TimingInfo_Now, cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, cs_TAU_Accept_Common(tsc_EpsUpdate_TaUpdate, f_GutiParameters2MobileIdentity(tsc_IEI_Guti, v_Guti_Params1), cds_TAIListNonConsecutive_tlv(v_PLMN, { bit2oct(v_Tac) }), v_EPS_ContextStatus, omit, omit, f_GetNBIOTAdditionalUpdateResult(v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.addUpdateType), -, -, -, f_GetNBIOTNetworkFeatureSupport(CONTROL_PLANE), -, -, cr_GPRS_Timer2_3('5E'O, tsc_Gprs3TimerUnit_1min, '00100'B), // @sic R5-199012 sic@ v_T3324Value1Min)))); <<SKIPPED CODE>>