3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210315

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1613 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_NR_RRC_SetupRequest_Common |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per clause 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases, and in the TTCN code, the TBS of UL grant is set to 4992. If the UE has long UE information (> 4992 *2), then the UE needs to send three SRs to ask for enough UL grant. But the duration will be 10.240s * 3 = 30.72s, the Timer T3510 will expire before all UE information is sent out. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Increase the TBS of UL grant from 4992 to 12808 to make sure all UE information will be sent out. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | All layer 2 test cases, which set the value of retxBSR-Timer to default sf10240. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

# Corrections required

| Function name | f_NR_RRC_SetupRequest_Common |
| --- | --- |
| Reason for change | As per clause 7.1.0 of 38523-1. The default value of retxBSR-Timer is set to sf10240 for all layer 2 test cases, and in the TTCN code, the TBS of UL grant is set to 4992. If the UE has long UE information (> 4992 *2), then the UE needs to send three SRs to ask for enough UL grant. But the duration will be 10.240s * 3 = 30.72s, the Timer T3510 will expire before all UE information is sent out. |
| Summary of change | Increase the TBS of UL grant from 4992 to 12808 to make sure all UE information will be sent out. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted in principle but implemented differently by configuring the specific layer 2 BSR settings when bringing UE into RRC_Connected state after the generic NR Idle preamble – see proposed implementation below |

Before change:

…

function f_NR_RRC_SetupRequest_Common(NR_CellId_Type p_NR_CellId,

timer p_Timer,

template  EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ?)

runs on NR_BASE_PTC return boolean

{ /* common procedure to receive the initial RRC setup request for an expected RRC connection establishment procedure;

when RRC setup is not explicitly sent from TTCN but is preconfigured at the SS to be sent together with the contention resolution,

the default UL grant configuration is configured at the SS */

var boolean v_TimerIsRunning := p_Timer.running;

var template (omit)  NR_UplinkTimeAlignment_Synch_Type v_PUCCH_Synch;

if (not f_NR_EarlyContentionResolution()) {

v_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Stop;  /* No automatic UL time alignment:

if the SS would send MAC PDU with TA while the waits for contention resolution, it will consider contention resolutions as being fail

=> in case of automatic handling of the RRCConnectionSetup in Msg4 (RRCConnectionSetup) UL time alignment shall be started not before RRCConnectionSetupComplete has been received */

f_NR_ULGrantConfiguration_Start(p_NR_CellId, -, -, -, v_PUCCH_Synch, -, -, tsc_NoCnfReq ); //@sic R5s200161 sic@

} // else: UL grant configuration is done just before sending of RRCSetup

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(p_EstablishmentCause, p_InitialUE_Identity)))

{

p_Timer.stop;       /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */

return true;

}

[v_TimerIsRunning] p_Timer.timeout

{

}

}

return false;

}

....

After change:

…

function f_NR_RRC_SetupRequest_Common(NR_CellId_Type p_NR_CellId,

timer p_Timer,

template  EstablishmentCause p_EstablishmentCause := cr_NR_EstablishmentCause_Any,

template InitialUE_Identity p_InitialUE_Identity := ?)

runs on NR_BASE_PTC return boolean

{ /* common procedure to receive the initial RRC setup request for an expected RRC connection establishment procedure;

when RRC setup is not explicitly sent from TTCN but is preconfigured at the SS to be sent together with the contention resolution,

the default UL grant configuration is configured at the SS */

var boolean v_TimerIsRunning := p_Timer.running;

var template (omit)  NR_UplinkTimeAlignment_Synch_Type v_PUCCH_Synch;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_NR_CellId, tsc_NR_BWP_Id);

if (not f_NR_EarlyContentionResolution()) {

v_PUCCH_Synch := cs_NR_UplinkTimeAlignment_Stop;  /* No automatic UL time alignment:

if the SS would send MAC PDU with TA while the waits for contention resolution, it will consider contention resolutions as being fail

=> in case of automatic handling of the RRCConnectionSetup in Msg4 (RRCConnectionSetup) UL time alignment shall be started not before RRCConnectionSetupComplete has been received */

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(12808, v_NR_UplinkBWP);

f_NR_ULGrantConfiguration_Start(p_NR_CellId,

-,

v_NR_ResourceAllocation.Imcs,

cs_NR_FreqDomainSchedulExplicit(-, v_NR_ResourceAllocation.Lrbs),

v_PUCCH_Synch,

-,

-,

tsc_NoCnfReq ); //@sic R5s200161 sic@

} // else: UL grant configuration is done just before sending of RRCSetup

alt {

[] SRB.receive(car_NR_SRB0_RrcPdu_IND(p_NR_CellId, cr_38508_RRCSetupRequest(p_EstablishmentCause, p_InitialUE_Identity)))

{

p_Timer.stop;       /* NOTE: acc. to ES 201 873-1 cl. 23.3 the timer can be stopped even if it has not been started before */

return true;

}

[v_TimerIsRunning] p_Timer.timeout

{

}

}

return false;

} …

MCC160 proposed implementation

=========================================================================================

f_NR_ServingCellConfig_Init =========================================================================================

function f_NR_ServingCellConfig_Init() return template (value) NR_ServingCellConfig_Type

{

var template (value) NR_ServingCellConfig_Type v_ServingCellConfig;

var CellGroupId v_CellGroupId := tsc_NR_CellGroupId_MCG;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := cs_38508_MAC_CellGroupConfig;  //@sic R5s190446 sic@

var P_Max v_P_Max := f_NR_InitP_Max();  //@sic R5-190784 sic@

var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig:= cs_38508_PhysicalCellGroupConfig(v_P_Max); //@sic R5-190784 sic@

var integer v_ServCellIndex := f_NR_ServCellIndex();                            //@sic R5-192274 sic@

if (f_GetTestcaseAttrib_ENDC(testcasename())) {

v_CellGroupId := tsc_NR_CellGroupId_SCG;

if (f_NR_TestcaseIsL2Testcase()) { //@sic R5s210315 sic@

// Common test specific parameters for L2 as per 38.523-1 Table 7.1.0-1

//For NR5GC layer test cases, these parameters are set after the NR5GC preamble when briging UE to RRC_CONNECTED state

v_MAC_CellGroupConfig.bsr_Config := cs_NR_BSR_Config_L2;

v_MAC_CellGroupConfig.phr_Config := { release := NULL };

}

}    v_ServingCellConfig := cs_NR_ServingCellConfig_SpCell(cs_NR_SS_SpCellConfig(v_ServCellIndex,

v_CellGroupId,

v_MAC_CellGroupConfig,

v_PhysicalCellGroupConfig));

return v_ServingCellConfig;

}

=========================================================================================

NEW FUNCTION  f_NR_CellInfo_SetMAC_CellGroupConfigL2

=======================================================================================

/*

* @desc      Set MAC_CellGroupConfig stored in the cell array as per layer  as per Common test specific parameters for L2 as per 38.523-1 Table 7.1.0-1

*/

function f_NR_CellInfo_SetMAC_CellGroupConfigL2(NR_CellId_Type p_NR_CellId) runs on NR_BASE_PTC

{ //@sic R5s210315 sic@

var MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_NR_CellId);

v_MAC_CellGroupConfig.bsr_Config := valueof(cs_NR_BSR_Config_L2);

v_MAC_CellGroupConfig.phr_Config := { release := NULL };

f_NR_CellInfo_SetMAC_CellGroupConfig(p_NR_CellId, v_MAC_CellGroupConfig);

}

=========================================================================================

f_NR5GC_RRC_ConnectedState3N_Def

=========================================================================================

function f_NR5GC_RRC_ConnectedState3N_Def(NR_CellId_Type p_NR_CellId,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (omit) NR_RLC_BearerToAddModList_Type p_RLCConfigList := omit,

template (omit) MAC_CellGroupConfig p_MACConfig := omit,

template (omit) DRB_ToAddModList p_DRBToAddMod := omit,

template (omit) NR_RadioBearerList_Type p_SS_Drb_Config := omit,

template (omit) PDSCH_ServingCellConfig.nrofHARQ_ProcessesForPDSCH  p_NrofHARQ_ProcessesForPDSCH := omit

) runs on NR5GC_PTC

{ // @sic R5-190710 re-organised procedures 4.5.4 and 4.9.4 sic@

// @sic R5-203533: added p_NrofHARQ_ProcessesForPDSCH sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template(omit)  MAC_CellGroupConfig v_MAC_CellGroupConfig;

// Step 1 Page UE

f_NR_Paging_Def(p_NR_CellId, RRC_IDLE);

v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_NR_CellId);

if (isvalue (p_MACConfig)) {

v_MAC_CellGroupConfig := p_MACConfig;

} else {

if (f_NR_TestcaseIsL2Testcase()) { //@sic R5s210315 sic@

f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_NR_CellId);

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_NR_CellId);

}

}

f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_NR_CellId, v_ReceivedMsg, p_RLCConfigList, v_MAC_CellGroupConfig, p_DRBToAddMod, p_SS_Drb_Config, -, p_NrofHARQ_ProcessesForPDSCH);

// Step 9a1 - 9a2

select (p_TestMode) {

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_NR_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_NR_CellId, p_IpPduDelayTime);

}

}

case else { // do nothing

}

}

}

=======================================================================================

f_NR5GC_RRC_ConnectedState3N_NRDC

=======================================================================================

function f_NR5GC_RRC_ConnectedState3N_NRDC(NR_CellId_Type p_MCG_CellId,

NR_CellId_Type p_SCG_CellId,

NRDC_DRB_Configuration_Type p_NRDC_DRB_Type,

NR_TESTMODE_STATE_Type p_TestMode := TESTMode_OFF,

template (value) UE_TestLoopModeA_NR_LB_Setup_Type p_UE_TestLoopModeA_NR_LB_Setup := cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

UE_TestLoopModeB_LB_Setup_Type p_IpPduDelayTime := '00'O,

template (value) PDCP_Config p_PDCP_ConfigSCG_DRB := cs_38508_PDCP_Config

) runs on NR5GC_PTC

{ //@sic R5-205332 R5-605339 R5-606298 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var template(omit) MAC_CellGroupConfig v_MAC_CellGroupConfig := omit;

if (f_NR_TestcaseIsL2Testcase()) { //@sic R5s210315 sic@

f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCG_CellId);

f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_MCG_CellId);

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_MCG_CellId);

}

//Steps 1-8: Same as table 4.5.4.2-3, steps 1-8.

f_NR_Paging_Def(p_MCG_CellId, RRC_IDLE);

v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(p_MCG_CellId);

f_NR5GC_RRC_ConnectedState3N_Steps5_8(p_MCG_CellId, v_ReceivedMsg, -, v_MAC_CellGroupConfig, -, -, -, -);  //Reconfigure SRB2 + MCG DRBs as per default

// Steps 8A to 8C

// The SS transmits an RRCReconfiguration message and a PDU SESSION MODIFICATION COMMAND to add a new SCG DRB or a new split DRB

// The UE transmits an RRCReconfigurationComplete message including nr-SCG-Response.

// The UE transmits a ULInformationTransfer message and an PDU SESSION MODIFICATION COMPLETE message.

f_NR_RRCReconfig_AddSCG_DRBs_NRDC(p_MCG_CellId, p_SCG_CellId, p_NRDC_DRB_Type, p_PDCP_ConfigSCG_DRB);

// Step 9a1 - 9a2

//IF Test Loop Function=On, the SS transmits a CLOSE UE TEST LOOP message to enter the UE test loop mode. The CLOSE UE TEST LOOP is using the associated condition for the test loop.

//The UE transmits a CLOSE UE TEST LOOP COMPLETE message to confirm that loopback entities for the radio bearer(s) have been created and loop back is activated.

select (p_TestMode) {

case (TEST_LOOPModeA_ON) {

f_NR5GC_CloseUE_TestLoopModeA (p_MCG_CellId, p_UE_TestLoopModeA_NR_LB_Setup);

}

case (TEST_LOOPModeB_ON) {

f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime);

}

case (PING_Or_TEST_LOOPModeB_ON) {

if (not pc_IP_Ping) {

f_NR5GC_CloseUE_TestLoopModeB (p_MCG_CellId, p_IpPduDelayTime);

}

}

case else { // do nothing

}

}

}

=======================================================================================

f_TC_7_1_1_3_8_NR_Common

Similar change in

f_TC_7_1_1_7_1_NR_Common

f_TC_7_1_1_1_1_NR5GC

f_TC_7_1_1_1_5_NR5GC

f_TC_7_1_1_2_3_NR5GC

f_TC_7_1_1_3_2_NR5GC

f_TC_7_1_1_3_2b_NR5GC

f_TC_7_1_1_3_5_NR5GC

==========================================================================================

function f_TC_7_1_1_3_8_NR_Common (NR_CellId_Type    p_PCell,

NR_CellId_Type    p_SCell,

NR_CA_Tested_Type p_NR_CA_Tested

) runs on NR5GC_PTC

{//UE power headroom reporting / SCell activation / DL pathloss change reporting

var template (value) CellGroupConfig v_SCGConfig;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var boolean v_RRC_MsgRcvd;

var boolean v_PHR_Rcvd;

var template (omit) RRCReconfiguration_v1530_IEs v_V1530Ext;

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP ;

// Multi PR initialisation is taken care in Test body.

f_NR5GC_Init_CA(NR_4, {p_PCell, p_SCell}, p_NR_CA_Tested); //@sic R5s211385 sic@

f_NR_CellInfo_SetMAC_CellGroupConfigL2(p_SCell);  //@sic R5s210315 sic@

// Create and configure cells

f_NR_CellConfig_PCell_Def(p_PCell);

f_NR_CellConfig_SCell(p_SCell,Scell_Active);

// Preamble: If pc_IP_Ping is set to TRUE then, the UE is in 5GS state 3N-A according to TS 38.508-1 [4], clause 4.4A.2 Table 4.4A.2-3.

//           Else, the UE is in 5GS state 3N-A and Test Loop Function (On) with UE test loop mode B on NR Cell 1 according to 38.508-1[4], clause 4.4A.2 Table 4.4A.2-3.

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (p_PCell, STATE_IDLE_1A, TESTMode_OFF);//@sic R5s211385 sic@

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NR5GC_RRC_ConnectedState3N_Def(p_PCell,

-,//@sic R5s211385 sic@

-,

-,  //p_IpPduDelayTime

-,

-,  //MAC_CellGroupConfig

-,

-

);

f_NR_TestBody_Set(true);

f_NR_SetCellPower(p_SCell, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2);

//@siclog Steps 1-2 siclog@

//The SS transmits an RRCReconfiguration message including a sCellToAddModList to add NR Cell 3 as a SCell.

//The UE transmits the RRCReconfigurationComplete message?

f_NR5GC_AddMod_1SCell(p_PCell, p_SCell,-, true); // NR Cell1, NR Cell3/10

v_RRC_MsgRcvd:= false;

v_PHR_Rcvd := false;

v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(p_PCell, tsc_NR_BWP_Id);

v_NR_ResourceAllocation := f_NR_ResourceAllocation_UplinkGrant(4224, v_NR_UplinkBWP);

//@siclog "Steps 4 preconfig" siclog@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_PCell);

v_MAC_CellGroupConfig.phr_Config := cs_38508_PHR_Config_Setup(cs_PHR_Config_71138);

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_SCGConfig := cs_38508_CellGroupConfig (tsc_NR_CellGroupId_MCG,

omit,

omit,

v_MAC_CellGroupConfig);

v_V1530Ext := cs_NR_RRCReconfiguration_v1530_IEs(bit2oct(encvalue(v_SCGConfig)));

//@siclog "Steps 3" siclog@

//The SS transmits UL grant to the UE at every 10ms in PDCCH occasion.

f_NR_ULGrantConfiguration_StartPeriodicGrant(p_PCell, cs_TimingInfo_Now, v_NR_ResourceAllocation, 10);

//@siclog "Steps 4" siclog@

f_NR_SS_SystemIndCtrlConfig(p_PCell, cds_NR_SystemIndCtrl_MAC_PHR(enable));

f_NR_SendRRCReconfiguration (p_PCell, -,-, v_V1530Ext,-,-,-,false);

…

f_NR_Postamble(p_PCell, STATE_CONNECTED_3A);

} //end of f_TC_7_1_1_3_8_NR_Common

=======================================================================================

f_TC_7_1_3_4_1_NR5GC

==========================================================================================

function f_TC_7_1_3_4_1_NR5GC() runs on NR5GC_PTC

{ //PDCP handover / Non-lossless handover / PDCP sequence number maintenance

//PDCP Status Report to convey the information on missing or acknowledged PDCP SDUs at handover/

//In order deliveryy and duplicate elimination in the downlink

var NR_PDCP_SS_State_Type v_NR_PDCP_State;

var template (value) DRB_ToAddMod v_DRB_ToAddMod;

var template (value) NR_RadioBearer_Type v_SS_Drb_Config;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var RLC_Config.am.ul_AM_RLC.t_PollRetransmit  v_T_PollRetransmit := ms150; //@sic R5-201219 sic@

var DRB_Identity v_DRBId;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;//@sic R5s200690 sic@

var DL_t_Reassembly_Type v_T_Reassembly;

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellInfo_InitMaxReferencePower(nr_Cell1,-82,-82); //@sic R5-195929 sic@

f_NR_CellConfig_Def(nr_Cell1);

//Create and configure NR Cell2

f_NR_CellInfo_InitMaxReferencePower(nr_Cell2,-82,-82); //@sic R5-195929 sic@

f_NR_CellConfig_Def(nr_Cell2);

//Preamble

f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

f_NR_AS_CipheringAlgorithm_Set(nea0);

//@sic R5s200690 R5s210315 sic@

f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell2);  //@sic R5s210315 sic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell2);

v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell2, v_MAC_CellGroupConfig);

f_NR_CellInfo_SetMAC_CellGroupConfigL2(nr_Cell1);  //@sic R5s210315 sic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

v_DRBId:= f_NR_GetDefaultDRB_ForFirstPDUSession();

v_SS_Drb_Config := f_NR_GetRadioBearerConfig_FullAM(nr_Cell1,

v_DRBId,

-,

cs_NR_PDCP_RbConfig_TransparentMode,

-,

cs_SDAP_Configuration_None,

-,

v_T_PollRetransmit);//@sic  R5-201219 sic@

v_DRB_ToAddMod := cs_38508_NRDRB_ToAddMod((f_NR_GetSDAP_InDRBList(v_DRBId)),

v_DRBId,

cs_38508_PDCP_Config);

//@sic R5s200690 sic@

v_T_Reassembly := f_NR_DL_AM_T_Reassembly(f_NR_CellInfo_GetIsFR1(nr_Cell1));

v_RLC_BearerConfigList := f_NR_ReconfigureRLC_AM(v_DRBId, -, v_T_PollRetransmit, -, -, -, -, v_T_Reassembly);

f_NR5GC_RRC_ConnectedState3N_Def(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

-,

v_RLC_BearerConfigList,

v_MAC_CellGroupConfig,  //@sic R5s210315 sic@

{v_DRB_ToAddMod},

{v_SS_Drb_Config});

//Initialize PDCP Parameters

f_NR_PDCP_Init(v_NR_PDCP_State, PDCP_SNLength18, v_DRBId);

f_NR_TestBody_Set(true);

fl_TC_7_1_3_4_1_NR5GC_TestBody(v_NR_PDCP_State);

f_NR_TestBody_Set(false);

f_NR5GC_OpenUE_TestLoopMode_Deactivate_TestMode(nr_Cell1);

//Postamble

f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A);

f_NR_ReleaseAllCells();

}

…

}

=======================================================================================

f_TC_7_1_1_3_3_NR5GC

same change in

f_TC_7_1_1_3_4_NR5GC

f_TC_7_1_1_3_6_NR5GC

=======================================================================================

function f_TC_7_1_1_3_3_NR5GC() runs on NR5GC_PTC

{ // Correct handling of MAC control information / Scheduling requests

var DRB_Identity v_DRBId1 ;

var DRB_Identity v_DRBId2 ;

var UInt_Type v_LCID1; //@sic R5-201519 sic@

var UInt_Type v_LCID2;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var IntegerList_Type v_ListOfDRBs;

var template (value) DRB_ToAddModList v_DRBToAddModList;

var template (value) SDAP_Config v_SDAP_Config_DRB;

var T_PollRetransmit v_T_PollRetransmit := ms80; //@sic R5-202539 sic@

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//@sic R5s210315 sic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.bsr_Config := cs_NR_BSR_Config_71133;

v_MAC_CellGroupConfig.phr_Config := { release := NULL };

v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

// Now that at least one PDU session has been established, get all DRB ids required

v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2);

//@sic R5-201519 sic@

v_DRBId1 := v_ListOfDRBs[0];v_LCID1 :=v_DRBId1+3;

v_DRBId2 := v_ListOfDRBs[1];v_LCID2 :=v_DRBId2+3;

//@sic R5s190933 sic@

//@sic R5-201519 sic@

v_SS_Drb_ConfigList  := {

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId1,-,-,-,-,v_LCID1),

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId2,-,-,-,-,v_LCID2)  //@sic R5s200075 sic@

};

v_SS_Drb_ConfigList[0].Config.AddOrReconfigure.RlcBearer.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := v_T_PollRetransmit; //@sic R5-202539 sic@

v_SS_Drb_ConfigList[1].Config.AddOrReconfigure.RlcBearer.Config.Rlc.Rb.AM.Tx.R15.t_PollRetransmit := v_T_PollRetransmit; //@sic R5-202539 sic@

v_RLC_BearerConfigList[0] := cs_38508_RLC_BearerConfig_DRB_LCID( v_LCID1,//@sic R5-201519 sic@

v_DRBId1,//@sic R5-202539 sic@

cs_38508_RLC_Config_AM_DRB(cs_38508_UL_AM_RLC(v_T_PollRetransmit), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))),

cs_38508_LogicalChannelConfig_DRB(2, kBps0, 7)) ;

v_RLC_BearerConfigList[1] := cs_38508_RLC_BearerConfig_DRB_LCID( v_LCID2,

v_DRBId2,//@sic R5-202539 sic@

cs_38508_RLC_Config_AM_DRB(cs_38508_UL_AM_RLC(v_T_PollRetransmit), f_NR_DL_AM_RLC(f_NR_CellInfo_GetIsFR1(nr_Cell1))),

cs_38508_LogicalChannelConfig_DRB(1, kBps0, 6, true)) ; //@sic R5s190933 sic@

v_SDAP_Config_DRB := f_NR_GetSDAP_InDRBList(f_NR_GetDefaultDRB_ForFirstPDUSession()); //@sic R5s191054 sic@

v_SDAP_Config_DRB.defaultDRB := false;

v_SDAP_Config_DRB.mappedQoS_FlowsToAdd := omit;

v_DRBToAddModList := {cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB,

v_DRBId1,

cs_38508_PDCP_Config),

cs_38508_NRDRB_ToAddMod(v_SDAP_Config_DRB,

v_DRBId2,

cs_38508_PDCP_Config)};

//Set null cipher Algorithm

f_NR_AS_CipheringAlgorithm_Set(nea0);

//Configure on SS the DRB in No MAC header manipulation in DL and RLC and PDCP in transparent mode

f_NRL2_RRC_ConnectedState3N_ExtraDRBsNoNAS(nr_Cell1,

TEST_LOOPModeA_ON,

cs_UE_TestLoopModeA_NR_LB_Setup_NoScaling,

v_RLC_BearerConfigList,

v_MAC_CellGroupConfig,  //MAC_CellGroupConfig

v_DRBToAddModList,

v_SS_Drb_ConfigList

);

f_NR_TestBody_Set(true);

====================================================================================