3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210345

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1624 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 8.1.2.1.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Starpoint |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-18 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of NR5GC test case 8.1.2.1.4 in FR1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 8.1.2.1.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1 Overview	3

2 Corrections required	3

2.1 Change 1	3

3 Branches executed	4

4 Execution Log Files	5

4.1 HiSilicon Balong 5000	5

5 References	5

# Overview

This document lists all the changes needed to correct issues in the ATS ATS NR5GC_IWD_20wk50 related to the title of this CR.

Contact:	Ting Gao
gaoting@starpointcomm.com

# Corrections required

## Change 1

| Function name | fl_NR_GetCellGroupConfigREEST |
| --- | --- |
| Reason for change | 1 entry[n]and entry[n-1]are exactly same in rlc_BearerToAddModList, UE may fail to read the configuration which will lead to test case failure. 2 According to TS38.523-1 Table 8.1.2.1.4.3.3-3,rlf-TimersAndConstants should be set to release. The correct rlf_TimersAndConstants is covered by default value in this function. |
| Summary of change | 1 Defined a new variable v_RLC_BearerToAddModList_2. 2 Move the code in yellow to the end of the function. |
| TTCN module | NR5GC_RRCSteps.ttcn |
| MCC160 Comment | - Accepted - Accepted |

Before change

function fl_NR_GetCellGroupConfigREEST( NR_CellId_Type                              p_TargetCellId,

template (omit) NR_SCellToAddModList_Type   p_SCellToAddModList := omit,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit

) runs on NR5GC_PTC

return template(value) CellGroupConfig

{ //@sic R5-206297 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) SpCellConfig v_SpCellConfig;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId);

var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId);

var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId);

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList(true_);

var integer i;

//Remove SRB1 - entry 0 - in BearerToAddModList

for (i:=1; i < lengthof(v_RLC_BearerToAddModList); i:=i+1) {

v_RLC_BearerToAddModList[i-1] := v_RLC_BearerToAddModList[i];

}

v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig);

if (isvalue(p_SCellToAddModList)) {

v_CellGroupConfig.sCellToAddModList := p_SCellToAddModList;

}

if (isvalue(p_RlfTimersAndConstants)) {

v_CellGroupConfig.spCellConfig.rlf_TimersAndConstants := p_RlfTimersAndConstants;

}

v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,

v_RLC_BearerToAddModList,

omit,               // NR_RLC_BearerToReleaseList_Type

v_MAC_CellGroupConfig,

v_PhysicalCellGroupConfig,

v_SpCellConfig);

return v_CellGroupConfig;

}

After change

function fl_NR_GetCellGroupConfigREEST( NR_CellId_Type                              p_TargetCellId,

template (omit) NR_SCellToAddModList_Type   p_SCellToAddModList := omit,

template(omit) SpCellConfig.rlf_TimersAndConstants p_RlfTimersAndConstants := omit

) runs on NR5GC_PTC

return template(value) CellGroupConfig

{ //@sic R5-206297 sic@

var template (value) CellGroupConfig v_CellGroupConfig;

var template (value) SpCellConfig v_SpCellConfig;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(p_TargetCellId);

var template (value) PhysicalCellGroupConfig v_PhysicalCellGroupConfig := f_NR_CellInfo_GetPhysicalCellGroupConfig(p_TargetCellId);

var template (omit) ServingCellConfig v_ServingCellConfig := f_NR_GetServinCellConfig(p_TargetCellId);

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList := f_NR5GC_GetExisting_BearerToAddModList_1(true_);

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerToAddModList_2;

var integer i;

//Remove SRB1 - entry 0 - in BearerToAddModList

for (i:=0; i < lengthof(v_RLC_BearerToAddModList)-1; i:=i+1) {

v_RLC_BearerToAddModList_2[i] := v_RLC_BearerToAddModList[i+1];

}

v_SpCellConfig := f_NR_GetSpCellConfigDef(p_TargetCellId, v_ServingCellConfig);

v_CellGroupConfig := cs_38508_CellGroupConfig(tsc_NR_CellGroupId_MCG,

v_RLC_BearerToAddModList_2,

omit,               // NR_RLC_BearerToReleaseList_Type

v_MAC_CellGroupConfig,

v_PhysicalCellGroupConfig,

v_SpCellConfig);

if (isvalue(p_SCellToAddModList)) {

v_CellGroupConfig.sCellToAddModList := p_SCellToAddModList;

}

if (isvalue(p_RlfTimersAndConstants)) {

v_CellGroupConfig.spCellConfig.rlf_TimersAndConstants := p_RlfTimersAndConstants;

}//ttcn cr

return v_CellGroupConfig;

}

# Branches executed

The test case was executed on NR band n41.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000  passed this test case on Starpoint SP9500 Test System. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:  
\ TC_8_1_2_1_4_NR5GC_PASS.spm

- PIXCIT settings used:

\ PCT_PICS_PIXIT.sppar

# References

| [1] | R5s210346: Supporting information for agreement of NR5GC test case 8.1.2.1.4 in FR1. |
| --- | --- |