3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210361

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1633 | rev | - | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_NR_InitialiseSiScheduling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | We can observe two problems in this function 1. Current implementation of function fl_NR_InitialiseSiScheduling declares v_SiSchedulList as template (value) variable when according to ASN.1 SiList is defined as optional field: type record NR_AllOtherSiSchedul_Type { NR_SiWindowLength_Type WindowLength optional, NR_OtherSiSchedulList_Type SiList optional, NR_OtherSiSchedulList_Type SegmentedSiList optional }; This can cause a problem later, as it is used as input parameter of cs_NR_SiSchedulList_Def template but as observed in template declaration: cs_NR_SiSchedulList_Def(SI_SchedulingInfo.si_WindowLength p_SiWindowLength, /* as in Sib1 */ template (value) NR_OtherSiSchedulList_Type p_SiSchedulList, template (omit) NR_OtherSiSchedulList_Type p_SegmentedSiSchedulList := omit) 2nd parameter is value restricted, does not accept an ”omit”. 2. During function execution, following assignment is done: v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); While current ASN.1 definition declares that Sis is an optional field, hence a runtime error can happen in case Sis is not a value: type record NR_BcchInfo_Type { BCCH_BCH_Message MIB optional, BCCH_DL_SCH_Message SIB1 optional, NR_SI_List_Type SIs optional, NR_SegmentedSI_List_Type SegmentedSIs optional - Can be solved declaring v_SiSchedulList with (omit) restriction and doing subsequent change in cs_NR_SiSchedulList_Def template. - Can be solved checking that SIs actually contains a value before doing any operation (lenghtof). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change cs_NR_SiSchedulList_Def 2nd input parameter to as SIList is an optional field. - Declared v_SiSchedulList with omit restriction and initialized to omit. Wrapped v_SiSchedulList further assignments depending on SIs being a value or not. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.5.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1 Correction to template cs_NR_SiSchedulList_Def	4

1.2 Correction to function fl_NR_InitialiseSiScheduling	4

# Corrections required

# 1.1 Correction to template cs_NR_SiSchedulList_Def

| Template name | cs_NR_SiSchedulList_Def |
| --- | --- |
| Reason for change | Template restriction of second parameter needs to be restricted to omit as SiList being of optional nature. |
| Summary of change | Changes cs_NR_SiSchedulList_Def 2nd input parameter as SIList is an optional field.. |
| TTCN module | ATS_NR5GC/Common/NR/NR_CellCfg_Templates.ttcn |
| MCC160 Comment | Accepted – see change 2 |

Before change:

template (value) NR_AllOtherSiSchedul_Type cs_NR_SiSchedulList_Def(SI_SchedulingInfo.si_WindowLength p_SiWindowLength,           /* as in Sib1 */

template (value) NR_OtherSiSchedulList_Type p_SiSchedulList,

template (omit) NR_OtherSiSchedulList_Type p_SegmentedSiSchedulList := omit) :=

{ /* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

WindowLength := p_SiWindowLength,

SiList := p_SiSchedulList,

SegmentedSiList := p_SegmentedSiSchedulList

};

After change:

template (value) NR_AllOtherSiSchedul_Type cs_NR_SiSchedulList_Def(SI_SchedulingInfo.si_WindowLength p_SiWindowLength,           /* as in Sib1 */

template (omit) NR_OtherSiSchedulList_Type p_SiSchedulList := omit,//WI#817388

template (omit) NR_OtherSiSchedulList_Type p_SegmentedSiSchedulList := omit) :=

{ /* @status    APPROVED (ENDC, IMS, NR5GC, NR5GC_IRAT, POS) */

WindowLength := p_SiWindowLength,

SiList := p_SiSchedulList,

SegmentedSiList := p_SegmentedSiSchedulList

};

# 1.2 Correction to function fl_NR_InitialiseSiScheduling

| Function name | fl_NR_InitialiseSiScheduling |
| --- | --- |
| Reason for change | Current implementation of function fl_NR_InitialiseSiScheduling declares v_SiSchedulList as template (value) variable when according to ASN.1 SiList is defined as optional field. Following assignment is done: v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs); Being SIs an optional field, this will be a non-valid operation in case SIs is omit. |
| Summary of change | Declared v_SiSchedulList with omit restriction and initialized to omit. Wrapped v_SiSchedulList further assignments depending on SIs being a value or not. |
| TTCN module | ATS_NR5GC/Common/NR/NR_ConfigurationSteps.ttcn |
| MCC160 Comment | Accepted, not because of the type reason mentioned above, but because in the NR sysinfo combination NR_9 and NR_11, there is no SI (there is only segmented SI), so the variable v_SiSchedulList needs to be set to omit. It might fail only test cases using sysinfo combinations NR_9 and NR_11. |

<<SKIPPED CODE>>

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId,

boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template(omit) NR_AllOtherSiSchedul_Type

{

var template(omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit;

var  NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId);

var SI_SchedulingInfo.si_WindowLength v_SiWindowLength;

var integer v_NoOfSIs;

var integer v_BroadcastIndex;

var integer i;

var template (value) NR_OtherSiSchedulList_Type v_SiSchedulList;

var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit;

var SI_SchedulingInfo v_SI_SchedulingInfo;

var integer v_NoOfSIsSegmented;

var NR_RedundancyVersion_Type v_RV := 0;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

var integer v_SegmentedSiPosistionInSIB1;

var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId);

select (v_NR_CellSysInfo.NR_SysinfoCombination) {

case (MibOnly, NR_1) {} //do nothing - no SI

case else {

v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId);

v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength;

v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs);

v_BroadcastIndex := 0;

for (i := 0; i < v_NoOfSIs; i:=i+1) {

if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting or p_BroadcastAllSI) {  //@sic R5-201006 sic@

v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity;

select (v_SubcarrierSpacing) { //@sic R5-192812 sic@

case(kHz15) { //Table 7.3.3.2-2: MIB, SIB1 and SI scheduling for FR1: FDD and TDD, SCS=15kHz

// 4 repetitions at offset {5, 6, 15, 16}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

v_SiSchedulList[v_BroadcastIndex].Window[1].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[1].SlotOffset := 6;

v_SiSchedulList[v_BroadcastIndex].Window[2].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[2].SlotOffset := 15;

v_SiSchedulList[v_BroadcastIndex].Window[3].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[3].SlotOffset := 16;

}

case(kHz30) { // Table 7.3.3.2-3: MIB, SIB1 and SI scheduling for FR1 TDD, SCS=30kHz

// 2 repetitions at offset {5, 6}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

v_SiSchedulList[v_BroadcastIndex].Window[1].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[1].SlotOffset := 6;

}

case(kHz120) { // Table 7.3.3.2-4: MIB, SIB1 and SI scheduling for FR2: TDD, SCS=120kHz

// 1 repetitions at offset {5}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); }

}

v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@

}

}

<<SKIPPED CODE>>

After change:

<<SKIPPED CODE>>

function fl_NR_InitialiseSiScheduling(NR_CellId_Type p_NR_CellId,

boolean p_BroadcastAllSI := false) runs on NR_BASE_PTC return template(omit) NR_AllOtherSiSchedul_Type

{

var template(omit) NR_AllOtherSiSchedul_Type v_AllOtherSiSchedul := omit;

var  NR_CellSysInfo_Type v_NR_CellSysInfo := f_NR_CellInfo_GetSysinfo(p_NR_CellId);

var SI_SchedulingInfo.si_WindowLength v_SiWindowLength;

var integer v_NoOfSIs;

var integer v_BroadcastIndex;

var integer i;

var template (omit) NR_OtherSiSchedulList_Type v_SiSchedulList := omit;

var template (omit) NR_OtherSiSchedulList_Type v_SegmentedSiSchedulList := omit;

var SI_SchedulingInfo v_SI_SchedulingInfo;

var integer v_NoOfSIsSegmented;

var NR_RedundancyVersion_Type v_RV := 0;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

var integer v_SegmentedSiPosistionInSIB1;

var SubcarrierSpacing v_SubcarrierSpacing := f_NR_CellInfo_GetSCS(p_NR_CellId);

select (v_NR_CellSysInfo.NR_SysinfoCombination) {

case (MibOnly, NR_1) {} //do nothing - no SI

case else {

v_SI_SchedulingInfo := f_NR_GetSIB1_SiSchedulingInfo(p_NR_CellId);

v_SiWindowLength := v_SI_SchedulingInfo.si_WindowLength;

v_BroadcastIndex := 0;

if(isvalue(v_NR_CellSysInfo.BcchInfo.SIs)) {

<<MOVED CODE>>

v_NoOfSIs := lengthof(v_NR_CellSysInfo.BcchInfo.SIs);

<<MOVED CODE>>

for (i := 0; i < v_NoOfSIs; i:=i+1) {

if (v_SI_SchedulingInfo.schedulingInfoList[i].si_BroadcastStatus == broadcasting or p_BroadcastAllSI) {  //@sic R5-201006 sic@

v_SiSchedulList[v_BroadcastIndex].Periodicity := v_SI_SchedulingInfo.schedulingInfoList[i].si_Periodicity;

select (v_SubcarrierSpacing) { //@sic R5-192812 sic@

case(kHz15) { //Table 7.3.3.2-2: MIB, SIB1 and SI scheduling for FR1: FDD and TDD, SCS=15kHz

// 4 repetitions at offset {5, 6, 15, 16}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

v_SiSchedulList[v_BroadcastIndex].Window[1].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[1].SlotOffset := 6;

v_SiSchedulList[v_BroadcastIndex].Window[2].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[2].SlotOffset := 15;

v_SiSchedulList[v_BroadcastIndex].Window[3].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[3].SlotOffset := 16;

}

case(kHz30) { // Table 7.3.3.2-3: MIB, SIB1 and SI scheduling for FR1 TDD, SCS=30kHz

// 2 repetitions at offset {5, 6}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

v_SiSchedulList[v_BroadcastIndex].Window[1].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[1].SlotOffset := 6;

}

case(kHz120) { // Table 7.3.3.2-4: MIB, SIB1 and SI scheduling for FR2: TDD, SCS=120kHz

// 1 repetitions at offset {5}

v_SiSchedulList[v_BroadcastIndex].Window[0].SearchSpaceAndDci := cs_NR_SearchSpaceDlDciAssignment_CSSType0A(v_NR_PhysicalParameters, v_RV); //Only one repetition within the SI-window

v_SiSchedulList[v_BroadcastIndex].Window[0].SlotOffset := 5;

}

case else { FatalError(__FILE__, __LINE__, "Invalid SubCarrierSapcing"); }

}

v_BroadcastIndex := v_BroadcastIndex + 1; //@sic R5-201006 sic@

}

}

}

<<SKIPPED CODE>>