3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210389

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1646 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC CA inter-cell handover procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Currently the C-RNTI configuration on target PCell / SCell is done with timing info “now”. This is done immediately after RRCReconfiguration is submitted for sending, and this may cause the message to be scrambled with the new C-RNTI value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “cs_TimingInfo_NR(v_Timing2)” to function “f_NR_SS_C_RNTI_Config()” for delay C-RNTI configuration by 20ms for both p_TargetPCellId and p_TargetSCellId. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail some NR5GC CA test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.1.7.1.NR5GC, 8.1.4.1.7.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common() |
| --- | --- |
| Reason for change | Currently the C-RNTI confdiguration on target PCell / SCell is done with timing info “now”. This is done immestaely after RRCReconfiguration is submitted for sending, and this may cause the messqage to be scrambled with the new C-RNTI value. |
| Summary of change | Add “cs_TimingInfo_NR(v_Timing2)” to function “f_NR_SS_C_RNTI_Config()” for delay C-RNTI configuration by 20ms for both p_TargetPCellId and p_TargetSCellId. |
| TTCN module | NR5GC_Handover |
| MCC160 Comment | Accepted. |

Before Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common( …… //8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based. // NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value); //Calculate T2 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId); if (isvalue(p_TargetSCellId)) { //@sic R5s210079 sic@ //@sic R5s201249 sic@ f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value); f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value); } f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated); //9. Target PCell: Activate security. if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo); } else { f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo); //@sic R5s200102 sic@ } //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo); } // After time T2 (without activation time): // 13. Target PCell: Receive RRCReconfigurationComplete. SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete)); //14. Target Pell: Start periodic TA. f_NR_ULGrantConfiguration_Common(p_TargetPCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); //15. Target Pell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPCellId)); //16. Target PCell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetPCellId); //17. Source PCell: If source PCell is not target SCell //17.1 Reset SRBs and release DRBs. //17.2 Configure from PCell to normal cell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ if (p_SourcePCellId != valueof(p_TargetSCellId)) { v_DrbIdList := f_NR_GetActiveDRBs(p_SourcePCellId); f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, -, v_DrbIdList ); } } //18. Source SCell: If source SCell is neither target PCell nor target SCell: //18.1 Re-configure source SCell to normal cell and remove source SCell from the SCell list in the source PCell. if (not(match(p_SourceSCellId, p_TargetPCellId)) and not match(p_SourceSCellId, p_TargetSCellId)) { f_NR_ReconfigSCellToNormalCell(p_SourceSCellId, v_RadioBearerList, -); f_NR_Rel_SCell_Common( p_SourcePCellId, p_SourceSCellId, tsc_PCellIndex_0, cs_TimingInfo_Now);//Release SCell from PCell @sic R5s201249 sic@ } } }

After Change

function f_NR5GC_508RRC_IntraNR_CA_HO_InterCell_Common( ...... //8. Target PCell: Configure RACH procedure either dedicated or C-RNTI based. // NOTE 4: The FollowOnFlag is set to true in the ASP reconfiguring C-RNTI f_NR_CellInfo_SetRNTI(p_TargetPCellId, p_RNTI_Value); //Calculate T2 in the TargetPCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), p_TargetPCellId); if (isvalue(p_TargetSCellId)) { //@sic R5s210079 sic@ //@sic R5s201249 sic@ f_NR_CellInfo_SetRNTI(valueof(p_TargetSCellId), p_RNTI_Value); f_NR_SS_C_RNTI_Config(valueof(p_TargetSCellId), p_RNTI_Value,cs_TimingInfo_NR(v_Timing2)); } f_NR_SS_C_RNTI_Config(p_TargetPCellId, p_RNTI_Value, cs_TimingInfo_NR(v_Timing2), tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ f_NR_SS_RachProcedureConfigHO(p_TargetPCellId, p_RachProcedureConfig, v_TimingInfo, p_RACH_ConfigDedicated); //9. Target PCell: Activate security. if ((p_NCC > tsc_NR_38508_NextHopChainingCount) or p_RBConfig_KeyChange) { // @sic R5s190795 R5s200159 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_TargetPCellId, p_NCC, v_TimingInfo); } else { f_NR_SS_AS_ActivateSecurity_NoRefresh(p_TargetPCellId, v_TimingInfo); //@sic R5s200102 sic@ } //10. Target PCell: Configure UL grant configuration ("OnSR", periodic TA is not started). f_NR_ULGrantConfiguration_Common(p_TargetPCellId, v_TimingInfo, -, -, p_UL_GrantConfig_TargetCell); //11. Target PCell: Configure Target PCell as PCell. //12. Target SCell: Configure Target SCell as SCell with new PCell association. //NOTE 5: The FollowOnFlag is set to true in the ASP reconfiguring PCell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ f_NR_ConfigPCell_AddRelSCells(p_TargetPCellId, {valueof(p_TargetSCellId)}, tsc_PCellIndex_0, v_TimingInfo, p_RNTI_Value, -, tsc_NoCnfReq, tsc_FollowOnFlag); //@sic R5-206369 sic@ //Calculate T2 in the TargetSCell v_TimingInfo := f_NR_TimingOtherCell(p_SourcePCellId, cs_TimingInfo_NR(v_Timing2), valueof(p_TargetSCellId)); f_NR_ConfigSCell(valueof(p_TargetSCellId), p_SCGConfig_SCell, v_TimingInfo); } // After time T2 (without activation time): // 13. Target PCell: Receive RRCReconfigurationComplete. SRB.receive(car_NR_SRB1_RrcPdu_IND(p_TargetPCellId, p_RRCConnReconfigComplete)); //14. Target Pell: Start periodic TA. f_NR_ULGrantConfiguration_Common(p_TargetPCellId, cs_TimingInfo_Now, -, cs_NR_UplinkTimeAlignment_Start, -); //15. Target Pell: Inform the SS about completion of the HO (e.g. to trigger PDCP STATUS REPORT PDU). f_NR_SS_PdcpHandoverCtrl(p_TargetPCellId, cas_NR_PdcpHandoverComplete_REQ(p_TargetPCellId)); //16. Target PCell: Re-configure RACH procedure as for initial access. f_NR_SS_RachProcedureConfig_Def(p_TargetPCellId); //17. Source PCell: If source PCell is not target SCell //17.1 Reset SRBs and release DRBs. //17.2 Configure from PCell to normal cell. if (isvalue (p_TargetSCellId)) { //@sic R5s210079 sic@ if (p_SourcePCellId != valueof(p_TargetSCellId)) { v_DrbIdList := f_NR_GetActiveDRBs(p_SourcePCellId); f_NR_ReconfigurePCellToNormalCell( p_SourcePCellId, tsc_PCellIndex_0, -, v_DrbIdList ); } } //18. Source SCell: If source SCell is neither target PCell nor target SCell: //18.1 Re-configure source SCell to normal cell and remove source SCell from the SCell list in the source PCell. if (not(match(p_SourceSCellId, p_TargetPCellId)) and not match(p_SourceSCellId, p_TargetSCellId)) { f_NR_ReconfigSCellToNormalCell(p_SourceSCellId, v_RadioBearerList, -); f_NR_Rel_SCell_Common( p_SourcePCellId, p_SourceSCellId, tsc_PCellIndex_0, cs_TimingInfo_Now);//Release SCell from PCell @sic R5s201249 sic@ } } }

## Additional change from MCC160

| Function name | f_NR_ReconfigSCellToNormalCell |
| --- | --- |
| Reason for change | When a cell is changed to SCell, ServingCellConfig in the cellArray is not changed so it will keep the default value assigned during the creation of the cell. So, when the cell is going to be changed back to PCell, the information in cellArray can be used directly to configure ServingCellConfig in the SS. The current code contains a superflous line setting ServingCellConfig in cellArray that has no effect. |
| Summary of change | Removed line setting ServingCellConfig in cellArray, as nothing is changed. |
| TTCN module | NR_CommonFunctions_CA.ttcn |

function f_NR_ReconfigSCellToNormalCell(NR_CellId_Type p_PCell,

template (omit) NR_RadioBearerList_Type p_DrbConfigList,

template (value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now

) runs on NR_BASE_PTC

{

var template(value) NR_ServingCellConfig_Type v_ServingCellConfig :=  f_NR_CellInfo_GetServingCellConfig(p_PCell);

// Reconfigure StaticInfo to CarrierAggregaion == spCell

// Reconfigure ServingCellInfo to normal cell

f_NR_SS_CommonCellConfig(p_PCell, cads_NR_ServingCellConfig_REQ(p_PCell, p_TimingInfo,v_ServingCellConfig, cs_NR_StaticCellResourceConfig (-, SpCell)));

if (not f_GetTestcaseAttrib_ENDC(testcasename())) {

//Configure SRBs + DRBs

f_NR_SS_SRBs_DRBs_Config(p_PCell, p_TimingInfo, p_DrbConfigList); //@sic R5s201249 sic@

} else {

//Configure DRBs

f_NR_SS_CommonRadioBearerConfig(p_PCell, p_DrbConfigList, p_TimingInfo);

}

}