3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210392

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0677 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | a=sendrecv |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UEs are allowed to include the direction attribute “a=sendrecv” in INVITE or UPDATE during call initiation but cannot be required to do so in the scenarios at hand |  |  |  |  |  |  |  |  |
| Summary of change: |  | Removed “a=sendrecv” from INVITE and UPDATE in call initiation procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Compliant UEs might fail |  |  |  |  |  |  |  |  |
| Clauses affected: |  | MO Calls 7.x.NR5GC with preconditions |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 34.229-5 CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | A draft prose CR is attached |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Hans Rohnert
	hans.rohnert@rohde-schwarz.com

# Corrections required

## Change 1

| Function name | f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7 |
| --- | --- |
| Reason for change | UEs are allowed to include the direction attribute “a=sendrecv” in INVITE or UPDATE during call initiation but cannot be required to do so in the scenarios at hand |
| Summary of change | Removed “a=sendrecv” from INVITE and UPDATE in call initiation procedures. |
| TTCN module | Common/IMS/IMS_Procedures_5G |
| MCC160 Comment | Accepted subject to approval of the proposed prose change |

Before Change:

function f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, charstring p_ContactUri := px_IMS_CalleeContactUri, boolean p_GeolocationInfoAvailable := false, IMS_CallType_Type p_CallType := NORMAL_CALL, IMS_SDP_AudioCodec_Type p_AudioCodec := EVS_16000, IMS_PreconditionIndication_Type p_PreconditionIndication := PreconditionRequired) runs on IMS_PTC return SDP_Message { var SipUrl v_ContactUrl := valueof(f_SIP_BuildSipUri_TX(p_ContactUri)); /* Contact address of the SS as assigned to the UE e.g. in the 183 Session Progress * NOTE 1: in other cases the SipUrl is taken from the 183 Session Progress as sent to the UE (f_MessageHeader_GetContactSipUrl) * => it is the exact value but not a template * NOTE 2: in general px_IMS_CalleeContactUri is used in the 183 Session Progress (i.e. we don't need to hand it over as parameter) */ var template (present) SDP_Message v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(p_InviteRequestWithSdp.SdpOffer, -, p_AudioCodec); v_SdpOffer.media_list[0].attributes := f_SDP_Attributes_Add_RX(v_SdpOffer.media_list[0].attributes, cs_SDP_Attribute_sendrecv); /* !!!! PROSE ISSUE: !!!! !!!! according to RFC 4566 there should be no need to include a sendrecv attribute as sendrecv is assumed per default; !!!! !!!! furthermore sendrecv can be at session or at media level !!!! */ return f_IMS_MOCallSetup_ReceiveUpdateSend200OK(p_InviteRequestWithSdp.Invite, v_ContactUrl, v_SdpOffer, p_GeolocationInfoAvailable, p_CallType, p_PreconditionIndication); }

After Change:

function f_IMS_MOCallSetup_NR5GC_A_4_1_Step6_7(IMS_InviteRequestWithSdp_Type p_InviteRequestWithSdp, charstring p_ContactUri := px_IMS_CalleeContactUri, boolean p_GeolocationInfoAvailable := false, IMS_CallType_Type p_CallType := NORMAL_CALL, IMS_SDP_AudioCodec_Type p_AudioCodec := EVS_16000, IMS_PreconditionIndication_Type p_PreconditionIndication := PreconditionRequired) runs on IMS_PTC return SDP_Message { var SipUrl v_ContactUrl := valueof(f_SIP_BuildSipUri_TX(p_ContactUri)); /* Contact address of the SS as assigned to the UE e.g. in the 183 Session Progress * NOTE 1: in other cases the SipUrl is taken from the 183 Session Progress as sent to the UE (f_MessageHeader_GetContactSipUrl) * => it is the exact value but not a template * NOTE 2: in general px_IMS_CalleeContactUri is used in the 183 Session Progress (i.e. we don't need to hand it over as parameter) */ var template (present) SDP_Message v_SdpOffer := f_IMS_BuildSDP_UpdatedOfferAudio_RX(p_InviteRequestWithSdp.SdpOffer, -, p_AudioCodec); //REMOVED v_SdpOffer.media_list[0].attributes := f_SDP_Attributes_Add_RX(v_SdpOffer.media_list[0].attributes, cs_SDP_Attribute_sendrecv); return f_IMS_MOCallSetup_ReceiveUpdateSend200OK(p_InviteRequestWithSdp.Invite, v_ContactUrl, v_SdpOffer, p_GeolocationInfoAvailable, p_CallType, p_PreconditionIndication); }