3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210398

# Online, 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-5 | CR | 0014 | rev | - | Current version: | 14.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of MCPTT test case 6.1.1.1 to the MCX ATS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | UPV/EHU |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add MCPTT TC 6.1.1.1 to the MCX ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to the TTCN code required to obtain a pass using the MCX IPCAN test model. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to the MCX ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Logs in R5s210399 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

3	Branches executed	5

4	Execution Log Files	5

4.1	Nemergent Solutions Client v7.0	5

5	References	6

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.1.1.1 which is part of the ATS iwd-TTCN3-B2020-09_D21wk24 test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Fidel Liberal
	fidel.liberal@ehu.eus

## Verification Test Summary

Test Case: 	6.1.1.1

ATS Version:	ATS  iwd-TTCN3-B2020-09_D21wk24

System Simulator used:	MCSTaaSting Tester

UE used:	Nemergent Client v7.0 on COTS LTE UE

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_MCX_SecurityParameters_Init |
| --- | --- |
| Reason for change | Keys are not fully initialized. |
| Summary of change | Keys are initialized. |
| TTCN module | MCX_SecurityParameters.ttcn |
| MCC160 Comment | Accepted |

- Before Change:

function f_MCX_SecurityParameters_Init(…) { … v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, GMK, v_GMK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, PCK, v_PCK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_CMS); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS1); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS2); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, TrK, cs_MCX_SecurityKey(px_MCX_TrK_ID, bit2oct(px_MCX_TrK))); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, InK, cs_MCX_SecurityKey(px_MCX_InK_ID, bit2oct(px_MCX_InK))); … }

- After Change:

function f_MCX_SecurityParameters_Init(…) { … v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, GMK, v_GMK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, PCK, v_PCK); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_CMS); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS1); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS2); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS3); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, CSK_GMS4); // initialised as omit v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, TrK, cs_MCX_SecurityKey(px_MCX_TrK_ID, bit2oct(px_MCX_TrK))); v_SecurityParameters := f_MCX_SecurityParameters_SetKey(v_SecurityParameters, InK, cs_MCX_SecurityKey(px_MCX_InK_ID, bit2oct(px_MCX_InK))); … }

## Change 2

| Function name | f_MCX_IPCAN_ReleaseConnection |
| --- | --- |
| Reason for change | In TC 6.1.1.1, at Step 45, f_MCX_IPCAN_ReleaseConnection is called without parameters, i.e. p_TestProcedure is omit, but exactly in this case the SSRC_UE must be released (its value will change in the new call setup) |
| Summary of change | Function f_MCX_IMS_PTC_SSRC_UE_Reset is called in the case p_TestProcedure is omit. |
| TTCN module | MCX_IMS_CommonProcedures.ttcn |
| MCC160 Comment | Accepted |

- Before Change:

function f_MCX_IPCAN_ReleaseConnection (…) { … if (isvalue(p_TestProcedure)) { f_MCX_IMS_PTC_SSRC_UE_Reset(); } f_IMS_IPCAN_EndProcedure(IPCAN, p_TestProcedure); }

- After Change:

function f_MCX_IPCAN_ReleaseConnection (…) { … if (not isvalue(p_TestProcedure)) { f_MCX_IMS_PTC_SSRC_UE_Reset(); } f_IMS_IPCAN_EndProcedure(IPCAN, p_TestProcedure); }

## Additional change by MCC160

function f_MCX_CallSetup_ConfigSRTP(MCX_CallType_Type p_CallType, IMS_TestProcedure_Type p_TestProcedure) runs on MCX_IMS_PTC { var MCX_CS_ID_Type v_CS_Id_RTP; select (p_CallType) { case (GROUP_CALL, CHAT_GROUP_CALL) { v_CS_Id_RTP := GMK_MCPTT; } case (PRIVATE_CALL) { select (p_TestProcedure) { case (IPCAN_MT_SpeechCall) { v_CS_Id_RTP := PCK_MCPTT_Receiver; } case (IPCAN_MO_SpeechCall) { v_CS_Id_RTP := PCK_MCPTT_Initiator; } case else { FatalError(__FILE__, __LINE__, ""); } } } } f_MCX_ConfigSRTP(v_CS_Id_RTP); // shall not be done before PCK is stored }

# Branches executed

- TLSv1.2

- IPSec ipsec-3gpp;alg=hmac-sha-1-96;ealg=aes-cbc

- XML integrity and encrpytion according to  http://www.w3.org/2009/xmlenc11#aes128-gcm

- XML signatures according to  http://www.w3.org/2001/04/xmlenc#sha256

- MIKEY SAKKE/ECCSI (RFC 6507, 6508, 6509)

- LTE Band 1

- EEA2/EIA2 RRC/NAS ciphering/integrity algorithms

# Execution Log Files

## Nemergent Solutions Client v7.0

The Nemergent Solutions Client v7.0 on COTS LTE UE passed this test case against the MCSTaaSting Tester. The documentation below is enclosed as evidence of the successful test case run [1]:

- SS_Log_Coversheet.txt

- Signalling-TC6.1.1.1.html (RRC/NAS signalling, IP flow decrypted/decoded)

- MCX-PIXIT.cfg (Titan configuration and PIXITs)

- TLI Interface log

# References

| [1] | R5s210399: Supporting information for the addition of MCPTT test case 6.1.1.1 to the MCX ATS |
| --- | --- |