3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210402

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1650 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 8.1.4.2.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A conformant UE may not include the EPS bearer context status IE in in TRACKING AREA UPDATE REQUEST at step 3 of this test case, based on TS 24.301 cl. 5.5.3.3.2: The UE shall include the EPS bearer context status IE in TRACKING AREA UPDATE REQUEST message: a) for the case g; b) for the case s; c) for the case zb; d) if the UE has established PDN connection(s) of "non IP" or Ethernet PDN type; and e) if the UE: 1) locally deactivated at least one dedicated EPS bearer context upon an inter-system mobility from WB-S1 mode to NB-S1 mode in EMM-IDLE mode; 2) locally deactivated at least one dedicated EPS bearer context upon an inter-system change from WB-N1 mode to NB-S1 mode in EMM-IDLE mode for the UE operating in single-registration mode (see subclause 6.4.2.1); or 3) locally deactivated at least one default EPS bearer context upon an inter-system change from N1 mode to NB-S1 mode in EMM-IDLE mode for the UE operating in single-registration mode (see subclause 6.5.0). The scenario in step 3 of this test case corresponds to case zd “when the UE performs inter-system change from N1 mode to S1 mode in EMM-CONNECTED mode”, so it is not mandated to include the EPS bearer context status IE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add “ifpresent” to allow EPS bearer context status to be omitted at step 3 Note : An associated prose CR on 38.523-1 will be raised at RAN5#91 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail some NR5GC IRAT test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.4.2.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | A conformant UE may not include the EPS bearer context status IE in in TRACKING AREA UPDATE REQUEST at step 3 of this test case, based on TS 24.301 cl. 5.5.3.3.2: The UE shall include the EPS bearer context status IE in TRACKING AREA UPDATE REQUEST message: a) for the case g; b) for the case s; c) for the case zb; d) if the UE has established PDN connection(s) of "non IP" or Ethernet PDN type; and e) if the UE: 1) locally deactivated at least one dedicated EPS bearer context upon an inter-system mobility from WB-S1 mode to NB-S1 mode in EMM-IDLE mode; 2) locally deactivated at least one dedicated EPS bearer context upon an inter-system change from WB-N1 mode to NB-S1 mode in EMM-IDLE mode for the UE operating in single-registration mode (see subclause 6.4.2.1); or 3) locally deactivated at least one default EPS bearer context upon an inter-system change from N1 mode to NB-S1 mode in EMM-IDLE mode for the UE operating in single-registration mode (see subclause 6.5.0). The scenario in step 3 of this test case corresponds to case zd “when the UE performs inter-system change from N1 mode to S1 mode in EMM-CONNECTED mode”, so it is not mandated to include the EPS bearer context status IE. |
| Summary of change | Add “ifpresent” to allow EPS bearer context status to be omitted at step 3 Note : An associated prose CR on 38.523-1 will be raised at RAN5#91 |
| TTCN module | IRAT_Handover_NR5GC_EUTRA |
| MCC160 Comment | Accepted subject to prose CR approval at RAN5 |

Before Change

function fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { ……………….. // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), // @sic R5-201638 sic@ v_SecurityRx.KSI, // @sic R5s200509 sic@ cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, // @sic R5s200509 sic@ -, tsc_NasKsi_MappedSecurityContext, // @sic R5-201215, R5s200509 sic@ cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent ))))-> value v_ReceivedMsg; // @sic R5s200509, R5s200968, Rs5200760, R5s210016 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @sic R5s191111 change 9 sic@ // @siclog "Step 6" siclog@ f_EUTRA_LoopbackIPdataOnDefaultBearer(eutra_Cell1, PDCP, v_DRBIdList[0], "Step 6"); // @sic R5-210367 sic@ }

After Change

function fl_TC_8_1_4_2_1_1_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { …………… // @siclog "Step 3" siclog@ SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected, cr_TAU_Request_N1ToS1((tsc_EpsUpdate_TaUpdate, tsc_EpsUpdate_Combined_IMSIAttach, tsc_EpsUpdate_Combined_TaLaUpdate), // @sic R5-201638 sic@ v_SecurityRx.KSI, // @sic R5s200509 sic@ cr_DRXparameter_Any ('5C'O) ifpresent, v_AdditionalUpdateType, v_MappedGUTI, v_TAI ifpresent, // @sic R5s200509 sic@ -, tsc_NasKsi_MappedSecurityContext, // @sic R5-201215, R5s200509 sic@ cdr_NAS_KeySetIdentifier_tlv (?, ?) ifpresent, -,-, cr_EPS_BearerContextStatus_Any ifpresent ))))-> value v_ReceivedMsg; // @sic R5s200509, R5s200968, Rs5200760, R5s210016 sic@ // @siclog "Step 4-5" siclog@ f_EUTRA38_TrackingAreaUpdateFromN1_WithoutReq (eutra_Cell1, v_ReceivedMsg.Signalling.Nas[0], -, noRrcConnectionRelease, -, true); // @sic R5s191111 change 9 sic@ // @siclog "Step 6" siclog@ f_EUTRA_LoopbackIPdataOnDefaultBearer(eutra_Cell1, PDCP, v_DRBIdList[0], "Step 6"); // @sic R5-210367 sic@ }