3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210407

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1655 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC common function |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | If an empty list is passed in p_DrbInfoList, the value of c_RadioBearerList can be returned uninitialized. This can cause a run-time error in the calling function, e.g. inf_NR_SendRRCResume_Def() |  |  |  |  |  |  |  |  |
| Summary of change: |  | Initialise v_RadioBearerList |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | e.g. 6.4.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_NR5GC_GetRadioBearerList() |
| --- | --- |
| Reason for change | If an empty list is passed in p_DrbInfoList, the value of c_RadioBearerList can be returned uninitialized. This can cause a run-time error in the calling function, e.g. inf_NR_SendRRCResume_Def() |
| Summary of change | Inisialise v_RadioBearerList |
| TTCN module | Rejected |
| MCC160 Comment | NR5GC_RRCSteps.ttcn |

Before Change

function f_NR5GC_GetRadioBearerList(template (value) NR_DRBInfoList_Type p_DrbInfoList, boolean p_ExcludeSCG_DRB := false ) return template (value) NR_RadioBearerList_Type { var template (value) NR_RadioBearerList_Type v_RadioBearerList; var integer i; for (i:= 0; i < lengthof(p_DrbInfoList); i:=i+1) { if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false)) { v_RadioBearerList[i] := p_DrbInfoList[i].SS_Config; } } return v_RadioBearerList; }

After Change

function f_NR5GC_GetRadioBearerList(template (value) NR_DRBInfoList_Type p_DrbInfoList, boolean p_ExcludeSCG_DRB := false ) return template (value) NR_RadioBearerList_Type { var template (value) NR_RadioBearerList_Type v_RadioBearerList := {}; var integer i; for (i:= 0; i < lengthof(p_DrbInfoList); i:=i+1) { if (not(p_ExcludeSCG_DRB and valueof(p_DrbInfoList[i].DC_SCG) == false)) { v_RadioBearerList[i] := p_DrbInfoList[i].SS_Config; } } return v_RadioBearerList; }

MCC160 Alternative Solution:

| Function name | f_TC_6_4_1_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | The call to f_NR5GC_MobileInfo_Init before the RRC Resume message is sent removes, among other things, the list of configured DRBs. This results in the RRC Resume message not including the RadioBearerConfig, which is not according to 38.508-1. |
| Summary of change | Replace call to f_NR5GC_MobileInfo_Init() with f_NR5GC_MobileInfo_SetSessionInfoList({}) |
| TTCN module | Inactive_PLMNSelection_NR5GC |

function f_TC_6_4_1_1_NR5GC_TestBody() runs on NR5GC_PTC

{// Power levels on FR2 are FFS

// @sic R5s201047, R5-203382, R5s210094, R5-210062 sic@

. . .

// @siclog "Step 0D" siclog@

// The steps specified in generic procedure TS 38.508-1 Table 4.5.3.2-1 are performed

v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell12);

v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN);

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_13(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity);

v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure,

v_GMM_MobilityInfo,

f_NR5GC_GetMobileIdGUTI (nr_Cell12),

cs_NG_TAIListNonConsecutive(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12)),

{bit2oct(f_NR_CellInfo_GetTAC(nr_Cell12))}),

cs_PLMN_List_1PLMN(f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13))));  // Table 6.4.1.1.3.3-3

SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell12,

tsc_NR_RbId_SRB1, // No SRB2 so must be on SRB1

cs_TimingInfo_Now,

cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend)));

f_NR5GC_RRC_Idle_Steps15_20(nr_Cell12, TESTMode_OFF, noRrcConnectionRelease);

f_NR_RRC_InactiveDef(nr_Cell12, -, false);

f_NR_SS_AS_ActivateSecurity_Current(nr_Cell13 ,tsc_NR_38508_NextHopChainingCount); // @sic R5s200657 sic@

f_NR5GC_MobileInfo_SetSessionInfoList({});

// @siclog "Step 1" siclog@

// SS adjusts cell levels according to row T1 of table 6.4.1.1.3.2-1/2

f_NR_SetCellPowerList(v_CellPowerList_T1);

. . .