3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210409

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1657 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR idle mdoe test case 6.4.3.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Postamble is performed in end state E-UTRA connected (E2_T3440). According to TS 36.523-3, Table 10.3.3.2-1, SS sends RRCConnectionReelease and waits for 5s to ensure UE goes to RRC_IDLE state before sending SWICTH OFF command. However. UE may try to perform deregistration in NR cell 1 due to the Power configuration at T2 1. NR Cell 1 priority higher than E-UTRA Cell 3 2. SrxlevNR Cell 1 = -98 - （-110 +0） - 0-1 = 11> ThreshNR Cell 1, HighP (2 * 2) If the user takes slightly longer to Switch OFF the UE, then the UE may try to camp on to NR Cell |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed threshX-High in SIB24 of E-UTRA Cell 3 to 10 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_3_1_NR5GC_EUTRA |
| --- | --- |
| Reason for change | Postamble is performed in end state E-UTRA connected (E2_T3440). According to TS 36.523-3, Table 10.3.3.2-1, SS sends RRCConnectionReelease and waits for 5s to ensure UE goes to RRC_IDLE state before sending SWICTH OFF command. However. UE may try to perform deregistration in NR cell 1 due to the Power configuration at T2 1. NR Cell 1 priority higher than E-UTRA Cell 3 2. SrxlevNR Cell 1 = -98 - （-110 +0） - 0-1 = 11> ThreshNR Cell 1, HighP (2 * 2) If the user takes slightly longer to Switch OFF the UE, then the UE may try to camp on to NR Cell |
| Summary of change | Changed threshX-High in SIB24 of E-UTRA Cell 3 to 10 |
| TTCN module | Inactive_CellReSelection_NR5GC_EUTRA |
| MCC160 Comment | Accepted subject to prose CR approval at RAN5#91e |

Before Change

function f_TC_6_4_3_1_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection From NR RRC_INACTIVE to E-UTRA RRC_IDLE (lower priority & higher priority, Srxlev based) // @sic R5-200366 sic@ f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508 Table 4.4.3.1.1-1 is used in E-UTRA cell 1 and E-UTRA cell 3. f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell1, -78); f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell3, -76); // @sic R5s191148 Ch. 2 sic@ // Table 6.4.3.1.3.3-3: SystemInformationBlockType24 for E-UTRA Cell 1 f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); //Preamble: finishes with EUTRA cell off (NR cell on) and co-ord msg received to say UE has completed registration on NR f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell3, E2_T3440); // @sic R5-204475 R5-211022 sic@ // take down the NR cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function f_TC_6_4_3_1_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { // Inter-RAT cell reselection From NR RRC_INACTIVE to E-UTRA RRC_IDLE (lower priority & higher priority, Srxlev based) // @sic R5-200366 sic@ f_EUTRA38_IRAT_Init(c31); // System information combination 31 as defined in TS 36.508 Table 4.4.3.1.1-1 is used in E-UTRA cell 1 and E-UTRA cell 3. f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell1, -78); f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell3, -76); // @sic R5s191148 Ch. 2 sic@ // Table 6.4.3.1.3.3-3: SystemInformationBlockType24 for E-UTRA Cell 1 f_EUTRA_CellInfo_SetSIB24_t_ReselectionNR_r15(eutra_Cell1, 7); f_EUTRA_CellInfo_SetSIB24_ThreshX_High_r15(eutra_Cell3, 10); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); //Preamble: finishes with EUTRA cell off (NR cell on) and co-ord msg received to say UE has completed registration on NR f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_6_4_3_1_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell3, E2_T3440); // @sic R5-204475 R5-211022 sic@ // take down the NR cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }