3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210410

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1658 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC IRAT test case 6.2.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - String for AT command is incorrect - According 38.523-1, test case procedure should make sure the Last Registered PLMN has been cleared in UE before test body. For state 0N-B preamble, UE shall camp to cell. So f_UT_ClearRPLMN should be called after preamble. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Changed ‘,’ to ‘=’ in the AT command string - Moved f_UT_ClearRPLMN after preamble |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | cs_AT_UPDATE_USIM |
| --- | --- |
| Reason for change | String for AT command is incorrect |
| Summary of change | Changed ‘,’ to ‘=’ in the AT command string |
| TTCN module | UpperTesterSystemCmds |
| MCC160 Comment | Accepted |

Before Change

template (value) Ut_System_Cmd_Type cs_AT_UPDATE_USIM (charstring p_Params) := cs_AT_Cmd("AT+CRSM," & p_Params & "<CR>");

After Change

template (value) Ut_System_Cmd_Type cs_AT_UPDATE_USIM (charstring p_Params) := cs_AT_Cmd("AT+CRSM=" & p_Params & "<CR>");

## Change 2

| Function name | f_TC_6_2_1_2_NR5GC |
| --- | --- |
| Reason for change | According 38.523-1, test case procedure should make sure the Last Registered PLMN has been cleared in UE before test body. For state 0N-B preamble, UE shall camp to cell. So f_UT_ClearRPLMN should be called after preamble. |
| Summary of change | Moved f_UT_ClearRPLMN after preamble |
| TTCN module | Idle_PLMNSelection_NR5GC_NR |
| MCC160 Comment | State 0N-B assigns the GUTI, security context etc. These are then immediately cleared with the USIM configuration in f_UT_ClearRPLMN. Prose CR required to clarify pre-test variables required. Same problem in 6.2.1.3. Decision postponed until outcome of prose CR. |

Before Change

function f_TC_6_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode var DeltaValues_Type v_DeltaPrimaryBand; f_NR5GC_IRAT_Init(NR_1); // Set Sys info and cell info for cell 1& 12 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13 //FFS pending prose CR for update on values of Q_RxLevMinInitial_DB on FR2 if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); } if (not f_NR_CellInfo_GetIsFR1(nr_Cell12)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell12, -91.0, v_DeltaPrimaryBand.DeltaNRf2); } // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // Equip UE with the USIM configuration 7 specified in table 6.4.1-7 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 7 specified in table 6.4.1-7 TS 38.508-1"); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); // Ensure that the UE has cleared the RPLMN @sic R5-211378 sic@ f_UT_ClearRPLMN(UT); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); // @sic R5-211378 sic@ // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_6_2_1_2_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change

function f_TC_6_2_1_2_NR5GC() runs on NR5GC_PTC { // Inter-RAT PLMN Selection / Selection of correct RAT for UPLMN / Automatic mode var DeltaValues_Type v_DeltaPrimaryBand; f_NR5GC_IRAT_Init(NR_1); // Set Sys info and cell info for cell 1& 12 to be used f_NR_CellInfo_SetPLMN(nr_Cell1, tsc_NR_HPLMN_002_11);//PLMN2 f_NR_CellInfo_SetPLMN(nr_Cell12, tsc_NR_HPLMN_002_81);//PLMN13 //FFS pending prose CR for update on values of Q_RxLevMinInitial_DB on FR2 if (not f_NR_CellInfo_GetIsFR1(nr_Cell1)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell1, -91.0, v_DeltaPrimaryBand.DeltaNRf1); } if (not f_NR_CellInfo_GetIsFR1(nr_Cell12)) { v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); f_NR_CellInfo_SetAndCalculateQ_RxLevMin(nr_Cell12, -91.0, v_DeltaPrimaryBand.DeltaNRf2); } // Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR_CellConfig_Def(nr_Cell12); // Equip UE with the USIM configuration 7 specified in table 6.4.1-7 TS 38.508-1 f_UT_USIM_Insert(UT, "USIM configuration 7 specified in table 6.4.1-7 TS 38.508-1"); f_UT_SwitchOnUE(UT, false, CNF_REQUIRED); // Ensure that the UE has cleared the RPLMN @sic R5-211378 sic@ // REMOVE f_UT_ClearRPLMN(UT); f_UT_AutomaticPLMN_Select (UT); f_UT_SwitchOffUE(UT, false, CNF_REQUIRED); // Preamble: The UE is in state Switched OFF (state 0N-B) according to TS 38.508-1 f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); // @sic R5-211378 sic@ // Ensure that the UE has cleared the RPLMN @sic R5-211378 sic@ f_UT_ClearRPLMN(UT); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_6_2_1_2_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }