3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210413

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0681 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for IMS NR5GC test case 7.10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The parameter p_PreconditionsRequired needs to be passed into f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress() to control the inclusion of precondition tag in expected contents for 183 Session Progress. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Pass precondition flag argument in call to f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress() |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.10.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6() |
| --- | --- |
| Reason for change | The parameter p_PreconditionsRequired needs to be passed into f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress() to control the inclusion of precondition tag in expected contents for 183 Session Progress. |
| Summary of change | Pass precondition flag argument in call to f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress() |
| TTCN module | IMS_NR5GC_CallControlTestcases.ttcn |
| MCC160 Comment | Done for R5s210393 already |

Before Change:

function f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6(boolean p_PreconditionsRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* NOTE: the returned InviteRequestWithSdp contains the SDP offer sent by the UE in the 183 Session Progress */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; // Step 2: INVITE v_AudioCodecList := {}; // => no SDP v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(p_PreconditionsRequired, v_AudioCodecList); // Step 3 - 4: 100 Trying, 183 Session Progress v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_RX(p_PreconditionsRequired, v_AudioCodecList); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol); f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // Step 5: PRACK v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_ResponseWithSdp.SdpMessage, p_PreconditionsRequired, EVS_16000, 65); v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_MessageHeader_Prack := f_IMS_PrackRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader, cs_ContentTypeSDP); IMS_Client.send(cas_IMS_Prack_Request(cs_IMS_RoutingInfo(v_Protocol), cs_PRACK_Request(v_ContactUrl, v_MessageHeader_Prack, cs_MessageBody_SDP(v_SdpAnswer)))); // Step 6: 200 OK a_IMS_ReceiveResponse200OK(v_MessageHeader_Prack, v_Protocol); // Establish Dedicated EPS bearer f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); /* trigger IPCAN to continue with activation of secondary PDP context or dedicated EPS bearer*/ v_InviteRequestWithSdp.SdpOffer := v_ResponseWithSdp.SdpMessage; return v_InviteRequestWithSdp; }

After Change:

function f_IMS_MTCallSetup_NoSdpInINVITE_Step1_6(boolean p_PreconditionsRequired) runs on IMS_PTC return IMS_InviteRequestWithSdp_Type { /* NOTE: the returned InviteRequestWithSdp contains the SDP offer sent by the UE in the 183 Session Progress */ var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var IMS_ResponseWithSdp_Type v_ResponseWithSdp; var template (present) SDP_Message v_SdpOffer; var template (value) MessageHeader v_MessageHeader_Prack; var template (value) SDP_Message v_SdpAnswer; var IMS_SDP_AudioCodecList_Type v_AudioCodecList; var boolean v_CheckForbiddenCodecParameters := true; var InternetProtocol_Type v_Protocol; var SipUrl v_ContactUrl; // Step 2: INVITE v_AudioCodecList := {}; // => no SDP v_InviteRequestWithSdp := f_IMS_MTCallSetup_NR5GC_A_5_X_Step1(p_PreconditionsRequired, v_AudioCodecList); // Step 3 - 4: 100 Trying, 183 Session Progress v_AudioCodecList := tsc_IMS_SDP_AudioCodecList_EVS_WBAMR_AMR; v_SdpOffer := f_IMS_BuildSDP_InitialOfferAudio_RX(p_PreconditionsRequired, v_AudioCodecList); v_ResponseWithSdp := f_IMS_MTCallSetup_Receive100Trying_Receive183SessionProgress(v_InviteRequestWithSdp.Invite, v_SdpOffer, v_InviteRequestWithSdp.RoutingInfo.Protocol, p_PreconditionsRequired); f_IMS_SDP_CheckMediaDescr_AudioCodecs(v_ResponseWithSdp.SdpMessage.media_list[0], v_AudioCodecList, v_CheckForbiddenCodecParameters); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // Step 5: PRACK v_SdpAnswer := f_IMS_BuildSDP_InitialAnswerAudio_TX(v_ResponseWithSdp.SdpMessage, p_PreconditionsRequired, EVS_16000, 65); v_Protocol := f_IMS_PTC_ImsInfo_GetTransportProtocol(); v_ContactUrl := f_IMS_PTC_ImsInfo_GetContactUrl(dialog); v_MessageHeader_Prack := f_IMS_PrackRequest_MessageHeaderTX(v_InviteRequestWithSdp.Invite, v_ResponseWithSdp.Response.msgHeader, cs_ContentTypeSDP); IMS_Client.send(cas_IMS_Prack_Request(cs_IMS_RoutingInfo(v_Protocol), cs_PRACK_Request(v_ContactUrl, v_MessageHeader_Prack, cs_MessageBody_SDP(v_SdpAnswer)))); // Step 6: 200 OK a_IMS_ReceiveResponse200OK(v_MessageHeader_Prack, v_Protocol); // Establish Dedicated EPS bearer f_IMS_CC_TriggerDedicatedBearerActivation(IPCAN_MT_SpeechCall); /* trigger IPCAN to continue with activation of secondary PDP context or dedicated EPS bearer*/ v_InviteRequestWithSdp.SdpOffer := v_ResponseWithSdp.SdpMessage; return v_InviteRequestWithSdp; }