3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210414

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1661 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Re-verification of NR5GC IRAT test case 11.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To re-verify NR5GC IRAT test case 11.1.1 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC IRAT test case 11.1.1 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

2.3	Change 3	6

2.4	Change 4	10

2.5	Change 5	11

2.6 Change 6	13

2.7	Change 7	14

3	Branches executed	16

3	Execution Log Files	16

3.1	Qualcomm SM8350 + SDX60	16

4	References	16

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.1 which is part of the 5G NRGC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.1.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset and Anritsu Protocol Conformance Test System ME7834NR

UE used:	Qualcomm SM8350 + SDX60

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_EUTRA38_MobileInfo_Init() |
| --- | --- |
| Reason for change | The list vc_EUTRA_Global.MobileInfo.PdnInfo.EpsBearerContextList is used in function f_EUTRA38_SendPDuSessionInfotToNR(), see also changes 3 and 4 |
| Summary of change | Initialise vc_EUTRA_Global.MobileInfo.PdnInfo.EpsBearerContextList |
| TTCN module | EUTRA_Component_Multi5GS.ttcn |
| MCC160 Comment | Accepted. Also removed definition and initialisation of vc_EUTRA_Multi5GS_Global.MobileInfo.ConfiguredPDNs |

Before Change

function f_EUTRA38_MobileInfo_Init() runs on EUTRA_Multi5GS_PTC { vc_EUTRA_Multi5GS_Global.MobileInfo.ConfiguredPDNs := {}; vc_EUTRA_Multi5GS_Global.MobileInfo.ListOfDRBs := {}; }

After Change

function f_EUTRA38_MobileInfo_Init() runs on EUTRA_Multi5GS_PTC { vc_EUTRA_Multi5GS_Global.MobileInfo.ConfiguredPDNs := {}; vc_EUTRA_Multi5GS_Global.MobileInfo.ListOfDRBs := {}; vc_EUTRA_Global.MobileInfo.PdnInfo.EpsBearerContextList := {}; }

## Change 2

| Function name | f_EUTRA38_MultiPDN_ConfigureDRB() |
| --- | --- |
| Reason for change | Incomplete implementation of R5s210157. From R5s210157: During DRB activation (from function f_EUTRA38_MultiPDN_InitialRegistration_Step14_15()), ciphering is not being activated on the new DRB.This causes test case failure during ICMPv6 exchange for IPv6 address allocation. |
| Summary of change | Update v_SecurityParams with new actTimeList including new DRB |
| TTCN module | EUTRA38_NR5GC_InitialRegistration.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_EUTRA38_MultiPDN_ConfigureDRB(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB := true) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {}; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var integer v_NewDRBId := f_EUTRA38_GetNextDRBId (p_PDN_Type, p_DefaultDRB); var integer v_BearerId := v_NewDRBId + 4; // EPS Bearer id = 4 + DRB Id var RadioBearer_Type v_SS_Config := valueof(cs_OneDRB_ConfigAM(v_NewDRBId)); // Add this to new bearer (always an AM bearer) to the list v_NewDRBInfoList := {cs_DRB_Info (v_NewDRBId, v_BearerId, true, v_SS_Config, valueof(cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId)), p_PDN_Type)}; // @sic R5-206296 sic@ if (v_NoOfDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList)); } else { v_ExistingDRBs := valueof(v_NewDRBInfoList); } f_EUTRA38_MobileInfo_SetDRBInfoList (v_ExistingDRBs); f_EUTRA_SS_CommonRadioBearerConfig(p_CellId, {v_SS_Config}); // now DRB is configured, security must be enabled f_EUTRA_RRC_CipherActTime_GetCurrent(p_CellId); // @sic R5s210157 sic@ f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(p_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); return int2hex(v_BearerId, 1); }

After Change

function f_EUTRA38_MultiPDN_ConfigureDRB(EUTRA_CellId_Type p_CellId, PDU_PDN_DNN_Type p_PDN_Type, boolean p_DefaultDRB := true) runs on EUTRA_Multi5GS_PTC return EPS_BearerIdentity { var integer v_NoOfDRBs := f_EUTRA38_MobileInfo_GetNoOfDRBs(); var EUTRA38_DRBInfoList_Type v_ExistingDRBs; var template (value) EUTRA38_DRBInfoList_Type v_NewDRBInfoList := {}; var EUTRA_SecurityParams_Type v_SecurityParams := f_EUTRA_Security_Get(); var integer v_NewDRBId := f_EUTRA38_GetNextDRBId (p_PDN_Type, p_DefaultDRB); var integer v_BearerId := v_NewDRBId + 4; // EPS Bearer id = 4 + DRB Id var RadioBearer_Type v_SS_Config := valueof(cs_OneDRB_ConfigAM(v_NewDRBId)); // Add this to new bearer (always an AM bearer) to the list v_NewDRBInfoList := {cs_DRB_Info (v_NewDRBId, v_BearerId, true, v_SS_Config, valueof(cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId)), p_PDN_Type)}; // @sic R5-206296 sic@ if (v_NoOfDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList)); } else { v_ExistingDRBs := valueof(v_NewDRBInfoList); } f_EUTRA38_MobileInfo_SetDRBInfoList (v_ExistingDRBs); f_EUTRA_SS_CommonRadioBearerConfig(p_CellId, {v_SS_Config}); // now DRB is configured, security must be enabled v_SecurityParams.AS_Ciphering.ActTimeList := f_EUTRA_RRC_CipherActTime_GetCurrent(p_CellId); // @sic R5s210157 sic@ f_EUTRA_SS_RRC_EnableIntProt_CiphULandDL(p_CellId, v_SecurityParams.AS_Integrity, v_SecurityParams.AS_Ciphering); return int2hex(v_BearerId, 1); }

## Change 3

| Function name | f_EUTRA38_TAUReqFromN1() |
| --- | --- |
| Reason for change | The list vc_EUTRA_Global.MobileInfo.PdnInfo.EpsBearerContextList is used in function f_EUTRA38_SendPDuSessionInfoToNR(). If this is incorrect, test case can fail during IMS deregistration in postamble after going back to NR, e.g. for 11.1.1. |
| Summary of change | Store EPS bearer IDs received from NR PTC in EUTRA_MobileInfo |
| TTCN module | EUTRA38_NR5GC_Procedures.ttcn |
| MCC160 Comment | Accepted, but only for the transferred bearers. Also removed definition and use of ConfiguredPDNInfoList. Same changes made in f_Encode_RRCConnectionReconfiguration_HO5GToEUTRA |

Before Change

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, template MobileIdentity p_MobileId := omit, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type { ……………….. // @sic R5-204399, R5s201390, R5-206289 sic@ v_EPS_BearerContextStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; v_TransferredBearers := f_EUTRA38_NR_ConvertEPS_BearerContextStatus(v_EPS_BearerContextStatus); // Store EPS bearers transferred, if not stored already for (i:=0; i < lengthof(v_PduSessionInfoList); i :=i+1) { if (f_IntegerList_Search(v_TransferredBearers, v_PduSessionInfoList[i].EPS_Bearer) != -1) { // don't store if not in transferred list @sic R5-206289 sic@ v_ExistingPDNs[i].BearerId := v_PduSessionInfoList[i].EPS_Bearer; v_ExistingPDNs[i].PDUSessionId := v_PduSessionInfoList[i].PDU_SessionId; v_ExistingPDNs[i].PdnIndex := v_PduSessionInfoList[i].PdnIndex; v_ExistingPDNs[i].IMS := v_PduSessionInfoList[i].IsIMS; v_ExistingPDNs[i].LinkedToDefaultBearer := 0; // not passed from NR v_ExistingPDNs[i].PCO := omit; // not passed from NR v_NewDRBId := (v_PduSessionInfoList[i].EPS_Bearer - 4); if (f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, v_NewDRBId) == -1) { // add new DRB to list v_NewDRBInfoList[j] := cs_DRB_Info (v_NewDRBId, (v_NewDRBId+4), false, cs_OneDRB_ConfigAM(v_NewDRBId), cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId), f_GetMultiPdnTypeFromIndex(v_PduSessionInfoList[i].PdnIndex)); // @sic R5s210305 sic@ j := j+1; } } // now store the full list, but don't configure any if (v_NoOfDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList)); } else { v_ExistingDRBs := valueof(v_NewDRBInfoList); } f_EUTRA38_MobileInfo_SetConfiguredPDNInfoList(v_ExistingPDNs); // @sic R5s201390 sic@ f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs)); // Now that we have the security parameters we can check the KSI received if (not match(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.asmeNasKeySetId.nasKeySetId, v_SecurityRx.KSI)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU Req KSI failed"); } v_SecurityParams := f_EUTRA38_MappedContextFromN1_InitNAS(p_CellId, v_SecurityRx, false); f_EUTRA_Security_Set(v_SecurityParams); return v_NasInd; }

After Change

function f_EUTRA38_TAUReqFromN1(EUTRA_CellId_Type p_CellId, RRC_TransactionIdentifier p_RRC_TI, template MobileIdentity p_MobileId := omit, N1ToS1_SecurityContext_Type p_CxtType := MAPPED_CXT, template EUTRA_CellId_Type p_CellIdPreviouslyCampedOn := eutra_Cell_NonSpecific, template (present) EstablishmentCause p_EstablishmentCause := mo_Signalling, boolean p_FirstN1_S1Transition := true) runs on EUTRA_Multi5GS_PTC return NAS_MSG_Indication_Type { ……………….. // @sic R5-204399, R5s201390, R5-206289 sic@ v_EPS_BearerContextStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus; v_TransferredBearers := f_EUTRA38_NR_ConvertEPS_BearerContextStatus(v_EPS_BearerContextStatus); // Store EPS bearers transferred, if not stored already for (i:=0; i < lengthof(v_PduSessionInfoList); i :=i+1) { if (f_IntegerList_Search(v_TransferredBearers, v_PduSessionInfoList[i].EPS_Bearer) != -1) { // don't store if not in transferred list @sic R5-206289 sic@ v_ExistingPDNs[i].BearerId := v_PduSessionInfoList[i].EPS_Bearer; v_ExistingPDNs[i].PDUSessionId := v_PduSessionInfoList[i].PDU_SessionId; v_ExistingPDNs[i].PdnIndex := v_PduSessionInfoList[i].PdnIndex; v_ExistingPDNs[i].IMS := v_PduSessionInfoList[i].IsIMS; v_ExistingPDNs[i].LinkedToDefaultBearer := 0; // not passed from NR v_ExistingPDNs[i].PCO := omit; // not passed from NR v_NewDRBId := (v_PduSessionInfoList[i].EPS_Bearer - 4); if (f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, v_NewDRBId) == -1) { // add new DRB to list v_NewDRBInfoList[j] := cs_DRB_Info (v_NewDRBId, (v_NewDRBId+4), false, cs_OneDRB_ConfigAM(v_NewDRBId), cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId), f_GetMultiPdnTypeFromIndex(v_PduSessionInfoList[i].PdnIndex)); // @sic R5s210305 sic@ j := j+1; } f_EUTRA_MobileInfo_SetEPS_BearerContext(int2hex(v_PduSessionInfoList[i].EPS_Bearer, 1),v_PduSessionInfoList[i] ); } // now store the full list, but don't configure any if (v_NoOfDRBs > 0) { v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList(); v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList)); } else { v_ExistingDRBs := valueof(v_NewDRBInfoList); } f_EUTRA38_MobileInfo_SetConfiguredPDNInfoList(v_ExistingPDNs); // @sic R5s201390 sic@ f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs)); // Now that we have the security parameters we can check the KSI received if (not match(v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.asmeNasKeySetId.nasKeySetId, v_SecurityRx.KSI)) { f_EUTRA_SetVerdictFailOrInconc(__FILE__, __LINE__, "TAU Req KSI failed"); } v_SecurityParams := f_EUTRA38_MappedContextFromN1_InitNAS(p_CellId, v_SecurityRx, false); f_EUTRA_Security_Set(v_SecurityParams); return v_NasInd; }

MCC160 Implementation:

// @sic R5-204399, R5s201390, R5-206289 sic@

v_EPS_BearerContextStatus := v_NasInd.Pdu.Msg.tRACKING_AREA_UPDATE_REQUEST.epsBearerContextStatus;

v_TransferredBearers := f_EUTRA38_NR_ConvertEPS_BearerContextStatus(v_EPS_BearerContextStatus);

// Store EPS bearers transferred, if not stored already

for (i:=0;  i < lengthof(v_PduSessionInfoList); i :=i+1) {

if (f_IntegerList_Search(v_TransferredBearers, v_PduSessionInfoList[i].EPS_Bearer) != -1) {

// don't store if not in transferred list @sic R5-206289 sic@

v_NewDRBId := (v_PduSessionInfoList[i].EPS_Bearer - 4);

if (f_EUTRA38_DRBInfo_FindEntry(v_ExistingDRBs, v_NewDRBId) == -1) {

// add new DRB to list

v_NewDRBInfoList[j] := cs_DRB_Info (v_NewDRBId, (v_NewDRBId+4), false, cs_OneDRB_ConfigAM(v_NewDRBId), cs_508_DRB_ToAddMod_DEFAULT_AM(v_NewDRBId), f_GetMultiPdnTypeFromIndex(v_PduSessionInfoList[i].PdnIndex));  // @sic R5s210305 sic@

j := j+1;

f_EUTRA_MobileInfo_SetEPS_BearerContext(int2hex(v_PduSessionInfoList[i].EPS_Bearer, 1),v_PduSessionInfoList[i]);

}

}

}

// now store the full list, but don't configure any

if (v_NoOfDRBs > 0) {

v_ExistingDRBs := f_EUTRA38_MobileInfo_GetDRBInfoList();

v_ExistingDRBs := f_EUTRA38_DRBInfo_AddListOfEntries (v_ExistingDRBs, valueof(v_NewDRBInfoList));

} else {

v_ExistingDRBs := valueof(v_NewDRBInfoList);

}

f_EUTRA38_MobileInfo_SetDRBInfoList (valueof(v_ExistingDRBs));

## Change 4

| Function name | f_EUTRA38_NR5GC_PreambleAndSwitchOff() |
| --- | --- |
| Reason for change | Following EUTRA registration in preamble of EPS fallback test cases, DRB info and EPS bearer info (see change 1) are not being cleared. This can lead to failure in the test case postamble due to incorrect PDU session information being transferred back to NR at the end of the test case. |
| Summary of change | Add call to f_EUTRA38_MobileInfo_Init() following UE switch off |
| TTCN module | EUTRA38_NR5GC_Procedures.ttcn |
| MCC160 Comment | Accepted in principle, but instead moved f_EUTRA38_MobileInfo_Init from f_EUTRA38_MultiPDN_Postamble to f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe |

Before Change

function f_EUTRA38_NR5GC_PreambleAndSwitchOff(EUTRA_CellId_Type p_CellId) runs on EUTRA_Multi5GS_PTC { // Attach on EUTRA f_EUTRA38_NR5GC_Preamble (p_CellId, TESTMode_OFF, RRC_IDLE, noRrcConnectionRelease); // Detach on Switch off f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe (p_CellId, RRC_CONNECTED); // @sic R5-204399 sic@ // Tell IMS PTC to stop waiting for De-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s210149 sic@ // Reset Routing Table f_IP_Handling_StopPDN(IP); // @sic R5s210149 sic@ // Turn Power Off f_EUTRA38_SetCellPower(p_CellId, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); // Tell NR to turn on and do registration f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Ok, UE now registered on both RATs }

After Change

function f_EUTRA38_NR5GC_PreambleAndSwitchOff(EUTRA_CellId_Type p_CellId) runs on EUTRA_Multi5GS_PTC { // Attach on EUTRA f_EUTRA38_NR5GC_Preamble (p_CellId, TESTMode_OFF, RRC_IDLE, noRrcConnectionRelease); // Detach on Switch off f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe (p_CellId, RRC_CONNECTED); // @sic R5-204399 sic@ // Tell IMS PTC to stop waiting for De-register f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); // @sic R5s210149 sic@ // Reset Routing Table f_IP_Handling_StopPDN(IP); // @sic R5s210149 sic@ // Turn Power Off f_EUTRA38_SetCellPower(p_CellId, tsc_NonSuitableOffCellRS_EPRE, tsc_NonSuitableOffCellRS_EPRE); f_EUTRA38_MobileInfo_Init(); // Tell NR to turn on and do registration f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Preamble")); f_EUTRA_NR_WaitForCoOrd_Trigger (NR, "Preamble"); // Ok, UE now registered on both RATs }

## Change 5

| Function name | f_NR5GC_SendPduSessionInfoListToEUTRA() |
| --- | --- |
| Reason for change | Currently QFI information is not sent to EUTRA as it is not needed. This causes an issue when the PDU session information is later sent from EUTRA back to NR and then IMS deregistration needs to be performed, e.g. in postamble of 11.1.1. |
| Summary of change | Set the QFI for IMS PDU session, to be stored in EUTRA PTC and sent back to NR PTC if needed. Note : This proposal just hardcodes QFI to 2 for IMS signalling, a full solution may need extension of GSM_MobilityInfo_Type |
| TTCN module | NR5GC_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Accepted in principle |

Before Change

function f_NR5GC_SendPduSessionInfoListToEUTRA() runs on NR5GC_PTC { var EUTRA_NR_PduSessionInfoList_Type v_CoordMsgList := {}; var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); var integer i; for (i := 0; i < lengthof(v_PDUSessionInfo); i := i + 1) { v_CoordMsgList[i].PDU_SessionId := oct2int(v_PDUSessionInfo[i].SessionId); v_CoordMsgList[i].EPS_Bearer := v_PDUSessionInfo[i].EPS_Bearer; v_CoordMsgList[i].QFI := 0; // Not needed by EUTRA v_CoordMsgList[i].PdnIndex := v_PDUSessionInfo[i].PdnIndex; v_CoordMsgList[i].IsIMS := match (v_PDUSessionInfo[i].PDUType, IMS_DNN); } f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_PduSessionInfoList(v_CoordMsgList)); }

After Change

function f_NR5GC_SendPduSessionInfoListToEUTRA() runs on NR5GC_PTC { var EUTRA_NR_PduSessionInfoList_Type v_CoordMsgList := {}; var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList(); var integer i; for (i := 0; i < lengthof(v_PDUSessionInfo); i := i + 1) { v_CoordMsgList[i].PDU_SessionId := oct2int(v_PDUSessionInfo[i].SessionId); v_CoordMsgList[i].EPS_Bearer := v_PDUSessionInfo[i].EPS_Bearer; v_CoordMsgList[i].QFI := 0; // Not needed by EUTRA v_CoordMsgList[i].PdnIndex := v_PDUSessionInfo[i].PdnIndex; v_CoordMsgList[i].IsIMS := match (v_PDUSessionInfo[i].PDUType, IMS_DNN); if(v_CoordMsgList[i].IsIMS) { v_CoordMsgList[i].QFI := 2; } } f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_PduSessionInfoList(v_CoordMsgList)); }

MCC160 Implementation:

function f_NR5GC_SendPduSessionInfoListToEUTRA() runs on NR5GC_PTC

{

var EUTRA_NR_PduSessionInfoList_Type v_CoordMsgList := {};

var PDUSessionInfoList_Type v_PDUSessionInfo := f_NR5GC_MobileInfo_GetSessionInfoList();

var integer i;

for (i := 0; i < lengthof(v_PDUSessionInfo); i := i + 1) {

v_CoordMsgList[i].PDU_SessionId := oct2int(v_PDUSessionInfo[i].SessionId);

v_CoordMsgList[i].EPS_Bearer := v_PDUSessionInfo[i].EPS_Bearer;

v_CoordMsgList[i].QFI :=valueof(f_NR_GetQosFlowIdentification_ForDRBId (f_NR_GetDefaultDRB_InDRBList (f_NR_GetDRBList_ForGivenPDUSession (v_CoordMsgList[i].PDU_SessionId))).QFI);

v_CoordMsgList[i].PdnIndex := v_PDUSessionInfo[i].PdnIndex;

v_CoordMsgList[i].IsIMS := match (v_PDUSessionInfo[i].PDUType, IMS_DNN);

}

f_EUTRA_NR_SendCoOrd(EUTRA, cms_EUTRA_NR_PduSessionInfoList(v_CoordMsgList));

}

## 2.6  Change 6

| Function name | f_TC_11_1_1_NR5GC_EUTRA() |
| --- | --- |
| Reason for change | RTP loopback needs to be enabled for the IMS voice call established during this test case. |
| Summary of change | Add HANDLE_UM_DATA to the f_EUTRA_Init() function which controls RTP loopback configuration. Note : Same change required for all other EPS fallback test cases |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_11_1_1_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { //MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success f_EUTRA38_IRAT_Init(c31); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_1_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // Test finishes in NR f_EUTRA_ReleaseAllCells(); }

After Change

function f_TC_11_1_1_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC // @sic R5-204399 sic@ { //MO MMTEL voice call setup from NR RRC_IDLE / EPS Fallback with redirection / Single registration mode with N26 interface / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_1_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); // Test finishes in NR f_EUTRA_ReleaseAllCells(); }

## Change 7

| Function name | fl_IMSFallback_PreambleRegistrations() |
| --- | --- |
| Reason for change | Trigger for IPCAN release from EUTRA PTC (sent from f_EUTRA38_NR5GC_PreambleAndSwitchOff() ) is currently not handled because IMS PTC is expecting on the wrong port. |
| Summary of change | Use function f_IMS_IpcanReleaseWithOptionalDeregistrationOhterIPCAN() instead of f_IMS_IpcanReleaseWithOptionalDeregistration() so that trigger from EUTRA PTC is handled. |
| TTCN module | EPS_Fallabck_NR5GC_IMS.ttcn |
| MCC160 Comment | Accepted |

Before Change

function fl_IMSFallback_PreambleRegistrations() runs on IMS_PTC { f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_Registration(OtherIPCAN_E); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function fl_IMSFallback_PreambleRegistrations() runs on IMS_PTC { f_IMS_PTC_Init(); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_Registration(OtherIPCAN_E); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistrationOhterIPCAN(); f_IMS_GlobalRelease(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_Registration(); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

# Branches executed

The test case was executed on EUTRAN<>NR band combinations 3<>n79 and 66<>n41.

# Execution Log Files

## Qualcomm SM8350 + SDX60

The Qualcomm SM8350 + SDX60 UE passed this test case on the Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_11_1_1_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_11_1_1_PIXIT.html

The Qualcomm SM8350 + SDX60 UE also passed this test case on the Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log:

Anritsu/ TC_11_1_1_LOGS.xml

- PICS/PIXIT parameter file:

Anritsu/TC_11_1_1_PIXIT.txt

# References

| [1] | R5s210473 : Supporting information for re-verification of NR5GC IRAT test case 11.1.1 |
| --- | --- |