3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210422

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1664 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to a_ApplicationCtrl |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In IRAT test case 8.1.1.3.2, function “f_IP_PTC_PDN_GetDrbInfoForDefaultDrb()” is called with the below flow f_EUTRA_UE_Detach_SwitchOffUe_Trigger f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, p_DRB, PDN_1); IP_ConfigReq::ChangeDrbInfo f_IP_PTC_PDN_GetDrbInfoForDefaultDrb In this test case PDN_3 ( internet PDN ) is used in association with DRB ID 2 . hence during PDN establishment procedure , IP CONFIG REQ : StartPDN is setting vc_IP_PTC_PDN_MappingTable[PDN_3].DefaultDrb. vc_IP_PTC_PDN_MappingTable[PDN_1].DefaultDrb and vc_IP_PTC_PDN_MappingTable[PDN_2].DefaultDrb are omit. With the function f_IP_ChangeDrbMappingEUTRA , the IP CONFIG REQ:ChangeDrbInfo is received for PDN_1 ; f_IP_PTC_PDN_GetDrbInfoForDefaultDrb is called with PDN_1 passed as paramter, which returns Omit. This leads to CompulsoryValueOmittedException run time Issue |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding a check condition to confirm whether vc_IP_PTC_PDN_MappingTable[PDN_Index] is having the values , before calling the function f_IP_PTC_PDN_GetDrbInfoForDefaultDrb with the PDN_Index |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Runtime error may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.1.3.2.NR5GC, 8.1.4.2.1.1.NR5GC, other IRAT testcases |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to altstep a_ApplicationCtrl	3

# Corrections required

## Correction to altstep a_ApplicationCtrl

| Altstep name | a_ApplicationCtrl |
| --- | --- |
| Reason for change | In IRAT test case 8.1.1.3.2, function “f_IP_PTC_PDN_GetDrbInfoForDefaultDrb()” is called with the below flow f_EUTRA_UE_Detach_SwitchOffUe_Trigger f_IP_ChangeDrbMappingEUTRA(IP, p_CellId, p_DRB, PDN_1); IP_ConfigReq::ChangeDrbInfo f_IP_PTC_PDN_GetDrbInfoForDefaultDrb In this test case PDN_3 ( internet PDN ) is used in association with DRB ID 2 . hence during PDN establishment procedure , IP CONFIG REQ : StartPDN , vc_IP_PTC_PDN_MappingTable[PDN_3].DefaultDrb is set ; vc_IP_PTC_PDN_MappingTable[PDN_1].DefaultDrb and vc_IP_PTC_PDN_MappingTable[PDN_2].DefaultDrb are omit As part of the above mentioned flow , the IP CONFIG REQ:ChangeDrbInfo is received for PDN_1 ; f_IP_PTC_PDN_GetDrbInfoForDefaultDrb is called with PDN_1 passed as paramter, which returns Omit. This leads to CompulsoryValueOmittedException run time Issue |
| Summary of change | Adding a check whether vc_IP_PTC_PDN_MappingTable[PDN_Index] is having the values , before calling the function f_IP_PTC_PDN_GetDrbInfoForDefaultDrb with the PDN_Index, |
| TTCN module | Common/IP_PTC/ IP_PTC_Main.ttcn |
| MCC160 Comment | Alternative solution implemented: as the change of DRB routing info during RAT change from NR to EUTRA is handled elsewhere, and there are no IRAT test cases moving between EUTRA cells, the DRB mapping does not need to be updated here. Therefore replaced the call to f_EUTRA_UE_Detach_SwitchOffUe_Trigger with simply switching/powering off the UE |

Before Change:

altstep a_ApplicationCtrl(IP_PTC_CTRL_PORT p_CtrlPort) runs on IP_PTC { var IP_ConfigReq_Type v_IP_ConfigReq; var PDN_Index_Type v_PdnIndex; var IP_DrbInfo_Type v_DrbInfo; var IP_DrbInfo_Type v_CurrentDrbInfo; var EUTRA_CellId_Type v_EutraCellId; var integer i; [] p_CtrlPort.receive(cr_IP_CtrlAnyReq) -> value v_IP_ConfigReq { if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3) { // @sic R5s191074 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { if (ischosen(v_IP_ConfigReq.StopPDN.PdnId)) { /* @sic R5s131000: stop particular PDN sic@ */ v_PdnIndex := v_IP_ConfigReq.StopPDN.PdnId; /* @sic R5s201177 sic@ */ fl_StopPDNs(v_PdnIndex); /* stop particular PDN */ } else { fl_StopPDNs(); /* => stop all PDNs and IMS servers @sic R5s201177 sic@ */ } } else if (ischosen(v_IP_ConfigReq.StartWLAN)) { fl_WLAN_Start(v_IP_ConfigReq.StartWLAN.AddressInfo, v_IP_ConfigReq.StartWLAN.DrbInfo, v_IP_ConfigReq.StartWLAN.ExplicitDnsHandling); } else if (ischosen(v_IP_ConfigReq.StopWLAN)) { fl_WLAN_Stop(); } else if (ischosen(v_IP_ConfigReq.ChangeEutraCell)) { /* @sic R5s150648 Change 6 sic@ */ v_EutraCellId := v_IP_ConfigReq.ChangeEutraCell; if (f_RoutingTable_ChangeEutraCell(v_EutraCellId)) { for (i:=0; i<tsc_NoOfPDNs; i:=i+1) { if (isvalue(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb)) { /* @sic R5s201501 sic@ */ f_IP_DrbInfo_ChangeEutraCell(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb, v_EutraCellId); } } } } else if (ischosen(v_IP_ConfigReq.ChangeDrbInfo)) { /* @sic R5s150648 Change 6 sic@ */ v_PdnIndex := v_IP_ConfigReq.ChangeDrbInfo.PdnId; v_DrbInfo := v_IP_ConfigReq.ChangeDrbInfo.DrbInfo; v_CurrentDrbInfo := f_IP_PTC_PDN_GetDrbInfoForDefaultDrb(v_PdnIndex); if (f_RoutingTable_ChangeDrbInfo(v_CurrentDrbInfo, v_DrbInfo)) { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } } else if (ischosen(v_IP_ConfigReq.SendRouterAdvertisement)) { fl_ICMPv6_SendRouterAdvertisement(v_IP_ConfigReq.SendRouterAdvertisement.PdnId); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting)) { if (ischosen(v_IP_ConfigReq.ExplicitRouting.Add)) { f_IP_PTC_RoutingTable_AddEntry(v_IP_ConfigReq.ExplicitRouting.Add.IpInfo, v_IP_ConfigReq.ExplicitRouting.Add.DRB, v_IP_ConfigReq.ExplicitRouting.Add.DataMode); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting.Remove)) { /* @sic R5s160131 sic@ */ f_IP_PTC_RoutingTable_RemoveEntry(v_IP_ConfigReq.ExplicitRouting.Remove); } } else if (ischosen(v_IP_ConfigReq.ChangeToRawmode)) { /* @sic R5s150074 - Additional change sic@ */ f_RoutingInfo_ChangeMode(true); // @sic R5s200044 change 4, R5s200490 sic@ } else if (ischosen(v_IP_ConfigReq.RestoreRoutingTable)) { // @sic R5s200044 change 4 sic@ f_RoutingInfo_ChangeMode(false); // @sic R5s200490 sic@ } p_CtrlPort.send(cs_IP_CommonCnf); } }

After Change:

altstep a_ApplicationCtrl(IP_PTC_CTRL_PORT p_CtrlPort) runs on IP_PTC { var IP_ConfigReq_Type v_IP_ConfigReq; var PDN_Index_Type v_PdnIndex; var IP_DrbInfo_Type v_DrbInfo; var IP_DrbInfo_Type v_CurrentDrbInfo; var EUTRA_CellId_Type v_EutraCellId; var integer i; [] p_CtrlPort.receive(cr_IP_CtrlAnyReq) -> value v_IP_ConfigReq { if (ischosen(v_IP_ConfigReq.StartPDN)) { v_PdnIndex := v_IP_ConfigReq.StartPDN.PdnId; v_DrbInfo := v_IP_ConfigReq.StartPDN.DrbInfo; f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); select (v_PdnIndex) { case (PDN_1, PDN_2, PDN_2a) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); fl_IP_ImsServer_Start(v_PdnIndex); } case (PDN_Internet, nrPDN_Internet, PDN_3) { // @sic R5s191074 sic@ fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo); } case (ePDG_IMS1, ePDG_IMS2) { fl_IP_ImsServer_Start(v_PdnIndex); } case (ePDG_XCAP) { fl_IP_AddrServers_Start(v_PdnIndex, v_DrbInfo, dnsServerOnly); } case (ePDG_Internet) { // nothing to be done } } } else if (ischosen(v_IP_ConfigReq.StopPDN)) { if (ischosen(v_IP_ConfigReq.StopPDN.PdnId)) { /* @sic R5s131000: stop particular PDN sic@ */ v_PdnIndex := v_IP_ConfigReq.StopPDN.PdnId; /* @sic R5s201177 sic@ */ fl_StopPDNs(v_PdnIndex); /* stop particular PDN */ } else { fl_StopPDNs(); /* => stop all PDNs and IMS servers @sic R5s201177 sic@ */ } } else if (ischosen(v_IP_ConfigReq.StartWLAN)) { fl_WLAN_Start(v_IP_ConfigReq.StartWLAN.AddressInfo, v_IP_ConfigReq.StartWLAN.DrbInfo, v_IP_ConfigReq.StartWLAN.ExplicitDnsHandling); } else if (ischosen(v_IP_ConfigReq.StopWLAN)) { fl_WLAN_Stop(); } else if (ischosen(v_IP_ConfigReq.ChangeEutraCell)) { /* @sic R5s150648 Change 6 sic@ */ v_EutraCellId := v_IP_ConfigReq.ChangeEutraCell; if (f_RoutingTable_ChangeEutraCell(v_EutraCellId)) { for (i:=0; i<tsc_NoOfPDNs; i:=i+1) { if (isvalue(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb)) { /* @sic R5s201501 sic@ */ f_IP_DrbInfo_ChangeEutraCell(vc_IP_PTC_PDN_MappingTable[i].DefaultDrb, v_EutraCellId); } } } } else if (ischosen(v_IP_ConfigReq.ChangeDrbInfo)) { /* @sic R5s150648 Change 6 sic@ */ v_PdnIndex := v_IP_ConfigReq.ChangeDrbInfo.PdnId; v_DrbInfo := v_IP_ConfigReq.ChangeDrbInfo.DrbInfo; if(ispresent(vc_IP_PTC_PDN_MappingTable[f_PDN_PdnIndex2Integer(v_PdnIndex)].DefaultDrb)) //WA_21wk12_Start { v_CurrentDrbInfo := f_IP_PTC_PDN_GetDrbInfoForDefaultDrb(v_PdnIndex); if (f_RoutingTable_ChangeDrbInfo(v_CurrentDrbInfo, v_DrbInfo)) { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } else { f_IP_PTC_PDN_SetDrbInfoForDefaultDrb(v_PdnIndex, v_DrbInfo); } } //WA_21wk12_End } else if (ischosen(v_IP_ConfigReq.SendRouterAdvertisement)) { fl_ICMPv6_SendRouterAdvertisement(v_IP_ConfigReq.SendRouterAdvertisement.PdnId); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting)) { if (ischosen(v_IP_ConfigReq.ExplicitRouting.Add)) { f_IP_PTC_RoutingTable_AddEntry(v_IP_ConfigReq.ExplicitRouting.Add.IpInfo, v_IP_ConfigReq.ExplicitRouting.Add.DRB, v_IP_ConfigReq.ExplicitRouting.Add.DataMode); } else if (ischosen(v_IP_ConfigReq.ExplicitRouting.Remove)) { /* @sic R5s160131 sic@ */ f_IP_PTC_RoutingTable_RemoveEntry(v_IP_ConfigReq.ExplicitRouting.Remove); } } else if (ischosen(v_IP_ConfigReq.ChangeToRawmode)) { /* @sic R5s150074 - Additional change sic@ */ f_RoutingInfo_ChangeMode(true); // @sic R5s200044 change 4, R5s200490 sic@ } else if (ischosen(v_IP_ConfigReq.RestoreRoutingTable)) { // @sic R5s200044 change 4 sic@ f_RoutingInfo_ChangeMode(false); // @sic R5s200490 sic@ } p_CtrlPort.send(cs_IP_CommonCnf); } }

MCC160 Implementation:

function f_EUTRA38_MultiPDN_UE_Detach_SwitchOffUe(EUTRA_CellId_Type p_CellId,

EUTRA_RRC_STATE_Type p_RRCState,

EUTRA_ATTACH_Type p_ForcedAttach := NORMAL,

template DetachType p_DetachType := omit,

boolean p_USIM_Rmvd := false)

runs on EUTRA_Multi5GS_PTC

{

if (pc_SwitchOnOff) { // The UE can actually be switched off

f_UT_SwitchOffUE(UT, p_USIM_Rmvd);

f_EUTRA38_MultiPDN_UE_DetachMO(p_CellId, p_RRCState, p_ForcedAttach, p_DetachType);

//Now the connection can be released

f_EUTRA_RRC_ConnectionRelease(p_CellId);

} else {

f_UT_PowerOffUE(UT);

//Reset all RBs

f_EUTRA_SS_SRBs_DRBs_Reset(p_CellId);   // @sic R5s110423 sic@

}

}