3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210426

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1666 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.2.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The value of variables “v_SrbIndDummy” and “v_SysIndDummy” need to be set to omit before the call to the function “a_NR_RacingCond_AwaitRrcMessageAndSysInd” otherwise it will return an error. It will have valid value after calling function ” a_NR_RacingCond_AwaitRrcMessageAndSysInd”. - The harq ack Ind and RRCSetupComplete may be received out of order due to race condition. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Set “v_SrbIndDummy” and “v_SysIndDummy” to omit after calling function “a_NR_RacingCond_AwaitRrcMessageAndSysInd”. - Used interleave to receive harq ack Ind and RRCSetupComplete and then compare their receiving time to make sure the physical timing is correct. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_2_3_NR_TestBody |
| --- | --- |
| Reason for change | - The value of variables “v_SrbIndDummy” and “v_SysIndDummy” need to be set to omit before the call to the function “a_NR_RacingCond_AwaitRrcMessageAndSysInd” otherwise it will return an error. It will have valid value after calling function ” a_NR_RacingCond_AwaitRrcMessageAndSysInd”. - The harq ack Ind and RRCSetupComplete may be received out of order due to race condition. |
| Summary of change | - Set “v_SrbIndDummy” and “v_SysIndDummy” to omit after calling function “a_NR_RacingCond_AwaitRrcMessageAndSysInd” - Used interleave to receive harq ack Ind and RRCSetupComplete and then compare their receiving time to make sure the physical timing is correct. |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Change 1 on settin og variables to imit accepted. Change 2 accepted in principle and implemented differently as shown below. |

Before Change

IMPORT：import from NAS_CommonTemplates all;//KS R5s21xxx import from NG_NAS_Templates all;//KS R5s21xxx ……

function f_TC_7_1_1_2_3_NR_TestBody() runs on NR5GC_PTC { //@sic R5s210108 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; var NG_NAS_MSG_Indication_Type v_NasInd; timer t_Watchdog := 5.0; //@sic R5s210108 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); t_Watchdog.start; // @siclog "Step 2" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 4-7" siclog@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 5 7"); } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is sending HARQ ACK/NACK "); } }; // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 10-13" siclog@ //@sic R5s210108 sic@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 11 13"); } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE is sending HARQ ACK/NACK "); } }; // @siclog "Step 14" siclog@ //Preconfigured at SS // @siclog "Step 15-18" siclog@ //@sic R5s210108 sic@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15: UE is sending HARQ ACK/NACK "); } }; // @siclog "Step 19" siclog@ //Preconfigured at SS // @siclog "Step 20-21" siclog@ // @sic R5s200080 R5s201287 sic@ SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack)); // disable HARQ reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); v_NasInd := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // disable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // @siclog "Step 22-25" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); };

After Change

function f_TC_7_1_1_2_3_NR_TestBody() runs on NR5GC_PTC { //@sic R5s210108 sic@ var template (omit) NR_SRB_COMMON_IND v_SrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; var NG_NAS_MSG_Indication_Type v_NasInd; timer t_Watchdog := 5.0; var NR_SYSTEM_IND v_SYSTEM_IND;//KS R5s21xxxx var NR_SRB_COMMON_IND v_ReceivedAsp;//KS R5s21xxxx var SubFrameTiming_Type v_Timing_SysInd;//KS R5s21xxxx var SubFrameTiming_Type v_Timing_SrbInd;//KS R5s21xxxx //@sic R5s210108 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1); t_Watchdog.start; // @siclog "Step 2" siclog@ SYSIND.receive(car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 2"); // @siclog "Step 3" siclog@ //Preconfigured at SS // @siclog "Step 4-7" siclog@ { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 5 7"); } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: UE is sending HARQ ACK/NACK "); } }; v_SrbIndDummy := omit;//KS R5s21xxxx v_SysIndDummy := omit;//KS R5s21xxxx // @siclog "Step 9" siclog@ //Preconfigured at SS // @siclog "Step 10-13" siclog@ //@sic R5s210108 sic@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), car_NR_PRACH_Preamble_IND(nr_Cell1, cr_TimingInfo_Any, ?)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 11 13"); } [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 10: UE is sending HARQ ACK/NACK "); } }; v_SrbIndDummy := omit;//KS R5s21xxxx v_SysIndDummy := omit;//KS R5s21xxxx // @siclog "Step 14" siclog@ //Preconfigured at SS // @siclog "Step 15-18" siclog@ //@sic R5s210108 sic@ alt { [] a_NR_RacingCond_AwaitRrcMessageAndSysInd(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SRB0_RrcPdu_IND(nr_Cell1, cr_38508_RRCSetupRequest), { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 15: UE is sending HARQ ACK/NACK "); } }; // @siclog "Step 19" siclog@ //Preconfigured at SS // @siclog "Step 20-21" siclog@ // @sic R5s200080 R5s201287 sic@ //KS R5s21xxxx start interleave { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack))-> value v_SYSTEM_IND { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20-21 Receive Ack"); // disable HARQ reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); } [] SRB.receive(car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) -> value v_ReceivedAsp { v_NasInd := v_ReceivedAsp.Signalling.Nas[0]; f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 20-21 Receive RRCSetupComplete"); } }; v_Timing_SrbInd := valueof(v_ReceivedAsp.Common.TimingInfo.SubFrame); v_Timing_SysInd := valueof(v_SYSTEM_IND.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(nr_Cell1, SysIndBeforeRrcMsg, v_Timing_SrbInd, v_Timing_SysInd); //KS R5s21xxxx end f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21"); // disable PRACH reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable)); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); // @siclog "Step 22-25" siclog@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_NasInd); };

MCC 160 Implementation

// @sic R5s200080 R5s201287 R5s210426 sic@ v_SrbIndDummy := omit; v_SysIndDummy := omit; a_NR_RacingCond_AwaitSysIndAndRrcMessage(nr_Cell1, SysIndBeforeRrcMsg, v_SrbIndDummy, v_SysIndDummy, car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, ack), car_NR_SRB1_RrcNasPdu_IND(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected), omit)) // disable HARQ reception indication f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(disable)); v_NasInd := valueof (v_SrbIndDummy.Signalling.Nas[0]); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 21");

New altstep a_NR_RacingCond_AwaitSysIndAndRrcMessage

altstep a_NR_RacingCond_AwaitSysIndAndRrcMessage(NR_CellId_Type p_NR_CellId, RacingCond_ExpectedSequence_Type p_ExpectedSequence, inout template (omit) NR_SRB_COMMON_IND p_SrbIndByRef, inout template (omit) NR_SYSTEM_IND p_SysIndByRef, template (present) NR_SYSTEM_IND p_ExpectedSysInd := car_NR_SYSTEM_IND_Any, template (present) NR_SRB_COMMON_IND p_ExpectedSrbInd := car_NR_SRB_IND_Any) runs on NR_BASE_PTC { /* note: since this is an altstep 'interleave' cannot be used */ /* note: since the altstep uses repeat to receive the second message the altstep is left and re-entered again * => local variables are uninitialised; the only way to store information of subsequent messages is by the inout parameters * (which for that reason have to be "inout"; "out" would not work */ var NR_SRB_COMMON_IND v_SRB_COMMON_IND; var NR_SYSTEM_IND v_SYSTEM_IND; var SubFrameTiming_Type v_Timing_SysInd; var SubFrameTiming_Type v_Timing_SrbInd; [not isvalue(p_SrbIndByRef)] SRB.receive (p_ExpectedSrbInd) -> value v_SRB_COMMON_IND { p_SrbIndByRef := v_SRB_COMMON_IND; if (not isvalue(p_SysIndByRef)) { // SYSTEM_IND has not been received yet repeat; // => wait for SYSTEM_IND } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd := valueof(p_SysIndByRef.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SysInd, v_Timing_SrbInd ); } } [not isvalue(p_SysIndByRef)] SYSIND.receive (p_ExpectedSysInd) -> value v_SYSTEM_IND { p_SysIndByRef := v_SYSTEM_IND; if (not isvalue(p_SrbIndByRef)) { // RRC message has not been received yet repeat; // => wait for RRC PDU } else { v_Timing_SrbInd := valueof(p_SrbIndByRef.Common.TimingInfo.SubFrame); v_Timing_SysInd := valueof(p_SysIndByRef.Common.TimingInfo.SubFrame); fl_NR_RacingCond_CheckSequence(p_NR_CellId, p_ExpectedSequence, v_Timing_SysInd, v_Timing_SrbInd ); } } }