3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210427

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1667 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR MAC test case 7.1.1.2.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The parameters to function f_Delay are passed in incorrect order. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Exchanged the order of parameters “v_SysinfoActivation” and “v_TimingDL” in function “f_Delay()” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.2.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_1_2_4_NR_TestBody |
| --- | --- |
| Reason for change | The parameters to function f_Delay are passed in incorrect order. |
| Summary of change | Exchanged the order of parameters “v_SysinfoActivation” and “v_TimingDL” in function “f_Delay()” |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before Change

function f_TC_7_1_1_2_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR5GC_PTC { //@sic R5s210108 sic@ var template (omit) NR_DRB_COMMON_IND v_DrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var SubFrameTiming_Type v_SysinfoActivation; var SubFrameTiming_Type v_TimingDL ; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); timer t_Watchdog := 10.0; if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_Prach_RootSequenceIndex := {l139 := 2}; } else //FR1 FDD { v_Prach_RootSequenceIndex := {l139 := 20}; } // @siclog "Step 6" siclog@ preconfig SS f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s201512 sic@ // @siclog "Step 1-2" siclog@ f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); //Modify Sysinfo in SS //@sic R5s200080 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_SIRNTI_REQ(nr_Cell1,-, Erroneous)); v_SysinfoActivation := f_NR_ModifySysinfo(nr_Cell1, true,RRC_CONNECTED); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 4" siclog@ //@sic R5s201038 sic@ v_TimingDL := f_NR_GetCurrentTiming(nr_Cell1); f_Delay(int2float(f_SubFrameTiming_Duration(v_SysinfoActivation, v_TimingDL)-200)/1000.0); v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_SysinfoActivation, 400); //@sic R5-204421 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_SIRNTI_REQ(nr_Cell1,cs_TimingInfo_NR(v_TimingDL), Normal)); // @siclog "Step 3, 5" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3 or 5: UE is sending HARQ ACK/NACK when sent addressed to SI-RNTI"); } ......

After Change

function f_TC_7_1_1_2_4_NR_TestBody(DRB_Identity p_NR_DRB_Id) runs on NR5GC_PTC { //@sic R5s210108 sic@ var template (omit) NR_DRB_COMMON_IND v_DrbIndDummy := omit; var template (omit) NR_SYSTEM_IND v_SysIndDummy := omit; var NR_PhysicalParameters_Type v_NR_PhysicalParameters ; var SubFrameTiming_Type v_SysinfoActivation; var SubFrameTiming_Type v_TimingDL ; var NR_Prach_RootSequenceIndex_Type v_Prach_RootSequenceIndex; var NR_DuplexMode_Type v_NR_DuplexMode := f_NR_CellInfo_GetDuplexMode(nr_Cell1); timer t_Watchdog := 10.0; if (ischosen(v_NR_DuplexMode.TDD)) //FR1 TDD { v_Prach_RootSequenceIndex := {l139 := 2}; } else //FR1 FDD { v_Prach_RootSequenceIndex := {l139 := 20}; } // @siclog "Step 6" siclog@ preconfig SS f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Stop); //@sic R5s201512 sic@ // @siclog "Step 1-2" siclog@ f_NR_CellInfo_SetSIB1_Prach_RootSequenceIndex(nr_Cell1, v_Prach_RootSequenceIndex); //Modify Sysinfo in SS //@sic R5s200080 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_SIRNTI_REQ(nr_Cell1,-, Erroneous)); v_SysinfoActivation := f_NR_ModifySysinfo(nr_Cell1, true,RRC_CONNECTED); f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_UL_HARQ(enable)); // @siclog "Step 4" siclog@ //@sic R5s201038 sic@ v_TimingDL := f_NR_GetCurrentTiming(nr_Cell1); f_Delay(int2float(f_SubFrameTiming_Duration(v_TimingDL, v_SysinfoActivation)-200)/1000.0);//KS R5s21xxxx v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_SysinfoActivation, 400); //@sic R5-204421 sic@ f_NR_SS_L1_TestModeConfig(nr_Cell1, cas_NR_L1_TestModeCRC_SIRNTI_REQ(nr_Cell1,cs_TimingInfo_NR(v_TimingDL), Normal)); // @siclog "Step 3, 5" siclog@ t_Watchdog.start; alt { [] SYSIND.receive(car_NR_SYSTEM_UL_HARQ_IND(nr_Cell1, (ack, nack))) { t_Watchdog.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 3 or 5: UE is sending HARQ ACK/NACK when sent addressed to SI-RNTI"); } ......