3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210431

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1670 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.1.5.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step 52, rejected NSSAI parameter is being returned NULL which is causing mismatch and resulting TC fail. Metacharacter * will match the longest possible number of characters. To filter exactly what is needed, matching of quotation marks around the rejected NSSAI should be added. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added quotation marks matching around the rejected NSSAI to get a non null return value of rejected NSSSAI. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_UT_ReadRejectedNSSAI |
| --- | --- |
| Reason for change | At step 52, rejected NSSAI parameter is being returned NULL which is causing mismatch and resulting TC fail. Metacharacter * will match the longest possible number of characters. To filter exactly what is needed, matching of quotation marks around the rejected NSSAI should be added. |
| Summary of change | Added quotation marks matching around the rejected NSSAI to get a non null return value of rejected NSSSAI. |
| TTCN module | UpperTesterFunctions.ttcn |
| MCC160 Comment | Alternative solution implemented: new function f_StringSplitLines used to ensure only a single ‘line’ of the AT response is included in the regexp. A similar change is also made in f_UT_ReadAllowedNSSAI |

Before Change:

function f_UT_ReadRejectedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, oolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected, p_PLMN_ID); var charstring v_Params := regexp(v_ResultString, “\+C5GNSSAIRDP: (*)”, 0); var CharStringList_Type v_CharStringList := f_StringSplit(v_Params, {“,”}); // a list of 2, 4 or 6 elements: each = length & value string var charstring v_Rejected := “”; var integer I; if (p_Access3GPP) { I := 3; } else { I := 5; } if (isvalue(v_CharStringList[i])) { // @sic R5s210134 sic@ v_Rejected := regexp(v_CharStringList[i], “””(*)”””, 0); // @sic R5s210362 sic@ } return v_Rejected; }

++++++++++++++++++++

After Change:

| function f_UT_ReadRejectedNSSAI(UT_PTC_MTC_PORT p_Port, template (omit) charstring p_PLMN_ID := omit, boolean p_Access3GPP := true) return charstring { var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected, p_PLMN_ID); var charstring v_Params := regexp(v_ResultString, "\+C5GNSSAIRDP: (*)", 0); var CharStringList_Type v_CharStringList := f_StringSplit(v_Params, {","}); // a list of 2, 4 or 6 elements: each = length & value string var charstring v_Rejected := ""; var integer i; if (p_Access3GPP) { i := 3; } else { i := 5; } if (isvalue(v_CharStringList[i])) { // @sic R5s210134 sic@ v_Rejected := regexp(v_CharStringList[i], """(*)""*", 0); // @sic R5s210362 sic@ //KS@9.1.5.1.3 } return v_Rejected; } |  |  |  |
| --- | --- | --- | --- |

MCC160 Implementation

function f_StringSplitLines(charstring p_Text,

charstring p_Delimitor :=”\r\n”) return CharStringList_Type

{

var charstring v_Text := p_Text;

var CharStringList_Type v_CharStringList1;

var CharStringList_Type v_CharStringList2 := {};

var charstring v_Expression := "(*)(" & p_Delimitor & ")(*)";

var integer i := 0;

var integer N;

while (regexp(v_Text, v_Expression, 1) != "") {

v_CharStringList1[i] := regexp(v_Text, v_Expression, 2);

v_Text := regexp(v_Text, v_Expression, 0);

i := i + 1;

}

v_CharStringList1[i] := v_Text;

N := i + 1;

for (i := 0; i < N; i := i + 1) {   // TITAN ISSUE: Titan does not allow "{ v_String } & v_CharStringList1", therefore we need to change the order of the array

v_CharStringList2[i] := v_CharStringList1[N-i-1];

}

return v_CharStringList2;

}

function f_UT_ReadRejectedNSSAI(UT_PTC_MTC_PORT p_Port,
template (omit) charstring p_PLMN_ID := omit,
oolean p_Access3GPP := true) return charstring
{
var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected, p_PLMN_ID);
var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>");  // a list charstrings separated by CR LF

var charstring v_Params := regexp( v_ListOfLines[0], “\+C5GNSSAIRDP: (*)”, 0);
var CharStringList_Type v_CharStringList := f_StringSplit(v_Params, {“,”}); // a list of 2, 4 or 6 elements: each = length & value string
var charstring v_Rejected := “”;
var integer I;

if (p_Access3GPP) {
I := 3;
} else {
I := 5;
}

if (isvalue(v_CharStringList[i])) { // @sic R5s210134 sic@
v_Rejected := regexp(v_CharStringList[i], “””(*)”””, 0); // @sic R5s210362 sic@
}
return v_Rejected;
}

function f_UT_ReadAllowedNSSAI(UT_PTC_MTC_PORT p_Port,

template (omit) charstring p_PLMN_ID := omit,

boolean p_Access3GPP := true) return charstring

{

var charstring v_ResultString := f_UT_ReadNSSAI_DynamicParams(p_Port, Default_Rejected_Configured_Allowed, p_PLMN_ID);

var CharStringList_Type v_ListOfLines := f_StringSplitLines(v_ResultString, "<CR><LF>");  // a list charstrings separated by CR LF @sic R5s210431 sic@

var charstring v_Allowed; // @sic R5s200573 sic@

v_ListOfLines := f_CharStringList_Remove(v_ListOfLines, ""); // remove empty lines from the list @sic R5s210431 sic@

v_Allowed := regexp(v_ListOfLines[1], "\+C5GNSSAIRDP: ""*"",[0-9]+,""*"",(*)", 0); // @sic R5s200573, R5s210431 sic@

if (p_Access3GPP) {

return regexp(v_Allowed, "[0-9]+,""(*)""", 0); // @sic R5s200573 sic@

} else {

return regexp(v_Allowed, "[0-9]+,""*"",[0-9]+,""(*)""", 0);

}

}