3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210437

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1675 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GSM test case 10.1.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At Step 16, the prose states “and DNN in UL NAS TRANSPORT message are the same values as released”. Current implementation will use tsc_RandomAPN (=”ABCC”) which would not necessarily be the same DNN as the PDU session which was previously released. This PDU session would have been created during preamble and DNN could have been given by the UE or taken from PC_APN_ID_Internet. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Take DNN value for the AT command at step 16 from v_RequestedDNN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.4.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_10_1_4_1_Body() |
| --- | --- |
| Reason for change | At Step 16, the prose states “and DNN in UL NAS TRANSPORT message are the same values as released”. Current implementation will use tsc_RandomAPN (=”ABCC”) which would not necessarily be the same DNN as the PDU session which was previously released. This PDU session would have been created during preamble and DNN could have been given by the UE or taken from PC_APN_ID_Internet. |
| Summary of change | Take DNN value for the AT command at step 16 from v_RequestedDNN. |
| TTCN module | GSM_TestCases_NR5GC.ttcn |
| MCC160 Comment | Postponed until RAN5 decision on R5s210383 |

Before change

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC

{

……………

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1");

//@siclog "Steps 2 - 15 void" siclog@

//@siclog "Steps 16" siclog@

//@sic R5-202664 sic@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2).

t_Wait.start (1.0);

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

t_Wait.stop;

}

[]t_Wait.timeout

{

// @sic R5s201028 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI);

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_RandomAPN, -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@ //KS@10.1.4.1

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

}

}

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

……………

}

After change

function fl_TC_10_1_4_1_Body() runs on NR5GC_PTC

{

……………….

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 1");

//@siclog "Steps 2 - 15 void" siclog@

//@siclog "Steps 16" siclog@

//@sic R5-202664 sic@

//The UE transmits a PDU SESSION ESTABLISHMENT REQUEST message and the S-NSSAI and DNN in UL NAS TRANSPORT message are the same values as released (Note 2).

t_Wait.start (1.0);

alt{

[]SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp

{

t_Wait.stop;

}

[]t_Wait.timeout

{

// @sic R5s201028 sic@

v_NSSAI := f_ConvertS_NSSAI_ForATCommand (v_RequestedNSSAI);

f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, oct2char(substr((v_RequestedDNN.dnnValue),1,lengthof(v_RequestedDNN.dnnValue)-1)), -, -, v_NSSAI); //AT command to trigger a PDU session @sic R5s201230 sic@ //KS@10.1.4.1

SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

}

}

if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo_Type,

v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, -, -, -, -, -, -, -, -, v_RequestedNSSAI, v_RequestedDNN)) // @sic R5s201028 sic@

{

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed");

}

………………..

}