3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210446

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1681 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for 5GMM test case 9.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, Upon updating the security from the mapped context received from EUTRA, the DL NAS count needs to be reset in order to successfully calculate the MAC-I for the Security Mode Command message. 2, for step 2, it is Mobility Registration, instead of initial registration. the 3rd input parameter leads to mismatch if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Initial_Secure)) |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, DL NAS count is reset 2, Change the 3rd input parameter to Mobility Possible alternative: change f_Check_NG_RegistrationReqMsg to not check the initial registration req in case of Initial_Secure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_9_3_1_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | 1, Upon updating the security from the mapped context received from EUTRA, the DL NAS count need to be reset in order to successfully calculate the MAC-I for the Security Mode Command message. 2, for step 2, it is Mobility Registration, instead of initial registration. the 3rd input parameter leads to mismatch if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Initial_Secure)) |
| Summary of change | 1, DL NAS count is reset 2, Change the 3rd input parameter to Mobility Possible alternative: change f_Check_NG_RegistrationReqMsg to not check the initial registration req in case of Initial_Secure |
| TTCN module | Mobility_Registration_NR5GC_NR |
| MCC160 Comment | 1. Accepted in principle but simply set both NAS counts to 0 in f_NR5GC_MappedContextFromS1_InitNAS 2. Accepted |

Before Change

… import from NG_NasEmu_CtrlFunctions all; … function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { ……… ………. // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); // derive 5G parameters from values received from EUTRA f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, false); // @sic R5s210078 sic@ // Do NAS Security Mode procedure v_SecurityParams := f_NR5GC_Security_Get(); v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Initial_NoSecurity, // @sic R5s210078 sic@ v_GMM_MobilityInfo); // Mapped security context with Kasme @sic R5-205142 sic@ v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity); // @sic R5s210078 sic@ v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg)) { // @sic R5s210359 sic@ // Check updated Registration Request and update v_GMM_MobilityInfo v_NASMsg.PiggybackedPduList[0].Msg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_NASMsg.PiggybackedPduList[0].PiggybackedPduList := omit; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Initial_Secure)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); … … }

After Change

… import from NG_NasEmu_CtrlFunctions all; … function fl_TC_9_3_1_2_NR5GC_TestBody() runs on NR5GC_PTC return NR_RadioBearerId_Type { ……… ………. // Send TAU Req for checking in EUTRA f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_OctetData(v_ReceivedMsg.Pdu.Msg.registration_Request.epsMessage.epsMsg, "TAUReq")); // derive 5G parameters from values received from EUTRA f_NR5GC_MappedContextFromS1_InitNAS (v_SecurityRx, false); // @sic R5s210078 sic@ // Do NAS Security Mode procedure v_SecurityParams := f_NR5GC_Security_Get(); f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering, true); /* reset NAS COUNT */ //ADDED v_MsgToSend := f_Get_NG_SecurityModeCmdMsg(v_SecurityParams, Initial_NoSecurity, // @sic R5s210078 sic@ v_GMM_MobilityInfo); // Mapped security context with Kasme @sic R5-205142 sic@ v_MsgToSend.security_Mode_Command.ngNasKSI := cs_NAS_KeySetIdentifier_lv(v_SecurityRx.KSI, tsc_NasKsi_MappedSecurityContext); // send SECURITY MODE COMMAND SRB.send(cas_NR_SRB_NasPdu_REQ(v_NGC_NASCellA, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_NewSecurityContext, v_MsgToSend))); // receive SECURITY MODE COMPLETE SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered_NewSecurityContext))) -> value v_ReceivedAsp; v_Result := f_Check_NG_SecurityModeCompleteMsg(v_ReceivedAsp.Signalling.Nas[0].Pdu, Initial_NoSecurity); // @sic R5s210078 sic@ v_NasCountUL := v_ReceivedAsp.Signalling.Nas[0].SecurityProtection.NasCount; if (not v_Result) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "NAS Security Mode Procedure Failed"); } if (ispresent(v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg)) { // @sic R5s210359 sic@ // Check updated Registration Request and update v_GMM_MobilityInfo v_NASMsg.PiggybackedPduList[0].Msg := v_ReceivedAsp.Signalling.Nas[0].Pdu.PiggybackedPduList[0].Msg; v_NASMsg.PiggybackedPduList[0].PiggybackedPduList := omit; if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_NASMsg, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Replayed Registration Request Message Failed"); } } f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); … … }

MCC160 Implementation

function f_NR5GC_MappedContextFromS1_InitNAS(EUTRA_NR_CoOrd_Security_Type p_EUTRA_Auth_Params,

boolean p_Handover )  runs on NR5GC_PTC

{

var NG_NAS_SecurityParams_Type v_SecurityParams := f_NR5GC_Security_Get();

var NR_SecurityParams_Type v_RRC_SecurityParams := f_NR_Security_Get();

if (p_Handover) {

v_SecurityParams.KAMF := f_NG_Authentication_A15_HO(p_EUTRA_Auth_Params.NH,   // @sic R5s201078 sic@

p_EUTRA_Auth_Params.Key,

v_SecurityParams.KDF);

} else {

v_SecurityParams.KAMF := f_NG_Authentication_A15_IdleMode(p_EUTRA_Auth_Params.NASCountUL,

p_EUTRA_Auth_Params.Key,

v_SecurityParams.KDF);

}

// Derives KNASenc key

v_SecurityParams.NAS_Ciphering.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Enc_Alg,v_SecurityParams.NAS_Ciphering.Algorithm,

v_SecurityParams.KAMF,

v_SecurityParams.KDF);

// Derives KNASint key

v_SecurityParams.NAS_Integrity.K_NAS := f_NG_Authentication_A8 (tsc_NAS_Int_Alg,v_SecurityParams.NAS_Integrity.Algorithm,

v_SecurityParams.KAMF,

v_SecurityParams.KDF);

// Now NAS parameters updated, store updated common values in RRC global variable @sic R5s201078 sic@

v_RRC_SecurityParams.KAMF := v_SecurityParams.KAMF;

v_RRC_SecurityParams.KSIamf := v_SecurityParams.KSIamf;

// Derive KgNB  @sic R5s201078 sic@

v_RRC_SecurityParams.KGNB := f_NR_Authentication_A9(v_RRC_SecurityParams.KDF, v_RRC_SecurityParams.KAMF, 'FFFFFFFF'O);

// set NH and NCC as per 33.501 cl.8.4.2 point 3  @sic R5s201078 sic@

v_RRC_SecurityParams.NH:= v_RRC_SecurityParams.KGNB;

v_RRC_SecurityParams.NCC := 0;

// Now update the SS

f_SS_NG_NAS_ConfigureSecurity(NASCTRL, v_SecurityParams.NAS_Integrity, v_SecurityParams.NAS_Ciphering);

f_SS_NG_NAS_CountSet(NASCTRL, '00000000'O,'00000000'O);

// @sic R5s201467 sic@

f_NR_Security_Set(v_RRC_SecurityParams);

f_NR5GC_Security_Set (v_SecurityParams);

}