3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210469

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0693 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS call control test case 7.34 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK, Anritsu, Rohde & Schwarz |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS call control test case 7.34 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS call control test case 7.34 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.34.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

3	Branches executed	4

4	Execution Log Files	4

4.1	Qualcomm SM8350+SDX60	4

5	References	5

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 7.34 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

Narendra Kalahasti

Narendra.kalahasti@anritsu.com

Hans Rohnert

Hans.Rohnert@rohde-schwarz.com

## Verification Test Summary

Test Case: 	7.34.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset, Anritsu Protocol Conformance Test System ME7834NR and Rohde & Schwarz 5G Protocol Conformance Test platform.

UE used:	Qualcomm SM8350+SDX60

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Testcase name | TC_7_34_NR5GC |
| --- | --- |
| Reason for change | Guard timer for this test is insufficient due to the delay at step 21 |
| Summary of change | Increase guard timer to 1500s |
| TTCN module | IMS_Testsuite_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

testcase TC_7_34_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Session Timer / MO Call / UE is able to refresh the session / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_7_34_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

After Change:

testcase TC_7_34_NR5GC() runs on MTC_IMS system SYSTEM_IMS { // @purpose // Session Timer / MO Call / UE is able to refresh the session / 5GS var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(1500); v_IMS1 := IMS_PTC.create("IMS1") alive; f_MTC_IMS_CreateMapAndConnectPTCs_NR5GC(system, v_IMS1, v_IMS2); v_IMS1.start(f_TC_7_34_NR5GC_IMS1()); t_GuardTimer.start; f_MTC_MainLoop_IMS(t_GuardTimer); }

# Branches executed

The test case was executed on NR bands n3 and n41

# Execution Log Files

## Qualcomm SM8350+SDX60

The Qualcomm SM8350+SDX60  UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_7_34_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_7_34_PIXIT.xml

The Qualcomm SM8350+SDX60  UE passed this test case on Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Anritsu/TC_7_34_LOG.xml

- PICS/PIXIT parameter file:

Anritsu/TC_7_34_PIXIT.txt

The Qualcomm SM8350+SDX60  UE passed this test case on Rohde & Schwarz 5G Protocol Conformance Test platform.. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log and PICS/PIXIT parameter file:

R&S/7_34_RS_QC_ttcn_tli

# References

| [1] | R5s210470 : Supporting information for addition of IMS 5GS call control tets case 7.34 |
| --- | --- |