3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210478

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1699 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC EPS fall back test case 11.1.4 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC IRAT test case 11.1.4 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC IRAT test case 11.1.4 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	8

3	Branches executed	11

4	Execution Log Files	12

4.1	MediaTek MT6297	12

5	References	12

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.4 which is part of the 5G NRGC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

## Verification Test Summary

Test Case: 	11.1.4.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Anritsu Protocol Conformance Test System ME7834NR

UE used:	MediaTek MT6297

Verification Status:	PASS

# Corrections required

In addition to the below changes, changes 2 and 7 from R5s210414 are required and applied.

## Change 1

| Function name | f_TC_11_1_4_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - As stated in R5s210414 Change 6, RTP loopback needs to be enabled for the IMS voice call established during this test case. - According to 38.523-1 Table 11.1.4.3.3-7, EUTRA Cell 3 is set to barred for all steps. To simply TTCN implementation, set SIB1 of EUTRA Cell 3 to barred at initial configuration of the EUTRA cells. |
| Summary of change | - Add HANDLE_UM_DATA to the f_EUTRA_Init() function which controls RTP loopback configuration. Note : Same change required for all other EPS fallback test cases - Set EUTRA Cell 3 to barred cell during initial configuration of the EUTRA cells. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment | 1. Accepted and implemented under R5s210414 2.Accepted as an improvement subject to Prose CR agreement. Prose CR required to indicate that Eutra Cell 3 is barred in preamble as well. Basically update the Table 11.1.4.3.3-7 accordingly |

Before Change

function f_TC_11_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell reselection using cell status barred / Success f_EUTRA38_IRAT_Init(c31); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-206360 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); // @sic R5-206360 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_1_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

After Change

function f_TC_11_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell reselection using cell status barred / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-206360 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); // @sic R5-206360 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_1_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

## Change 2

| Function name | fl_TC_11_1_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | As in Change 1, according to 38.523-1 Table 11.1.4.3.3-7, EUTRA Cell 3 is set to barred for the all steps. To simply TTCN implementation, set SIB1 of EUTRA Cell 3 to barred at initial configuration of the EUTRA cells. |
| Summary of change | Set EUTRA Cell 3 to barred cell during initial configuration of the EUTRA cells. Remove SIB modification during test body. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment | Accepted as an improvement subject to Prose CR agreement. Prose CR required to indicate that Eutra Cell 3 is barred in preamble as well. Basically update the Table 11.1.4.3.3-7 accordingly |

Before Change

function fl_TC_11_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; // @sic R5s210304 sic@ //Eutra Cell1 is set as barred and modified Sysinfo updated, setting the Cell power to suitable neighbour cell FFS : Prose CR ?? f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); // @sic R5-206360 sic@ f_EUTRA_ModifySysinfo(eutra_Cell3); // @sic R5-206360 sic@ // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE); // @sic R5-206360 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @sic R5-206360 sic@ // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1, eutra_Cell3})); // @sic R5s210304 sic@ // @siclog "Step 18" siclog@ //Generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed without 'connected without release' on E-UTRAN cell upon mobility from NR. v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, -, noRrcConnectionRelease, -, -, -, -, -, true); // @sic R5s210149 sic@ // @sic R5s210304 sic@ // @sic R5s210304 sic@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount,-,-, -,true); // @sic R5s210304 sic@ f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Step 19 - 21" siclog@ f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s210304 sic@ // wait for Trigger for Step 22 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 22" siclog@ f_Delay(1.0); // @siclog "Step 23a1" siclog@ //Void @sic R5-211407 sic@ //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //Send a trigger to NR for it to release its cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; // @sic R5s210304 sic@ //Eutra Cell1 is set as barred and modified Sysinfo updated, setting the Cell power to suitable neighbour cell FFS : Prose CR ?? //REMOVED f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); // @sic R5-206360 sic@ //REMOVED f_EUTRA_ModifySysinfo(eutra_Cell3); // @sic R5-206360 sic@ // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE); // @sic R5-206360 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @sic R5-206360 sic@ // Send EUTRA frequency to NR and wait for NR info f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell1, eutra_Cell3})); // @sic R5s210304 sic@ // @siclog "Step 18" siclog@ //Generic test procedure in TS 38.508-1 [4] Table 4.9.7.2.2-1 is performed without 'connected without release' on E-UTRAN cell upon mobility from NR. v_NasCount := f_EUTRA38_TrackingAreaUpdateFromN1(eutra_Cell1, tsc_RRC_TI_Def, -, -, noRrcConnectionRelease, -, -, -, -, -, true); // @sic R5s210149 sic@ // @sic R5s210304 sic@ // @sic R5s210304 sic@ f_EUTRA38_MultiPDN_Activate_SRB2_DRB_SendRrcMsg(eutra_Cell1, tsc_RRC_TI_Def, v_NasCount,-,-, -,true); // @sic R5s210304 sic@ f_EUTRA_Activate_SRB2_DRB_ReceiveRrcAndNasMsg(eutra_Cell1, tsc_RRC_TI_Def); // Tell IMS PTC the RAT has changed f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1], cms_IPCAN_IMS_QueryResponse(f_EUTRA_IMS_IpcanInfo())); // @siclog "Step 19 - 21" siclog@ f_EUTRA_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // @sic R5s210304 sic@ // wait for Trigger for Step 22 f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); // @siclog "Step 22" siclog@ f_Delay(1.0); // @siclog "Step 23a1" siclog@ //Void @sic R5-211407 sic@ //Now send the Coord to IMS to release the call f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS1]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //Send a trigger to NR for it to release its cell f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 3

| Function name | fl_TC_11_1_4_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The UE shall perform cell selection, rather than cell reselection after receiving RRCRelease message with RedirectedCarrierInfo according to core Spec. [TS 38.331 clause 5.3.8.3] 1> if the RRCRelease message includes redirectedCarrierInfo indicating redirection to eutra: 2> if cnType is included: 3> after the cell selection, indicate the available CN Type(s) and the received cnType to upper layers; - Cell selection procedure doesn’t need to take Priorities into account. [TS 36.304 clause 5.2.3.1] The UE shall use one of the following two cell selection procedures: a) Initial Cell Selection This procedure requires no prior knowledge of which RF channels are E-UTRA or NB-IoT carriers. The UE shall scan all RF channels in the E-UTRA bands according to its capabilities to find a suitable cell. On each carrier frequency, the UE need only search for the strongest cell. Once a suitable cell is found this cell shall be selected. b) Stored Information Cell Selection This procedure requires stored information of carrier frequencies and optionally also information on cell parameters, from previously received measurement control information elements or from previously detected cells. Once the UE has found a suitable cell the UE shall select it. If no suitable cell is found the Initial Cell Selection procedure shall be started. NOTE 1: Priorities between different frequencies or RATs provided to the UE by system information or dedicated signalling are not used in the cell selection process. NOTE 2: If BL UE, UE in enhanced coverage or NB-IoT UE has been provisioned with EARFCN, the UE may use this information during Initial Cell Selection and Stored Information Cell Selection to find a suitable cell. - According to TS 38.331, RRCRelease message can only include RedirectedCarrierInfo once. [TS 38.331 clause 6.2.2] RRCRelease-IEs ::= SEQUENCE { redirectedCarrierInfo RedirectedCarrierInfo OPTIONAL, -- Need N cellReselectionPriorities CellReselectionPriorities OPTIONAL, -- Need R … } RedirectedCarrierInfo ::= CHOICE { nr CarrierInfoNR, eutra RedirectedCarrierInfo-EUTRA, ... } Prose CR will be raised accordingly. |
| Summary of change | - Change Step 17 RRC Release redirectedCarrierInfo to Cell 3’s EARFCN - Remove CellReselectionPriorities in Step 17 RRC Release |
| TTCN module | EPS_Fallback_NR5GC_NR |
| MCC160 Comment | - Accepted - Accepted subject to Prose CR agreement. |

Before Change

function fl_TC_11_1_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ // return the Sysinfo before the test body really starts //f_EUTRA_NR_SendSysinfoCoOrd(EUTRA, f_NR5GC_IRAT_ListOfFrequencies()); //@sic R5s210304 sic@ //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0010'B, mt_Access); // @sic R5-206360 sic@ //@sic R5s210304 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10a1" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#2 associated with the first PDU session? //Void @sic R5-211407 sic@ //@siclog "Step 11" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 12 - 16" siclog@ //Steps 1-5 of the MTSI MO speech call for 5GS procedure according TS 34.229-1 [35] subclause C.21g take place, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo, cs_CellReselectionPriorities ({cs_FreqPriorityEUTRA_NRDef(v_CoOrd_SysInfo.Eutra[1].Arfcn, 1), //@sic R5s210304 sic@ cs_FreqPriorityEUTRA_NRDef(v_CoOrd_SysInfo.Eutra[0].Arfcn, 0)} )))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); //@sic R5s210304 sic@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change

function fl_TC_11_1_4_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ // return the Sysinfo before the test body really starts //f_EUTRA_NR_SendSysinfoCoOrd(EUTRA, f_NR5GC_IRAT_ListOfFrequencies()); //@sic R5s210304 sic@ //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0010'B, mt_Access); // @sic R5-206360 sic@ //@sic R5s210304 sic@ f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10a1" siclog@ //Check: Does the test result of generic test procedure in TS 38.508-1[4] subclause 4.9.1 indicate that the UE is capable of exchanging IP data on DRB#2 associated with the first PDU session? //Void @sic R5-211407 sic@ //@siclog "Step 11" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 12 - 16" siclog@ //Steps 1-5 of the MTSI MO speech call for 5GS procedure according TS 34.229-1 [35] subclause C.21g take place, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 17" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[1].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); //@sic R5s210304 sic@ // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

# Branches executed

The test case was executed on EUTRAN<>NR band combination 1<>n78.

# Execution Log Files

## MediaTek MT6297

The MediaTek MT6297 UE passed this test case on the Anritsu Protocol Conformance Test System ME7834NR. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log:

Anritsu/ TC_11_1_4_LOGS.xml

- PICS/PIXIT parameter file:

Anritsu/TC_11_1_4_PIXIT.txt

# References

| [1] | R5s210479 |
| --- | --- |