3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210483

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1702 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function f_TC_7_1_1_3_3_NR5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SDAP related configuration is not provided to the SS when setting up the DRB. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated in function f_NR_GetRadioBearerConfig_FullAM the parameter to pass “cs_SDAP_Configuration_None”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Configuration provided to setup DRB will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.3.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_7_1_1_3_3_NR5GC	2

# Corrections required

## Correction to function f_TC_7_1_1_3_3_NR5GC

| Function name | f_TC_7_1_1_3_3_NR5GC |
| --- | --- |
| Reason for change | SDAP related configuration is not provided to the SS when setting up the DRB |
| Summary of change | In function f_NR_GetRadioBearerConfig_FullAM updated the parameter to pass “cs_SDAP_Configuration_None” |
| TTCN module | MAC_NR5GC |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_1_3_3_NR5GC() runs on NR5GC_PTC { // Correct handling of MAC control information / Scheduling requests var DRB_Identity v_DRBId1 ; var DRB_Identity v_DRBId2 ; var UInt_Type v_LCID1; //@sic R5-201519 sic@ var UInt_Type v_LCID2; var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList; var template (value) NR_RadioBearerList_Type v_SS_Drb_ConfigList; var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig; var IntegerList_Type v_ListOfDRBs; var template (value) DRB_ToAddModList v_DRBToAddModList; var template (value) SDAP_Config v_SDAP_Config_DRB; var T_PollRetransmit v_T_PollRetransmit := ms80; //@sic R5-202539 sic@ //Init Cell parameters f_NR5GC_Init(NR_1); //Create and configure NR cell1 f_NR_CellConfig_Def(nr_Cell1); // Get NR capabilities sent during EUTRA preamble f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON); //@sic R5s210315 sic@ v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1); v_MAC_CellGroupConfig.bsr_Config := cs_NR_BSR_Config_71133; v_MAC_CellGroupConfig.phr_Config := { release := NULL }; v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64; f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig); // Now that at least one PDU session has been established, get all DRB ids required v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2); //@sic R5-201519 sic@ v_DRBId1 := v_ListOfDRBs[0];v_LCID1 :=v_DRBId1+3; v_DRBId2 := v_ListOfDRBs[1];v_LCID2 :=v_DRBId2+3; //@sic R5s190933 sic@ //@sic R5-201519 sic@ v_SS_Drb_ConfigList := { f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId1,-,-,-,-,v_LCID1), f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId2,-,-,-,-,v_LCID2) //@sic R5s200075 sic@ };

After Change:

function f_TC_7_1_1_3_3_NR5GC() runs on NR5GC_PTC

{ // Correct handling of MAC control information / Scheduling requests

var DRB_Identity v_DRBId1 ;

var DRB_Identity v_DRBId2 ;

var UInt_Type v_LCID1; //@sic R5-201519 sic@

var UInt_Type v_LCID2;

var template (value) NR_RLC_BearerToAddModList_Type v_RLC_BearerConfigList;

var template (value)  NR_RadioBearerList_Type v_SS_Drb_ConfigList;

var template (value) MAC_CellGroupConfig v_MAC_CellGroupConfig;

var IntegerList_Type v_ListOfDRBs;

var template (value) DRB_ToAddModList v_DRBToAddModList;

var template (value) SDAP_Config v_SDAP_Config_DRB;

var T_PollRetransmit v_T_PollRetransmit := ms80; //@sic R5-202539 sic@

//Init Cell parameters

f_NR5GC_Init(NR_1);

//Create and configure NR cell1

f_NR_CellConfig_Def(nr_Cell1);

// Get NR capabilities sent during EUTRA preamble

f_NR5GC_Preamble (nr_Cell1, STATE_IDLE_1A, TESTModeA_ON);

//@sic R5s210315 sic@

v_MAC_CellGroupConfig := f_NR_CellInfo_GetMAC_CellGroupConfig(nr_Cell1);

v_MAC_CellGroupConfig.bsr_Config := cs_NR_BSR_Config_71133;

v_MAC_CellGroupConfig.phr_Config := { release := NULL };

v_MAC_CellGroupConfig.schedulingRequestConfig.schedulingRequestToAddModList[0].sr_TransMax := n64;

f_NR_CellInfo_SetMAC_CellGroupConfig(nr_Cell1, v_MAC_CellGroupConfig);

// Now that at least one PDU session has been established, get all DRB ids required

v_ListOfDRBs := f_NR_GetListOfDRBs_ForFirstPDUSession(2);

//@sic R5-201519 sic@

v_DRBId1 := v_ListOfDRBs[0];v_LCID1 :=v_DRBId1+3;

v_DRBId2 := v_ListOfDRBs[1];v_LCID2 :=v_DRBId2+3;

//@sic R5s190933 sic@

//@sic R5-201519 sic@

v_SS_Drb_ConfigList  := {

f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId1,-,-,-,cs_SDAP_Configuration_None,v_LCID1),      f_NR_GetRadioBearerConfig_FullAM(nr_Cell1, v_DRBId2,-,-,-,cs_SDAP_Configuration_None,v_LCID2)  //@sic R5s200075 sic@    };