3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210501

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1707 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC Testcase 6.4.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Incorrect SMTC window causes UE to not be able to measure NR CELL6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify SMTC window in Sib4 for NR CELL3 (Serving cell during measurement for NR CELL6 is done) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.4.2.2 which is part of the 5G NRGC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_6_4_2_2_NR5GC |
| --- | --- |
| Reason for change | Incorrect SMTC window causes UE to not be able to measure NR CELL6. |
| Summary of change | Modify SMTC window in Sib4 for NR CELL3 (Serving cell during measurement for NR CELL6 is done) |
| TTCN module | Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted but implemented differently, see MCC160 implementation below. Same implementation done for tests 6.1.2.11 and 6.1.2.20 to avoid use of hard coded templates |

Before Change

function f_TC_6_4_2_2_NR5GC() runs on NR5GC_PTC

{ // Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state

// @sic R5-201500 R5s200403 R5s201118 sic@

var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList;

f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells

// Table 6.4.2.2.3.3-2: SIB2 of NR Cell 6

f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10);

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 1

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[1], v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 3

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList[1]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, 10);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 6

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell6, {v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4);

//Set maximum cell power level for the Cells to be used

f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78);

f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78);

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell3);

f_NR_CellConfig_Def(nr_Cell6);

// Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2

f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false);

f_NR_TestBody_Set(true);

f_TC_6_4_2_2_NR5GC_TestBody();

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell3, STATE_INACTIVE_2A);

} //end of f_TC_6_4_2_2_NR5GC

After Change

function f_TC_6_4_2_2_NR5GC() runs on NR5GC_PTC

{ // Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state

// @sic R5-201500 R5s200403 R5s201118 sic@

var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList;

var template (value) SSB_MTC.periodicityAndOffset v_NR_PeriodicityAndOffsetSf20_10 := { sf20 := 10 };

f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells

// Table 6.4.2.2.3.3-2: SIB2 of NR Cell 6

f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10);

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 1

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[1], v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 3

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList[1]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, 10);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_NR_PeriodicityAndOffsetSf20_10);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 6

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell6, {v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4);

//Set maximum cell power level for the Cells to be used

f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78);

f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78);

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell3);

f_NR_CellConfig_Def(nr_Cell6);

// Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2

f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false);

f_NR_TestBody_Set(true);

f_TC_6_4_2_2_NR5GC_TestBody();

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell3, STATE_INACTIVE_2A);

} //end of f_TC_6_4_2_2_NR5GC

MCC160 implementation

function f_TC_6_4_2_2_NR5GC() runs on NR5GC_PTC

{ // Inter-frequency cell reselection according to cell reselection priority provided by SIBs in RRC_INACTIVE state

// @sic R5-201500 R5s200403 R5s201118 R5s210501 sic@

var template (value) InterFreqCarrierFreqList v_InterFreqCarrierFreqList;

var template(value) SSB_MTC v_SSB_MTC;

f_NR5GC_Init(NR_4); // System information combination NR-4 as defined in TS 38.508-1 Table 4.4.3.1.2-1 is used in NR cells

// Table 6.4.2.2.3.3-2: SIB2 of NR Cell 6

f_NR_CellInfo_SetSIB2_ThreshServingLowP(nr_Cell6, 10);

f_NR_CellInfo_SetSIB2_CellReselectionPriority(nr_Cell6, 5);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 1

v_InterFreqCarrierFreqList := f_NR5GC_CellInfo_GetSIB4_InterFreqCarrierFreqList(nr_Cell1);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell1, {v_InterFreqCarrierFreqList[1], v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell1, 1, 4);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 3

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell3, {v_InterFreqCarrierFreqList[1]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell3, 0, 5);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_ThreshX_HighP(nr_Cell3, 0, 10);

v_SSB_MTC := f_NR_GetSSB_MTC_InterFreqOdd(nr_Cell3, nr_Cell6);

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_PeriodicityAndOffset(nr_Cell3, 0, v_SSB_MTC.periodicityAndOffset);

// Table 6.4.2.2.3.3-3: SIB4 for NR Cell 6

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreqList(nr_Cell6, {v_InterFreqCarrierFreqList[0]});

f_NR5GC_CellInfo_SetSIB4_InterFreqCarrierFreq_CellReselectionPriority(nr_Cell6, 0, 4);

//Set maximum cell power level for the Cells to be used

f_NR_CellInfo_InitMaxReferencePower(nr_Cell3, -78, -78);

f_NR_CellInfo_InitMaxReferencePower(nr_Cell6, -78, -78);

// Create and configure cell

f_NR_CellConfig_Def(nr_Cell1);

f_NR_CellConfig_Def(nr_Cell3);

f_NR_CellConfig_Def(nr_Cell6);

// Preamble: The UE is in state 2N-A on NR Cell 1(serving cell) according to TS 38.508-1 Table 4.4A.2-2

f_NR5GC_Preamble(nr_Cell1, STATE_INACTIVE_2A, -, false);

f_NR_TestBody_Set(true);

f_TC_6_4_2_2_NR5GC_TestBody();

f_NR_TestBody_Set(false);

// Switch/Power off UE

f_NR_Postamble(nr_Cell3, STATE_INACTIVE_2A);

} //end of f_TC_6_4_2_2_NR5GC