3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210518

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1714 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NR5GC FR1 : Addition of NR5GC multilayer test case 11.4.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add NR5GC multilayer test case 11.4,3 to the 5G/NR5GC ATS for FR1 path. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC multilayer test case 11.4.3 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.4.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	5

2.3	Change 3	7

2.4	Change 4	10

2.5	Change 5	11

2.6	Change 6	19

2.7	Change 7	21

3	Branches executed	22

4	Execution Log Files	23

4.1	HiSilicon Balong 5000	23

5	References	23

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.4.3 which is part of the 5G NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	11.4.3.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk12

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_NR_ConvertIntAlg2Bitstring |
| --- | --- |
| Reason for change | Current implementation does not handle NIA0 algorithm |
| Summary of change | Add case for NIA0 |
| TTCN module | NR_SecurityFunctions |
| MCC160 Comment | Accepted in principle, but created new local function to only convert the null algorithm if allowed by the attribute f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed. Also called in f_NR_InitAS_KeyChaining_KeyRefresh This attribute is set for TCs: 11.4.2, 11.4.3 and 11.4.9 |

Before Change

function f_NR_ConvertIntAlg2Bitstring(IntegrityProtAlgorithm p_Alg) return B4_Type { var B4_Type v_B4 := '1111'B; select (p_Alg) { case (nia1) { v_B4 := '0001'B; } case (nia2) { v_B4 := '0010'B; } case (nia3) { v_B4 := '0011'B; } case else { FatalError (__FILE__, __LINE__, "Non defined AS Integrity algorithm Selected"); } } return v_B4; }

After Change

function f_NR_ConvertIntAlg2Bitstring(IntegrityProtAlgorithm p_Alg) return B4_Type { var B4_Type v_B4 := '1111'B; select (p_Alg) { case (nia0) { v_B4 := '0000'B; } case (nia1) { v_B4 := '0001'B; } case (nia2) { v_B4 := '0010'B; } case (nia3) { v_B4 := '0011'B; } case else { FatalError (__FILE__, __LINE__, "Non defined AS Integrity algorithm Selected"); } } return v_B4; }

MCC160 Implementation:

function f_NR_Authentication_InitAS(NR_SecurityParams_Type p_Auth_Params,

O4_Type p_UL_NAS_Count,

boolean p_IsSMC_Procedure := true,

template (omit) PhysCellId p_TargetPhysicalCellIdentity := omit,

template (omit) ARFCN_ValueNR  p_ARFCN_DL := omit)

return NR_SecurityParams_Type

{

var NR_SecurityParams_Type v_Auth_Params := p_Auth_Params;

// Derive KgNB

v_Auth_Params.KGNB := f_NR_Authentication_A9(v_Auth_Params.KDF, v_Auth_Params.KAMF, p_UL_NAS_Count);

//Derive NH corresponding to NCC =1;

v_Auth_Params.NH := f_NR_Authentication_A10(v_Auth_Params.KDF, v_Auth_Params.KAMF, v_Auth_Params.KGNB);

if (p_IsSMC_Procedure == false) {

if (not (isvalue(p_TargetPhysicalCellIdentity) and isvalue(p_ARFCN_DL))) {

FatalError(__FILE__, __LINE__, "New key is to be used at RRC Connection Re-establishment or Reconfiguration but no Physical Cell ID and/or ARFCN value is provided");

}

else { // Refresh of KeNB; horizontal key derivation

v_Auth_Params.KGNB := f_NR_Authentication_A11(v_Auth_Params.KDF,

v_Auth_Params.KGNB,

v_Auth_Params.NH,

valueof(p_TargetPhysicalCellIdentity),

valueof(p_ARFCN_DL));

}

}

// NCC is set to default/initial value of 0

v_Auth_Params.NCC := 0;

// Derive KRRC enc key

v_Auth_Params.AS_Ciphering.KRRCenc := f_NG_Authentication_A8(tsc_RRC_Enc_Alg,

f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm),

v_Auth_Params.KGNB,

v_Auth_Params.KDF);

// Derive KRRC int key

v_Auth_Params.AS_Integrity.KRRCint := f_NG_Authentication_A8(tsc_RRC_Int_Alg,

fl_NR_AS_IntegrityInfo_GetIntAlg(v_Auth_Params.AS_Integrity),

v_Auth_Params.KGNB,

v_Auth_Params.KDF);

// Derive UP enc key;

v_Auth_Params.AS_Ciphering.KUPenc := f_NG_Authentication_A8(tsc_UP_Enc_Alg,

f_NR_ConvertCiphAlg2Bitstring(v_Auth_Params.AS_Ciphering.Algorithm),

v_Auth_Params.KGNB,

v_Auth_Params.KDF);

// Derive UP int key;

v_Auth_Params.AS_Integrity.KUPint := f_NG_Authentication_A8(tsc_N_UP_Int_Alg,

fl_NR_AS_IntegrityInfo_GetIntAlg(v_Auth_Params.AS_Integrity),

v_Auth_Params.KGNB,

v_Auth_Params.KDF);

return v_Auth_Params;

}

function fl_NR_AS_IntegrityInfo_GetIntAlg(NR_AS_IntegrityInfo_Type p_AS_Integrity) return B4_Type

{

var B4_Type v_IntAlg;

if ((f_GetTestcaseAttrib_NullIntegrityAlgorithmAllowed(testcasename())) and (p_AS_Integrity.Algorithm == nia0 )) {

v_IntAlg := '0000'B;

} else {

v_IntAlg := f_NR_ConvertIntAlg2Bitstring(p_AS_Integrity.Algorithm);

}

return v_IntAlg;

}

## Change 2

| Function name | f_NR5GC_EmergencyCallPDUSession() |
| --- | --- |
| Reason for change | - For limited service scenarios e.g. 11.4.3, SRB2 has not been established before the PDU session establish procedure - SSC Mode IE is format V in PDU SESSION ESTABLISHMENT ACCEPT so IEI should be omitted |
| Summary of change | - Add one additional parameter to allow the signalling on SRB1 - Set IEI to omit for SSC mode in PDU SESSION ESTABLISHMENT ACCEPT |
| TTCN module | NR5GC_IMSProcedures |
| MCC160 Comment | Accepted in principle but also specified SSC mode 1 for p_ExpectedSSC_Mode (with IEI) |

Before Change

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, -, -, omit, omit, crs_SSC_Mode('A'H, '001'B) -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2"); // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); }

After Change

function f_NR5GC_EmergencyCallPDUSession(NR_CellId_Type p_NR_CellId, NR_RadioBearerId_Type p_SRB := tsc_NR_RbId_SRB2) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo; f_NR5GC_PDUSessionEstablishment(p_NR_CellId, 1, p_SRB, -, omit, omit, crs_SSC_Mode(-, '001'B), -, f_BuildDefaultQoSRules("2"), -, -, -, -, -, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('011'B), -, -, -, -, -, -, -, -, "2"); // Get the Info for the PDU Session just created v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // Wait until need to add voice QoS f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_ModifyPDUSession_AddVoice(p_NR_CellId, valueof(v_GSM_MobilityInfo)); f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); }

## Change 3

| Function name | f_NR5GC_EmergencyCallLimitedService() |
| --- | --- |
| Reason for change | - For limited service scenarios e.g. 11.4.3, no security context has been established at step 2 - For limited service scenarios e.g. 11.4.3, SRB2 has not been established before the PDU session establish procedure - The wait for coordination trigger at the end of the function is not needed. The next trigger sent from IMS PTC will be sent following IMS call release and will be handled in f_NR5GC_EmergencyCallRelease() |
| Summary of change | - Use Initial_NoSecurity while calling f_NR_NASSecurityModeProcedure - Pass SRB1 to f_NR5GC_EmergencyCallPDUSession) - Remove wait for trigger |
| TTCN module | NR5GC_IMSProcedures |
| MCC160 Comment | 1, 2. Accepted 3. Rejected. There is no co-ordination message sent at this point |

Before Change

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { ………………….. //Steps 4-5 // Set to Initial_Secure as the repeated Registration Request should not be requested or expected v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_Secure, v_GMM_MobilityInfo); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }

After Change

function f_NR5GC_EmergencyCallLimitedService(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { …………………… // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); // Set all algorithms to 0 v_NAS_SecurityParams := f_NR5GC_Security_Get(); v_RRC_SecurityParams := f_NR_Security_Get(); v_NAS_SecurityParams.NAS_Integrity.Algorithm := '0000'B; // IA0 v_NAS_SecurityParams.NAS_Ciphering.Algorithm := '0000'B; // EA0 v_NAS_SecurityParams.KSIamf := '000'B; f_NR5GC_Security_Set(v_NAS_SecurityParams); v_RRC_SecurityParams.AS_Integrity.Algorithm := nia0; v_RRC_SecurityParams.AS_Ciphering.Algorithm := nea0; f_NR_Security_Set(v_RRC_SecurityParams); //Steps 4-5 // Set to Initial_Secure as the repeated Registration Request should not be requested or expected v_NAS_SecurityReturnValue := f_NR_NASSecurityModeProcedure(p_NR_CellId, v_NAS_SecurityParams, Initial_NoSecurity, v_GMM_MobilityInfo); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 5"); //Steps 6-7 f_NR_RRC_ActivateSecurity (p_NR_CellId, valueof(v_NAS_SecurityReturnValue.NasCount)); // Use NasCount returned from NAS S.M. Complete f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Step 7"); //Steps 8-9 f_NR_Capability(p_NR_CellId); //Step 10 SRB.send(cas_NR_SRB_NasPdu_REQ(p_NR_CellId, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, // @sic R5-206426 sic@ v_GUTIToSend, v_TAIList)))); // Step 11 f_NR5GC_Registration_Complete(p_NR_CellId); //REMOVED f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); // Steps 13-18 f_NR5GC_EmergencyCallPDUSession(p_NR_CellId, tsc_NR_RbId_SRB1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); // Emergency call now active f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); }

## Change 4

| Function name | f_NR5GC_EmergencyCallRelease() |
| --- | --- |
| Reason for change | The trigger sent to IMS PTC is not needed, IMS PTC isready to receive BYE after receiving the trigger which was sent at the end of f_NR5GC_EmergencyCallPDUSession(). |
| Summary of change | Remove redundant coordination message |
| TTCN module | NR5GC_IMSProcedures |
| MCC160 Comment | Alternative solution implemented: removed send co-ordination message at end of f_NR5GC_EmergencyCallPDUSession |

Before Change

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

After Change

function f_NR5GC_EmergencyCallRelease(NR_CellId_Type p_NR_CellId) runs on NR5GC_PTC { var template (omit) GSM_MobilityInfo_Type v_GSM_MobilityInfo := f_Get_PDUSessionForDNNType(f_NR5GC_MobileInfo_GetSessionInfoList(), Emergency_PDN); // @sic R5s210363 sic@ var GSM_MobilityInfo_Type v_PDUSession := valueof(v_GSM_MobilityInfo); // Must be there as previously established it var O1_Type v_PDUSessionId := v_PDUSession.SessionId; //REMOVE f_IMS_IPCAN_SendCoOrdMsg(IMS[tsc_Index_IMS2]); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR5GC_PDUSessionRelease (p_NR_CellId, v_PDUSessionId, cs_GMM_GSM_Cause (omit, '00100100'B)); f_IP_RtpRtcpLoopback(IP, v_PDUSession.PdnIndex, tsc_IMS_MediaPort_M1, omit); // @sic R5s210363 sic@ f_NR_PreliminaryPass(__FILE__, __LINE__, "Emergency Call Release"); }

## Change 5

| Function name | f_NR5GC_PDUSessionEstablishment_InnerLoop |
| --- | --- |
| Reason for change | - v_GSM_MobilityInfo.PDUType needs to be set earlier so it can be used to determine the v_PDN_Index which is then used to confgiure the routing table. - QFI for emergency is configured incorrectly to QFI 1. According to 38.523-1 Table 4.9.12.2.3-10 QFI 2 should be used |
| Summary of change | - move setting of v_GSM_MobilityInfo.PDUType to before setting of v_PDN_Index - set QFI 2 for emergency PDU session |
| TTCN module | NR5GC_NASSteps |
| MCC160 Comment | Accepted |

Before Change

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { …………… if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { if (v_GSM_MobilityInfo.PDUType == IMS_DNN) { // @sic R5-206296 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); } else { v_SDAP_Config := cs_SDAP_Config; } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function if (isvalue(p_DNN)) { // @sic R5s210148 sic@ v_DNN := p_DNN; } else { v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); // @sic R5-205939 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { v_DNN := f_NG_GetDNN(v_PLMN, v_PDUType, v_GSM_MobilityInfo.DNN); } } //MOVE if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@ // According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU v_GSM_MobilityInfo.PDUType := Emergency_PDN; // @sic R5-206296 sic@ } // Send Accept v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionEstablishmentAccept (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, omit, p_BackOff, p_BuildQosFlow); // Add this PDU Session to stored list f_NR5GC_MobileInfo_AddPDUSession (v_GSM_MobilityInfo); // Send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SSConfig, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); // @sic R5s190629 sic@ return v_PDUType; // @sic R5-206296 sic@ }

After Change

function f_NR5GC_PDUSessionEstablishment_InnerLoop(NR_CellId_Type p_NR_CellId, NR_SRB_COMMON_IND p_ReceivedAsp, SRBs_AlreadyConfigured p_SRB := SRB1_2, template (omit) SDAP_Config p_SDAP_Config, template S_NSSAI_Type p_ExpectedS_NSSAI := *, template DNN p_ExpectedDNN := *, template (value) SSC_Mode p_SSC_Mode := crs_SSC_Mode(-, '001'B), template (omit) DNN p_DNN := omit, template (omit) QoS_Rules p_QoS_Rules := omit, template (value) Session_AMBR p_Session_AMBR := cs_Session_AMBR ('06'O, '05'O, '0004'O, '05'O, '0004'O), template (omit) S_NSSAI_Type p_S_NSSAI := cs_S_NSSAI_SST1eMBB_WithIEI, // @sic R5s190919 sic@ template (omit) QoSFlowDescr p_QoSFlowDescr := omit, template (present) O1_Type p_PDU_SessionId := ?, template ProcedureTransactionIdentifier p_PTI := omit, template (omit) PDU_SessionType p_PDU_SessionType := omit, template SSC_Mode p_ExpectedSSC_Mode := *, // @sic R5s190290 sic@ template NG_UE_SM_Cap p_UECap := *, template MaxNumPacketFilters p_MaxNumPacketFilters := *, template IntegrityProtMaxDataRate p_IntegrityProtMaxDataRate := ?, template AlwaysOnPDUSessionReq p_AlwaysOnPDUSessionReq := *, template SM_PDU_DN_RequestContainer p_PDUReq := *, template ExtdProtocolConfigOptions p_ExtdPCO := *, template NG_PDU_SessionId p_OldPDU_SessionId := *, template NG_Request_Type p_RequestType := ?, template AdditionalInformation p_AdditionalInfo := *, template (omit) GMM_GSM_Cause p_Cause := omit, template (omit) GPRS_Timer p_RQTimer := omit, template (omit) EAP_Message p_EAP := omit, template (omit) GPRS_Timer3 p_BackOff := omit, boolean p_PDUSessionAuthenticationRequired := false, template (omit) charstring p_BuildQosFlow := omit, template (omit) EUTRA_NR_PduSessionInfoList_Type p_PduInfoFromLTE := omit) runs on NR5GC_PTC return PDU_PDN_DNN_Type // @sic R5-198184, R5-206296 sic@ { // Split function so list of DRBs can be built elsewhere @sic R5-199103 sic@ ……………. if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@ // According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU v_GSM_MobilityInfo.PDUType := Emergency_PDN; // @sic R5-206296 sic@ } if (isvalue (p_SDAP_Config)) { // @sic R5s201327 sic@ v_SDAP_Config := p_SDAP_Config; //@sic R5-190720 sic@ } else { if ((v_GSM_MobilityInfo.PDUType == IMS_DNN) or (v_GSM_MobilityInfo.PDUType == Emergency_PDN)) { // @sic R5-206296 sic@ v_SDAP_Config := cs_SDAP_Config(-, -, {2}); } else { v_SDAP_Config := cs_SDAP_Config; } } f_BuildRRCReconfigParamsNewDRBs (v_RadioBearerConfig, v_CellGroupConfig, v_SSConfig, v_SDAP_ConfigurationList, p_NR_CellId, {v_GSM_MobilityInfo}, p_SRB, v_SDAP_Config); // @sic R5s201532 sic@ // Only one PDU session input, so only 1st element relevant in output v_SdapConfig := valueof(v_SDAP_ConfigurationList[0].Config.SdapConfig); //It is assumed that by default only 1 QoS Flow is specified v_QosFlowId := cs_IP_QosFlowId(v_SdapConfig.Pdu_SessionId, v_SdapConfig.MappedQoS_Flows[0]); v_PDN_Index := f_NR5GC_GetPdnIndex(v_GSM_MobilityInfo); // @sic R5-204400 sic@ if (isvalue(v_GMM_MobilityInfo.GMMCap)) { // @sic R5s201358 sic@ if (v_GMM_MobilityInfo.GMMCap.s1Cap == '1'B) { v_GSM_MobilityInfo.EPS_Bearer := valueof(f_GetNextEPSBearerId (v_GSM_MobilityInfo, v_GMM_MobilityInfo)); v_GSM_MobilityInfo.PdnIndex := v_PDN_Index; } } // If this is a default DRB, enable IP handling for this DRB if (valueof(v_SDAP_Config.defaultDRB)) { if (isvalue(p_PduInfoFromLTE)) { // @sic R5s210149 sic@ // The UE has just moved from EUTRA. If so, data from EUTRA should be used, rather than created anew for (i := 0; i < lengthof (p_PduInfoFromLTE); i := i + 1) { if (v_PDN_Index == valueof(p_PduInfoFromLTE[i].PdnIndex)) { // Data from EUTRA should be used, rather than created anew v_StartPDN := false; // no need to change DRB mapping as this has already been done in calling function if (isvalue(p_DNN)) { // @sic R5s210148 sic@ v_DNN := p_DNN; } else { v_PDUType := f_GetPDNTypeFromDNN(v_GSM_MobilityInfo.DNN); // @sic R5-205939 sic@ if (v_PDUType == None) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "DNN PICS do not match received DNN"); } else { v_DNN := f_NG_GetDNN(v_PLMN, v_PDUType, v_GSM_MobilityInfo.DNN); } } /* MOVED if (match(p_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.ul_Nas_Transport.requestType.requestValue, '011'B)) { // @sic R5-202550 sic@ // According to 4.9.11 and 4.9.12, this is the only way to flag it's an emergency PDU v_GSM_MobilityInfo.PDUType := Emergency_PDN; // @sic R5-206296 sic@ } */ // Send Accept v_PiggyBackedMsgToSend := f_Get_NG_PDUSessionEstablishmentAccept (v_GSM_MobilityInfo, v_GMM_MobilityInfo, v_DNN, // @sic R5-205939 sic@ p_SSC_Mode, p_Session_AMBR, p_PDU_SessionType, p_Cause, p_RQTimer, p_S_NSSAI, -, p_EAP, p_QoS_Rules, p_QoSFlowDescr, omit, p_BackOff, p_BuildQosFlow); // Add this PDU Session to stored list f_NR5GC_MobileInfo_AddPDUSession (v_GSM_MobilityInfo); // Send RRC Reconfiguration message f_NR_RRCReconfigNewDRBWithSSConfig(p_NR_CellId, v_RadioBearerConfig, v_SSConfig, v_CellGroupConfig, { cs_NG_NAS_RequestWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered, v_PiggyBackedMsgToSend) }); // @sic R5s190629 sic@ return v_PDUType; // @sic R5-206296 sic@ }

## Change 6

| Function name | fl_IMS_EmergencyCallAndReleaseWithoutRegistration |
| --- | --- |
| Reason for change | - Initial coordination message is not required and not handled anywhere on NR PTC - The function f_IMS_EmergencyCallSetup () should use context A_2_1_A6 Emergency Call Without Registration |
| Summary of change | - remove first coordination message - Set context parameter to A_2_1_A6 |
| TTCN module | Emergency_NR5GC_IMS |
| MCC160 Comment | 1.Accepted 2. Accepted in principle, but passed this in as a parameter |

Before Change

function fl_IMS_EmergencyCallAndReleaseWithoutRegistration () runs on IMS_PTC { var INVITE_Request v_InviteRequest; f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup(); // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Release IMS as this will be (re)started in the next loop f_IMS_GlobalRelease(); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

After Change

function fl_IMS_EmergencyCallAndReleaseWithoutRegistration () runs on IMS_PTC { var INVITE_Request v_InviteRequest; //REMOVE f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); // Emergency call setup: 34.229-5 Annex A.6 v_InviteRequest := f_IMS_EmergencyCallSetup(A_2_1_A6); // Inform NR: Emergency Call established f_IMS_IPCAN_SendCoOrdMsg(IPCAN); f_IMS_IPCAN_WaitForTrigger(IPCAN); //Release Emergency Call MO f_IMS_EmergencyCallRelease(v_InviteRequest); // Release IMS as this will be (re)started in the next loop f_IMS_GlobalRelease(); // Inform NR: Emergency Call released. f_IMS_IPCAN_SendCoOrdMsg(IPCAN); }

## Change 7

| Function name | f_TC_11_4_3_NR5GC |
| --- | --- |
| Reason for change | - The UE will not register during the preamble because the USIM Card is already removed before preamble - Since there is no USIM, UE will not de-register during postamvle |
| Summary of change | - Remove USIM after preamble Note : Possible alternative solution is to remove the preamble function altogether. - Use postamble state DEREGISTERED |
| TTCN module | Emergency_NR5GC |
| MCC160 Comment | Decision postponed until outcome of prose CR R5-212433 at RAN5 |

Before Change

function f_TC_11_4_3_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); f_UT_USIM_Remove(UT); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_NR_TestBody_Set(true); fl_TC_11_4_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_CONNECTED_3A); }

After Change

function f_TC_11_4_3_NR5GC() runs on NR5GC_PTC { f_NR5GC_Init(NR_1);// System information combination NR-1 as defined in TS 38.508-1 [4], subclause 4.4.3.1.2. SIB1 indicates ims-EmergencySupport. //Create and configure cells f_NR_CellConfig_Def(nr_Cell1); //The UE is in test state 0N-B (Switched Off) as defined in TS 38.508-1 [4], subclause 4.4A.2. f_NR5GC_Preamble (nr_Cell1, STATE_OFF_0B); f_UT_USIM_Remove(UT); f_NR_TestBody_Set(true); fl_TC_11_4_3_NR5GC_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_DEREGISTERED); }

# Branches executed

The test case was executed on NR band n41

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

TC_11_4_3_LOG.html

- PICS/PIXIT parameter file:

TC_11_4_3_PIXIT.xml

# References

| [1] | R5s210519 : NR5GC FR1 : Supporting information for addition of NR5GC multilayer test case 11.4.3 |
| --- | --- |