3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210523

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1718 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC RRC test case 8.1.3.1.18.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following the power level change at step 7 there is not enough time for the UE to measure the power levels before they are changes again at step 7A, and hence the UE may send the measurement report at step 8 still based on the power levels of T1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a delay of 30 seconds between steps 7 and 7A to ensure that the UE has read the updated power levels and sends correct measurement report at step8. Note: an associated prose CR on 38.523-1 will be raised at RAN5#91-e |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.3.1.18.1.NR5GC, 8.1.3.1.18.2.NR5GC, 8.1.3.1.18.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 R5-213376 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | fl_TC_8_1_3_1_18_x_TestBody |
| --- | --- |
| Reason for change | Following the power level change at step 7 there is not enough time for the UE to measure the power levels before they are changes again at step 7A, and hence the UE may send the measurement report at step 8 still based on the power levels of T1. |
| Summary of change | Added a delay of 30 seconds between steps 7 and 7A to ensure that the UE has read the updated power levels and sends correct measurement report at step8. Note: an associated prose CR on 38.523-1 will be raised at RAN5#91-e |
| TTCN module | RRC_Measurement_CA_NR5GC.ttcn |
| MCC160 Comment | Accepted, will be implemented as per R5-213473 |

Before Change

function fl_TC_8_1_3_1_18_x_TestBody(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId1,  NR_CellId_Type p_NeigbourCellId2) runs on NR_BASE_PTC

{

…

…

…

//@siclog "Step 7" siclog@

//The SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.1.3.1.18.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//@siclog "Step 7A" siclog@

//The SS re-adjusts the cell-specific reference signal level according to row "T3" in table 8.1.3.1.18.1.3.2-1/2, 1 second after step 7

//@sic R5-202530, R5s200734 sic@

f_Delay(1.0);

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

…

…

…

After Change

function fl_TC_8_1_3_1_18_x_TestBody(NR_CellId_Type p_SCellId, NR_CellId_Type p_NeigbourCellId1,  NR_CellId_Type p_NeigbourCellId2) runs on NR_BASE_PTC

{

…

…

…

//@siclog "Step 7" siclog@

//The SS re-adjusts the cell-specific reference signal level according to row "T2" in table 8.1.3.1.18.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

//Wait 30s to ensure UE reads the new cell powers

f_Delay(30.0);

//@siclog "Step 7A" siclog@

//The SS re-adjusts the cell-specific reference signal level according to row "T3" in table 8.1.3.1.18.1.3.2-1/2, 1 second after step 7

//@sic R5-202530, R5s200734 sic@

f_Delay(1.0);

f_NR_SetCellPowerList(v_CellPowerList_AtT3);

…

…

}//End of fl_TC_8_1_3_1_18_x_TestBody