3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210575

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1725 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR Idle Mode test case 6.4.2.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 38.304 Cl. 5.2.7, the UE will stay in Any Cell Selection state. Once the serving cell becomes suitable again at Step 5, UE triggers RRC connection procedure during switchoff procedure at step 6B. Therefore, switchoff should be applied before serving cell becomes suitable again ie step 5 and step 6B needs to be switched. R5-212084 is raised for RAN5#91-e meeting |  |  |  |  |  |  |  |  |
| Summary of change: |  | Switch Step 5 and Step 6B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | function f_TC_6_4_2_1_NR5GC_TestBody |
| --- | --- |
| Reason for change | According to TS 38.304 Cl. 5.2.7, the UE will stay in Any Cell Selection state. Once the serving cell becomes suitable again at Step 5, UE triggers RRC connection procedure during switchoff procedure at step 6B. Therefore, switchoff should be applied before serving cell becomes suitable again ie step 5 and step 6B needs to be switched. R5-212084 is raised for RAN5#91-e meeting. |
| Summary of change | Switch Step 5 and Step 6B |
| TTCN module | Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted conditional to prose CR agreement at RAN5#91e. |

Before change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5s200377 R5-204415 R5-205600 sic@

//FR2 Power levels are FFS

var integer v_T1_Cell1_RS_EPRE_FR1 := -115;

var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1;

var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@

var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1;

var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2;

var octetstring v_EncodedRrcSetup;

var template(value) DL_CCCH_Message v_RrcSetup;

var template(value) CellGroupConfig v_CellGroupConfig;

var template(value) PDSCH_Config v_PDSCH_Config;

var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2;

var template(omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config;

var integer v_SSB_IndexCell2;

var template (value) PUSCH_Config v_PUSCH_Config;

v_RrcSetup := f_NR_38508_RRCSetup(nr_Cell1);

v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup);

v_CellPowerList_AtT0:={

cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR1),

cs_NR_CellPower(nr_Cell2, v_T0_Cell2_RS_EPRE_FR1, v_T0_Cell2_RS_EPRE_FR1)

};

v_CellPowerList_AtT1:={

cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1)

};

v_CellPowerList_AtT2:={

cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1),

cs_NR_CellPower(nr_Cell2, v_T2_Cell2_RS_EPRE_FR1, v_T2_Cell2_RS_EPRE_FR1)

};

// @siclog "Step 1" siclog@

// The SS changes NR Cell 1 SSS level according to the row "T1" in table 6.4.2.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

// @siclog "Step 2 - Void" siclog@

// @siclog "Step 2A" siclog@

// Wait 15s (Note 1)

f_Delay(15.0);

// @siclog "Step 3" siclog@

// The SS transmits a Paging message.

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

// @siclog "Step 4" siclog@

// Check: Does the UE attempt to transmit an uplink message within the next 10s?

f_NR_Check_NoUplinkMessage(10.0);

// @siclog "Step 5" siclog@

// The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2.

f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now);

f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1));

f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit);

f_NR_SetCellPowerList(v_CellPowerList_AtT0);

f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup)));

// @siclog "Steps 6/6A - Void" siclog@

// @siclog "Step 6B" siclog@

// The UE is switched off.

f_UT_SwitchOffUE(UT, false);

// @siclog "Step 6C" siclog@

// The UE is switched on.

f_UT_SwitchOnUE(UT, false);

f_NR5GC_MobileInfo_Init();// @sic R5s210092 sic@

// @siclog "Step 7" siclog@

// Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE.

f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s210092 sic@

f_NR_RRC_InactiveDef(nr_Cell1, -, false);

f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount);

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_CBRA_Msg4Based(f_NR_CellInfo_GetPhysicalParameters(nr_Cell2), cs_NR_RachProcedureMsg4WithNoRrcMsg));

// @siclog "Step 8-15 Void" siclog@

// @siclog "Step 16" siclog@

// The SS changes NR Cell 2 SSS level according to the row "T2" in table 6.4.2.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

// @siclog "Step 17 - Void" siclog@

// @siclog "Step 18" siclog@

// Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2?

SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update)));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18");

// @siclog "Step 18A" siclog@

// The SS transmits an RRCResume message

v_SSB_IndexCell2 := f_NR_CellInfo_GetSSB_Index(nr_Cell2);

v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id);

v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; // Table 6.4.2.1.3.3-6

v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit;

v_PDSCH_Config.tci_StatesToAddModList[0] := cs_38508_TCI_State(v_SSB_IndexCell2);

v_PDSCH_Config.prb_BundlingType := cs_NR_Prb_OmitBundleSize;

v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config};

v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2);

v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config);

v_PUSCH_Config := cs_38508_PUSCH_Config_FR1(cs_38508_DMRS_UplinkConfig(pos1), v_SSB_IndexCell2);

v_CellGroupConfig.spCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config := {setup := v_PUSCH_Config}; // @sic R5s210092 sic@

f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig);

// @siclog "Step 18B" siclog@

// The UE transmit an RRCResumeComplete message

f_NR_RRCResumeComplete_Def(nr_Cell2);

// @siclog "Step 19" siclog@

// The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition.

f_NR_RRC_InactiveDef(nr_Cell2);

}//end of f_TC_6_4_2_1_NR5GC_TestBody

After change:

function f_TC_6_4_2_1_NR5GC_TestBody() runs on NR5GC_PTC

{ // @sic R5s200377 R5-204415 R5-205600 sic@

//FR2 Power levels are FFS

var integer v_T1_Cell1_RS_EPRE_FR1 := -115;

var integer v_T2_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1;

var integer v_T2_Cell2_RS_EPRE_FR1 := -78; // @sic R5-198879 sic@

var integer v_T0_Cell1_RS_EPRE_FR1 := tsc_NR_ServingCellSSS_EPRE_FR1;

var integer v_T0_Cell2_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT0;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2;

var octetstring v_EncodedRrcSetup;

var template(value) DL_CCCH_Message v_RrcSetup;

var template(value) CellGroupConfig v_CellGroupConfig;

var template(value) PDSCH_Config v_PDSCH_Config;

var NR_DownlinkBWP_Type v_NR_DownlinkBWP_Cell2;

var template(omit) NR_BWP_DownlinkDedicated_PDSCH_Config_Type v_BWP_DownlinkDedicated_PDSCH_Config;

var integer v_SSB_IndexCell2;

var template (value) PUSCH_Config v_PUSCH_Config;

v_RrcSetup := f_NR_38508_RRCSetup(nr_Cell1);

v_EncodedRrcSetup := f_NR_RRC_DL_CCCH_Encvalue(v_RrcSetup);

v_CellPowerList_AtT0:={

cs_NR_CellPower(nr_Cell1, v_T0_Cell1_RS_EPRE_FR1, v_T0_Cell1_RS_EPRE_FR1),

cs_NR_CellPower(nr_Cell2, v_T0_Cell2_RS_EPRE_FR1, v_T0_Cell2_RS_EPRE_FR1)

};

v_CellPowerList_AtT1:={

cs_NR_CellPower(nr_Cell1, v_T1_Cell1_RS_EPRE_FR1, v_T1_Cell1_RS_EPRE_FR1)

};

v_CellPowerList_AtT2:={

cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR1),

cs_NR_CellPower(nr_Cell2, v_T2_Cell2_RS_EPRE_FR1, v_T2_Cell2_RS_EPRE_FR1)

};

// @siclog "Step 1" siclog@

// The SS changes NR Cell 1 SSS level according to the row "T1" in table 6.4.2.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT1);

// @siclog "Step 2 - Void" siclog@

// @siclog "Step 2A" siclog@

// Wait 15s (Note 1)

f_Delay(15.0);

// @siclog "Step 3" siclog@

// The SS transmits a Paging message.

f_NR_Paging_Def(nr_Cell1, RRC_INACTIVE);

// @siclog "Step 4" siclog@

// Check: Does the UE attempt to transmit an uplink message within the next 10s?

f_NR_Check_NoUplinkMessage(10.0);

// @siclog "Step 6B" siclog@

// The UE is switched off.

f_UT_SwitchOffUE(UT, false);

// @siclog "Step 5" siclog@

// The SS changes NR Cell 1 SSS levels according to the row "T0" in table 6.4.2.1.3.2-1/2.

f_NR_SS_RRC_ReleaseSecurity(nr_Cell1, cs_TimingInfo_Now);

f_NR_SS_SRBs_DRBs_Release(nr_Cell1, cs_TimingInfo_Now, f_NR_GetActiveDRBs(nr_Cell1));

f_NR_SS_SRBs_DRBs_Config(nr_Cell1, cs_TimingInfo_Now, omit);

f_NR_SetCellPowerList(v_CellPowerList_AtT0);

f_NR_SS_RachProcedureConfig(nr_Cell1, cs_NR_RachProcedureConfig_CBRA_Msg4Based( f_NR_CellInfo_GetPhysicalParameters(nr_Cell1), cs_NR_RachProcedureMsg4WithCcchMsg(v_EncodedRrcSetup)));

// @siclog "Steps 6/6A - Void" siclog@

// @siclog "Step 6C" siclog@

// The UE is switched on.

f_UT_SwitchOnUE(UT, false);

f_NR5GC_MobileInfo_Init();// @sic R5s210092 sic@

// @siclog "Step 7" siclog@

// Generic procedure specified in TS 38.508-1 Table 4.5.3.2-1 are performed, in which the RRCRelease message includes the suspendConfig with NR_RRC_INACTIVE condition thereby moving the UE to RRC_INACTIVE.

f_NR5GC_RRC_IdleState1N_Def(nr_Cell1, -, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s210092 sic@

f_NR_RRC_InactiveDef(nr_Cell1, -, false);

f_NR_SS_AS_ActivateSecurity_Current(nr_Cell2,tsc_NR_38508_NextHopChainingCount);

f_NR_SS_RachProcedureConfig(nr_Cell2, cs_NR_RachProcedureConfig_CBRA_Msg4Based(f_NR_CellInfo_GetPhysicalParameters(nr_Cell2), cs_NR_RachProcedureMsg4WithNoRrcMsg));

// @siclog "Step 8-15 Void" siclog@

// @siclog "Step 16" siclog@

// The SS changes NR Cell 2 SSS level according to the row "T2" in table 6.4.2.1.3.2-1/2.

f_NR_SetCellPowerList(v_CellPowerList_AtT2);

// @siclog "Step 17 - Void" siclog@

// @siclog "Step 18" siclog@

// Check: Does the UE transmit an RRCResumeRequest message on the NR Cell 2?

SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell2, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update)));

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18");

// @siclog "Step 18A" siclog@

// The SS transmits an RRCResume message

v_SSB_IndexCell2 := f_NR_CellInfo_GetSSB_Index(nr_Cell2);

v_NR_DownlinkBWP_Cell2 := f_NR_CellInfo_GetDownlinkBWP(nr_Cell2, tsc_NR_BWP_Id);

v_PDSCH_Config := v_NR_DownlinkBWP_Cell2.Pdsch.ConfigDedicated.R15; // Table 6.4.2.1.3.3-6

v_PDSCH_Config.dmrs_DownlinkForPDSCH_MappingTypeA := omit;

v_PDSCH_Config.tci_StatesToAddModList[0] := cs_38508_TCI_State(v_SSB_IndexCell2);

v_PDSCH_Config.prb_BundlingType := cs_NR_Prb_OmitBundleSize;

v_BWP_DownlinkDedicated_PDSCH_Config := {setup := v_PDSCH_Config};

v_CellGroupConfig := f_NR_GetSCGConfigResume(nr_Cell2);

v_CellGroupConfig.spCellConfig.spCellConfigDedicated.initialDownlinkBWP := cs_BWP_DownlinkDedicated_Def(-, v_BWP_DownlinkDedicated_PDSCH_Config);

v_PUSCH_Config := cs_38508_PUSCH_Config_FR1(cs_38508_DMRS_UplinkConfig(pos1), v_SSB_IndexCell2);

v_CellGroupConfig.spCellConfig.spCellConfigDedicated.uplinkConfig.initialUplinkBWP.pusch_Config := {setup := v_PUSCH_Config}; // @sic R5s210092 sic@

f_NR_SendRRCResume_Def(nr_Cell2, -, -, v_CellGroupConfig);

// @siclog "Step 18B" siclog@

// The UE transmit an RRCResumeComplete message

f_NR_RRCResumeComplete_Def(nr_Cell2);

// @siclog "Step 19" siclog@

// The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition.

f_NR_RRC_InactiveDef(nr_Cell2);

}//end of f_TC_6_4_2_1_NR5GC_TestBody