3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210576

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1726 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC Testcase 6.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-04-23 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Network does not establish SRB2 after security activation, thus all RRC messages should be sent and received on SRB1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify expected SRB to receive PDU Session Establishment message from SRB2 to SRB1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.3.1.1 which is part of the 5G NRGC_IRAT test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Narendra Kalahasti

Narendra.kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | function fl_TC_6_3_1_1_TestBody() |
| --- | --- |
| Reason for change | Network does not establish SRB2 after security activation, thus all RRC messages should be sent and received on SRB1. |
| Summary of change | Modify expected SRB to receive PDU Session Establishment message from SRB2 to SRB1 |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC{

…

//EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@

//@siclog "Step 18a1" siclog@//@sic R5-211379 sic@

// The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2,

//takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

if (pc_noOf_PDUsSameConnection > 0 ){

f_NR5GC_PDUSessionEstablishment (nr_Cell13);

}

}

After Change

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC{

…

//EXCEPTION: Step 18a1 is performed in pc_noOf_PDUsSameConnection > 0.//@sic R5-211379 sic@

//@siclog "Step 18a1" siclog@//@sic R5-211379 sic@

// The generic procedure for UE-requested PDU session establishmentspecified in subclause 4.5A.2,

//takes place performing establishment of UE-requested PDU session(s) with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

if (pc_noOf_PDUsSameConnection > 0 ){

f_NR5GC_PDUSessionEstablishment (nr_Cell13, -, tsc_NR_RbId_SRB1);

}

}