3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210577

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1727 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 9.1.5.1.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation of the Prose, function fl_TC_9_1_5_1_3_Body, for Step 2 the function f_NR5GC_PDUSessionEstablishment is called for Steps 13 – 18. In case the PICS pc_noOf_PDUsSameConnection is set to 0, the whole PDU Session establishment procedure will get missed. This needs to be corrected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Instead of calling function f_NR5GC_PDUSessionEstablishment at Steps 13-18, called function f_NR5GC_RRC_Idle_Steps16_20. This is how it is done for Steps 66-71 as well. Please see below. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TTCN not implementing the Prose fully. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.5.1.3.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  | TTCN impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Corrections	3

1.1	Correction to fl_TC_9_1_5_1_3_Body	3

- Corrections

  - Correction to fl_TC_9_1_5_1_3_Body

| Template name | fl_TC_9_1_5_1_3_Body |
| --- | --- |
| Reason for change | In the current TTCN implementation of the Prose, function fl_TC_9_1_5_1_3_Body, for Step 2 the function f_NR5GC_PDUSessionEstablishment is called for Steps 13 – 18. In case the PICS pc_noOf_PDUsSameConnection is set to 0, the whole PDU Session establishment procedure will get missed. This needs to be corrected. |
| Summary of change | Instead of calling function f_NR5GC_PDUSessionEstablishment at Steps 13-18, called function f_NR5GC_RRC_Idle_Steps16_20. This is how it is done for Steps 66-71 as well. Please see below. |
| TTCN module | Initial_Registration_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

<<SKIPPED CODE>> //@siclog "Step 13-18" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit)); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); f_NR_RRCRelease(v_NGC_NASCellA); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> //@siclog "Step 13-18" siclog@ SRB.receive(car_NR_SRB_NasPdu_IND(v_NGC_NASCellA, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed"); } /*COMMENTED OUT f_NR5GC_PDUSessionEstablishment (v_NGC_NASCellA, -, tsc_NR_RbId_SRB1, //@sic R5s200758 sic@ -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit));*/ f_NR5GC_RRC_Idle_Steps16_20 (v_NGC_NASCellA, -, -, -, -, -, -, -, -, -, -, -, -, cs_S_NSSAI('22'O, '01'O, '02'O, omit));//WA#9_1_5_1_3 f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); f_NR_RRCRelease(v_NGC_NASCellA); <<SKIPPED CODE>>