3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210596

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1740 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR idle mode test case 6.1.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | From R5s210085: Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add call to f_NR5GC_MobileInfo_Init() before step 24 Note : This change was previously included and accepted as part of R5s200085 but was not implemented in the following delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.1.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

2.1 Change 1

| Function name | f_TC_6_1_1_1_NR5GC_TestBody() |
| --- | --- |
| Reason for change | From R5s210085: Following deregistration of the UE, all stored information about DRBs and PDU sessions should be cleared since these no longer exist in the UE. For some test cases this is not being done via a call to f_NR5GC_MobileInfo_Init(), this can give execution issues later in the test case (e.g. AT commands for creating new PDU sessions are not issued) |
| Summary of change | Add call to f_NR5GC_MobileInfo_Init() before step 24 Note : This change was previously included and accepted as part of R5s200085 but was not implemented in the following delivery. |
| TTCN module | Idle_PLMNSelection_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_6_1_1_1_NR5GC_TestBody() runs on NR5GC_PTC { ... ... // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } ... ... …………… }

After Change:

function f_TC_6_1_1_1_NR5GC_TestBody() runs on NR5GC_PTC { ... ... // @siclog "Step 22A-22B" siclog@ // Steps 3-4 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed f_NR_RRCSetup_Def (nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?), (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { // @siclog "Step 23b1-23b2 Void" siclog@ // @siclog "Step 23b3-23b5a1" siclog@ // IF 5GS registration type is set as Mobility Registration in step 22B, THEN Steps 4 to 6a1 of the generic test procedure in TS 38.508-1 Table 4.9.5.2.2-1 are performed on NR Cell 1 f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell1, v_GMM_MobilityInfo); } else if (f_Check_NG_RegistrationReqMsg(v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_Secure)) { // @siclog "Step 23a1-23a17a1" siclog@ // IF 5GS registration type is set as Initial Registration in step 22B, THEN Steps 4 to 20a1of the generic test procedure in TS 38.508-1 Table 4.5.2.2-2 are performed on NR Cell 1 f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, TESTMode_OFF, -, Initial_Secure); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR5GC_MobileInfo_Init(); ... ... …………… }