3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210605

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1747 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the testcase 6.4.2.2.NR5GC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation , At step4 , SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE which is done by the function f_NR_RRC_InactiveDef. Inside the function f_NR_RRC_InactiveDef , TTCN also Configures SS with RACH procedure not to send any RRC msg in msg4 . Sending this local configuration is redundant as the SS is already updated to not send any RRC msg as part of msg4 during the earlier part of the test case preamble |  |  |  |  |  |  |  |  |
| Summary of change: |  | Redundant Rach configuration before sending the release message is removed |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_6_4_2_2_NR5GC_TestBody()	2

1.2	Correction to function f_NR_RRC_InactiveDef()	5

# Corrections required

## Correction to function f_TC_6_4_2_2_NR5GC_TestBody()

| Function name | f_TC_6_4_2_2_NR5GC_TestBody() |
| --- | --- |
| Reason for change | In the current TTCN implementation , At step4 , SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE which is done by the function f_NR_RRC_InactiveDef. Inside the function f_NR_RRC_InactiveDef , TTCN also Configures SS with RACH procedure not to send any RRC msg in msg4 . Sending this local configuration is redundant as the SS is already updated to not send any RRC msg as part of msg4 during the earlier part of the test case preamble |
| Summary of change | Redundant Rach configuration before sending the release message is removed. Fourth parameter passed in the function f_NR_RRC_InactiveDef decides whether to send the local rach configuration or not . |
| TTCN module | \ttcn-3\ttcn\develop\NR5GC\6_4_2 \ Inactive_CellReSelection_NR5GC.ttcn |
| MCC160 Comment | See change 2 |

Before Change:

function f_TC_6_4_2_2_NR5GC_TestBody() runs on NR_BASE_PTC { // @sic R5s200403 R5-203502 sic@ //FR2 Power levels are FFS <<< SKIPPED CODE >>> // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 4" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3, -, false); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 and NR Cell 6 SSS levels according to the row "T2" in table 6.4.2.2.3.2-1/2. f_NR_SS_AS_ActivateSecurity_Current(nr_Cell6); f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 6 - Void" siclog@ // @siclog "Step 7" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 6? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell6, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 8" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell6,-,false); // @siclog "Step 9" siclog@ // The SS changes NR Cell 6 SSS level according to the row "T3" in table 6.4.2.2.3.2-1/2. f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 10 - Void" siclog@ // @siclog "Step 11" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3); }//end of f_TC_6_4_2_2_NR5GC_TestBody

After Change:

function f_TC_6_4_2_2_NR5GC_TestBody() runs on NR_BASE_PTC { // @sic R5s200403 R5-203502 sic@ //FR2 Power levels are FFS <<< SKIPPED CODE >>> // @siclog "Step 2 - Void" siclog@ // @siclog "Step 3" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 3"); // @siclog "Step 4" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3, -, false, false); // @siclog "Step 5" siclog@ // The SS changes NR Cell 1 and NR Cell 6 SSS levels according to the row "T2" in table 6.4.2.2.3.2-1/2. f_NR_SS_AS_ActivateSecurity_Current(nr_Cell6); f_NR_SetCellPowerList(v_CellPowerList_AtT2); // @siclog "Step 6 - Void" siclog@ // @siclog "Step 7" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 6? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell6, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 8" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell6,-,false, false); // @siclog "Step 9" siclog@ // The SS changes NR Cell 6 SSS level according to the row "T3" in table 6.4.2.2.3.2-1/2. f_NR_SS_AS_ActivateSecurity_Current(nr_Cell3); f_NR_SetCellPowerList(v_CellPowerList_AtT3); // @siclog "Step 10 - Void" siclog@ // @siclog "Step 11" siclog@ // Check: Does the UE transmit an RRCResumeRequest message on the cell 3? SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell3, cr_38508_RRCResumeRequest(tsc_NR_ShortI_RNTI_Value1, rna_Update))); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 11"); // @siclog "Step 12" siclog@ // The SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE condition. f_NR_RRC_InactiveDef(nr_Cell3,-,-,false); }//end of f_TC_6_4_2_2_NR5GC_TestBody

## Correction to function f_NR_RRC_InactiveDef()

| Function name | function f_NR_RRC_InactiveDef() |
| --- | --- |
| Reason for change | In the current TTCN implementation , At step4 , SS transmits an RRCRelease message including suspendConfig with NR_RRC_INACTIVE which is done by the function f_NR_RRC_InactiveDef. Inside the function f_NR_RRC_InactiveDef , TTCN also Configures SS with RACH procedure not to send any RRC msg in msg4 . Sending this local configuration is redundant as the SS is already updated to not send any RRC msg as part of msg4 during the earlier part of the test case preamble |
| Summary of change | Redundant Rach configuration before sending the release message is removed. Fourth parameter passed in the function f_NR_RRC_InactiveDef decides whether to send the local rach configuration or not . |
| TTCN module | \ttcn-3\ttcn\develop\Common\NR5GC\ NR5GC_RRCSteps.ttcn |
| MCC160 Comment | Accepted but implemented using the attribute f_NR_EarlyContentionResolution – see below |

Before Change:

function f_NR_RRC_InactiveDef(NR_CellId_Type p_NR_CellId, template(omit) PLMN_Identity p_PLMN_Identity := omit, boolean p_ActivateSecurity := true ) runs on NR_BASE_PTC { //@sic R5-195325 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(p_NR_CellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); //Configure RACH procedure not to send any RRC msg in msg4 f_NR_SS_RachProcedureConfig(p_NR_CellId, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg)); f_NR_RRCRelease_Common (p_NR_CellId, cs_38508_RRCRelease ( tsc_NR_RRC_TI_Def, cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( v_CellIdentity, p_PLMN_Identity)))); if (p_ActivateSecurity){ //Activate security at SS side in advance so that SS is ready to send RRCResume message // Only SRBs are configured - so security is activated on SRBs //@sic R5s190505 change 2 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_NR_CellId,tsc_NR_38508_NextHopChainingCount); } }

After Change:

function f_NR_RRC_InactiveDef(NR_CellId_Type p_NR_CellId, template(omit) PLMN_Identity p_PLMN_Identity := omit, boolean p_ActivateSecurity := true, boolean p_ConfigureRach := true ) runs on NR_BASE_PTC { //@sic R5-195325 sic@ var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(p_NR_CellId); var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId); //Configure RACH procedure not to send any RRC msg in msg4 if(p_ConfigureRach) { f_NR_SS_RachProcedureConfig(p_NR_CellId, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg)); } f_NR_RRCRelease_Common (p_NR_CellId, cs_38508_RRCRelease ( tsc_NR_RRC_TI_Def, cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( v_CellIdentity, p_PLMN_Identity)))); if (p_ActivateSecurity){ //Activate security at SS side in advance so that SS is ready to send RRCResume message // Only SRBs are configured - so security is activated on SRBs //@sic R5s190505 change 2 sic@ f_NR_SS_AS_ActivateSecurity_Current(p_NR_CellId,tsc_NR_38508_NextHopChainingCount); } }

MCC160 proposed implementation:

function f_NR_RRC_InactiveDef(NR_CellId_Type p_NR_CellId,

template(omit) PLMN_Identity p_PLMN_Identity := omit,

boolean p_ActivateSecurity := true

) runs on NR_BASE_PTC

{ //@sic R5-195325 sic@

var CellIdentity v_CellIdentity := f_NR_CellInfo_GetCellIdentity(p_NR_CellId);

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(p_NR_CellId);

if (not(f_NR_EarlyContentionResolution())) {    //@sic R5s210605 sic@

//Configure RACH procedure not to send any RRC msg in msg4 - not needed when EarlyContention Resolution is applied

f_NR_SS_RachProcedureConfig(p_NR_CellId, cs_NR_RachProcedureConfig_CBRA_Msg4Based(v_NR_PhysicalParameters, cs_NR_RachProcedureMsg4WithNoRrcMsg));

}

f_NR_RRCRelease_Common (p_NR_CellId, cs_38508_RRCRelease ( tsc_NR_RRC_TI_Def, cs_38508_SuspendConfig(cs_RAN_NotificationAreaInfo_1Cell ( v_CellIdentity, p_PLMN_Identity))));

if (p_ActivateSecurity){

//Activate security at SS side in advance so that SS is ready to send RRCResume message

// Only SRBs are configured - so security is activated on SRBs

//@sic R5s190505 change 2 sic@

f_NR_SS_AS_ActivateSecurity_Current(p_NR_CellId,tsc_NR_38508_NextHopChainingCount);

}

}