3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210613

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1753 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to the NR5GC Testcase 6.3.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-27 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As per TS 38.523-1, steps 18a1-18a5 shall be executed if pc_SOR_ACKNotReqLocalRel set to false. Currently, steps are executed for true |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifed to execute 18a1-a5 when pc_SOR_ACKNotReqLocalRel set to false. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE will fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

  - Change 1

| Function name | function fl_TC_6_3_1_2_TestBody() |
| --- | --- |
| Reason for change | As per TS 38.523-1, steps 18a1-18a5 shall be executed if pc_SOR_ACKNotReqLocalRel set to false. Currently, steps are executed for true. |
| Summary of change | Modifed to execute 18a1-a5 when pc_SOR_ACKNotReqLocalRel set to false. |
| TTCN module | SOR_NR5GC.ttcn |
| MCC160 Comment | Accepted in principle. |

Before Change

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC{

…

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

//EXCEPTION: Steps 18a1-18a5 are performed IF pc_SOR_ACKNotReqLocalRel is False, ELSE step 18b1 is performed.//@sic R5-211380 sic@

if (pc_SOR_ACKNotReqLocalRel){//@sic R5-211380 sic@

//EXCEPTION: Step 18a1a1 is performed in pc_noOf_PDUsSameConnection > 0.

if (pc_noOf_PDUsSameConnection > 0 ){

//@siclog "Step 18a1a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

f_NR5GC_PDUSessionEstablishment (nr_Cell13);

}

//@siclog "Step 18a2" siclog@//@sic R5-211380 sic@

//The SS transmits an RRCRelease message.

f_NR_RRCRelease(nr_Cell13);

//EXCEPTION: Step 18a3a1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically

if (pc_noOf_PDUsNewConnection > 0 ){

//@siclog "Step 18a3a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection.

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

f_NR5GC_ExIdlePDUsession(nr_Cell13);

}

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

}

[] t_Wait.timeout{}

}

}

//@siclog "Steps 18a4-18a5a1"-Void siclog@//@sic R5-211380 sic@

}

else{

//@siclog "Step 18b1" siclog@//@sic R5-202598 sic@

//SS performs local release

f_NR_RRCRelease_Local(nr_Cell13, f_NR_GetNextSendOccasion_DL(nr_Cell13));

}

…

}

After Change

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC{

…

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

//EXCEPTION: Steps 18a1-18a5 are performed IF pc_SOR_ACKNotReqLocalRel is False, ELSE step 18b1 is performed.//@sic R5-211380 sic@

if (not pc_SOR_ACKNotReqLocalRel){//@sic R5-211380 sic@

//EXCEPTION: Step 18a1a1 is performed in pc_noOf_PDUsSameConnection > 0.

if (pc_noOf_PDUsSameConnection > 0 ){

//@siclog "Step 18a1a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

f_NR5GC_PDUSessionEstablishment (nr_Cell13);

}

//@siclog "Step 18a2" siclog@//@sic R5-211380 sic@

//The SS transmits an RRCRelease message.

f_NR_RRCRelease(nr_Cell13);

//EXCEPTION: Step 18a3a1 is performed if pc_noOf_PDUsNewConnection > 0 and the UE starts PDU session establishment automatically

if (pc_noOf_PDUsNewConnection > 0 ){

//@siclog "Step 18a3a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5.2.2-4, takes place from step 2 to 9a1 performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsNewConnection and then release the RRC Connection.

t_Wait.start;

alt {

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell13, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

f_NR5GC_ExIdlePDUsession(nr_Cell13);

}

[] SRB.check(receive(car_NR_SRB0_RrcPdu_IND(nr_Cell11, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_Any))))

{

t_Wait.stop;

}

[] t_Wait.timeout{}

}

}

//@siclog "Steps 18a4-18a5a1"-Void siclog@//@sic R5-211380 sic@

}

else{

//@siclog "Step 18b1" siclog@//@sic R5-202598 sic@

//SS performs local release

f_NR_RRCRelease_Local(nr_Cell13, f_NR_GetNextSendOccasion_DL(nr_Cell13));

}

…

}

MCC160 Comments:

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC{

…

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

//EXCEPTION: Steps 18a1-18a5 are performed IF pc_SOR_ACKNotReqLocalRel is False, ELSE step 18b1 is performed.//@sic R5-211380 sic@

if (not pc_SOR_ACKNotReqLocalRel){//@sic R5-211380 sic@//@sic R5s210613 sic@

//EXCEPTION: Step 18a1a1 is performed in pc_noOf_PDUsSameConnection > 0.

if (pc_noOf_PDUsSameConnection > 0 ){

//@siclog "Step 18a1a1" siclog@//@sic R5-211380 sic@

//The generic procedure for UE-requested PDU session establishment, specified in subclause 4.5A.2, takes place performing establishment of UE-requested PDU session(s)

//with ExpectedNumberOfNewPDUSessions = pc_noOf_PDUsSameConnection.

f_NR5GC_PDUSessionEstablishment (nr_Cell13,-, tsc_NR_RbId_SRB1);//@sic R5s210613 sic@

…

}