3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210614

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1754 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR MAC testcase 7.1.1.1.1a |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-05-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [TS 38.212, 7.3.1.2.1] If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: As per above, PDCCH Order shall be transmitted using C-RNTI using DCI format 1_0. [TS38.213, cl.10.1] - a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell - a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG - a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, or PS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and As per above, DCI for PDCCH order with C-RNTI can be transmitted in “Type-3 common search space” which shall be configured in PDCCH-Config. TS38.508-1 have not defined commonSearchSpace in PDCCH-Config but only in PDCCH-ConfigCommon. [TS38.213, cl.10.1] For a DL BWP, if a UE is not provided ra-SearchSpace for Type1-PDCCH CSS set, the UE does not monitor PDCCH for Type1-PDCCH CSS set on the DL BWP. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set. However, as highlighted above, there is an exception that Type1 CSS can be used if Type3-CSS is not available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PDCCH order has been configured to use cssType1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.1a.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | template cads_NR_DciTriggerPDCCHOrder_REQ |
| --- | --- |
| Reason for change | [TS 38.212, 7.3.1.2.1] If the CRC of the DCI format 1_0 is scrambled by C-RNTI and the "Frequency domain resource assignment" field are of all ones, the DCI format 1_0 is for random access procedure initiated by a PDCCH order, with all remaining fields set as follows: As per above, PDCCH Order shall be transmitted using C-RNTI using DCI format 1_0. [TS38.213, cl.10.1] - a Type1-PDCCH CSS set configured by ra-SearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a RA-RNTI, a MsgB-RNTI, or a TC-RNTI on the primary cell - a Type2-PDCCH CSS set configured by pagingSearchSpace in PDCCH-ConfigCommon for a DCI format with CRC scrambled by a P-RNTI on the primary cell of the MCG - a Type3-PDCCH CSS set configured by SearchSpace in PDCCH-Config with searchSpaceType = common for DCI formats with CRC scrambled by INT-RNTI, SFI-RNTI, TPC-PUSCH-RNTI, TPC-PUCCH-RNTI, TPC-SRS-RNTI, CI-RNTI, or PS-RNTI and, only for the primary cell, C-RNTI, MCS-C-RNTI, or CS-RNTI(s), and As per above, DCI for PDCCH order with C-RNTI can be transmitted in “Type-3 common search space” which shall be configured in PDCCH-Config. TS38.508-1 have not defined commonSearchSpace in PDCCH-Config but only in PDCCH-ConfigCommon. [TS38.213, cl.10.1] For a DL BWP, if a UE is not provided ra-SearchSpace for Type1-PDCCH CSS set, the UE does not monitor PDCCH for Type1-PDCCH CSS set on the DL BWP. If the UE has not been provided a Type3-PDCCH CSS set or a USS set and the UE has received a C-RNTI and has been provided a Type1-PDCCH CSS set, the UE monitors PDCCH candidates for DCI format 0_0 and DCI format 1_0 with CRC scrambled by the C-RNTI in the Type1-PDCCH CSS set. However, as highlighted above, there is an exception that Type1 CSS can be used if Type3-CSS is not available. |
| Summary of change | PDCCH order has been configured to use cssType1 |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted |

Before change:

// ASP TEMPLATES for PDCCH ORder

template(value) NR_SYSTEM_CTRL_REQ cads_NR_DciTriggerPDCCHOrder_REQ(NR_CellId_Type  p_NR_CellId,

template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template(value) NR_PDCCH_Order_Type p_NR_PDCCH_Order,

template(value) NR_AssignedBWPs_Type p_NR_AssignedBWPs_Type := cs_NR_AssignedBWPs_InitialBWP)

modifies cas_NR_CellConfigAllOmit_REQ :=

{ /* @status    APPROVED (ENDC, NR5GC) */

Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, tsc_NoCnfReq),

Request := {

DciTrigger := {

AssignedBWPs := p_NR_AssignedBWPs_Type,

SearchSpaceType := cssType2,

DciFormat := {

PdcchOrder := p_NR_PDCCH_Order

}

}

}

};

After change:

// ASP TEMPLATES for PDCCH ORder

template(value) NR_SYSTEM_CTRL_REQ cads_NR_DciTriggerPDCCHOrder_REQ(NR_CellId_Type  p_NR_CellId,

template(value) TimingInfo_Type p_TimingInfo := cs_TimingInfo_Now,

template(value) NR_PDCCH_Order_Type p_NR_PDCCH_Order,

template(value) NR_AssignedBWPs_Type p_NR_AssignedBWPs_Type := cs_NR_AssignedBWPs_InitialBWP)

modifies cas_NR_CellConfigAllOmit_REQ :=

{ /* @status    APPROVED (ENDC, NR5GC) */

Common := cs_NR_ReqAspCommonPart_CellCfg(p_NR_CellId, p_TimingInfo, tsc_NoCnfReq),

Request := {

DciTrigger := {

AssignedBWPs := p_NR_AssignedBWPs_Type,

SearchSpaceType := cssType1,

DciFormat := {

PdcchOrder := p_NR_PDCCH_Order

}

}

}

};