3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210653

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1765 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC testcase 11.3.9 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-16 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 24.501 cl 4.5.4.2, the barring is alleviated for the access category 33 when UE has camped on NR Cell 12 at step 6, UE in RRC_CONNECTED state may initiate the NAS transport procedure to send the PDU SESSION ESTABLISHMENT REQUEST message. Optional behaviour for UE shall be supplemented to resolve this issue. [TS 24.501, clause 4.5.4.2] … If the lower layers indicate that the access attempt is barred, the NAS shall take the following action depending on the event which triggered the access attempt: … c) if the event which triggered the access attempt was a request from upper layers to establish a new PDU session, 5GMM shall not initiate the NAS transport procedure to send the PDU SESSION ESTABLISHMENT REQUEST message. Upon receiving an indication from the lower layers that the barring is alleviated for the access category with which the access attempt was associated, the NAS may initiate the NAS transport procedure as specified in subclause 5.4.5, if still needed; |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify step 6, add steps 6A-6Ab2 and modify the steps number. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function fl_TC_11_3_9_TestBody	3

# Corrections required

## Correction to function fl_TC_11_3_9_TestBody

| Function name | fl_TC_11_3_9_TestBody |
| --- | --- |
| Reason for change | - According to CR R5-213507, the time should be 11.2s. - According to TS 24.501 cl 4.5.4.2, the barring is alleviated for the access category 33 when UE has camped on NR Cell 12 at step 6, UE in RRC_CONNECTED state may initiate the NAS transport procedure to send the PDU SESSION ESTABLISHMENT REQUEST message. Optional behaviour for UE shall be supplemented to resolve this issue. Note :it is supported with a prose CR. |
| Summary of change | - Modify the time 20 to 11.2. - Modify the procedure of step6. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | - Accepted. Implemented in 21wk24 - Accepted and implemented according to R5-214783 |

Before Change:

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) DNN v_DNN; var boolean v_ReceiveRRCSetupRequest; timer t_Wait := 20.0; v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); // @sic R5s210307 sic@ //@siclog "Step 3" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s? t_Wait.start; alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // @sic R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } // @siclog "Step 5" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2. v_CellPowerList_T1 := { //@sic R5-205388 sic@ cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 6" siclog@ //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection. f_NR5GC_MobilityRegistration(nr_Cell12); //@siclog "Step 7" siclog@ //Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 8" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data within 20s? t_Wait.start; v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Steps 9 - 14" siclog@ //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0000'B)) { // @sic R5s200654 R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); //@siclog "Steps 15 - 16" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 //The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, -, -, - , v_DNN, -, v_DNN); }

After Change:

function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2; var NR_SRB_COMMON_IND v_ReceivedAsp; var template (value) NR_CellPowerList_Type v_CellPowerList_T1; var template (omit) DNN v_DNN; var boolean v_ReceiveRRCSetupRequest; timer t_Wait; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; v_DNN := cs_DNN(-, f_DomainName_Encode(tsc_DNNValue)); // @sic R5s210307 sic@ //@siclog "Step 3" siclog@ f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 4" siclog@ // Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is the same value in UL NAS TRANSPORT message in preamble within 20s? t_Wait.start(11.2); alt{ []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { // @sic R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } []t_Wait.timeout { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 4"); } } // @siclog "Step 5" siclog@ //The SS adjusts the NR Cells power levels according to row "T1" in table 11.3.9.3.2-1/2. v_CellPowerList_T1 := { //@sic R5-205388 sic@ cs_NR_CellPower (nr_Cell1, tsc_NR_NonSuitableOffCellSSS_EPRE, tsc_NR_NonSuitableOffCellSSS_EPRE), cs_NR_CellPower (nr_Cell12, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2) }; f_NR_SetCellPowerList(v_CellPowerList_T1); // @siclog "Step 6" siclog@ //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection. //REMOVE f_NR5GC_MobilityRegistration(nr_Cell12,-, -, -, -, noRrcConnectionRelease); v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Mobility)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } f_NR_RRC_ActivateSecurity (nr_Cell12, valueof(v_ReceivedMsg.SecurityProtection.NasCount)); f_NR5GC_MobilityRegistration_Steps4_6(nr_Cell12, v_GMM_MobilityInfo, -, -, noRrcConnectionRelease); v_SRB:= tsc_NR_RbId_SRB1; //EXCEPTION: Steps 6Aa1-6Ab describes optional behaviour that depends on the UE implementation. t_Wait.start(10.0); alt{ //Check: Does the UE transmit a PDU SESSION ESTABLISHMENT REQUEST and the DNN value in UL NAS TRANSPORT message is the same as the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 []SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell12, v_SRB, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp { t_Wait.stop; f_NR5GC_PDUSessionEstablishment_InnerLoop(nr_Cell12, v_ReceivedAsp, SRB1, omit, -, v_DNN, -, v_DNN, -, -, -, -, -, -, -, -, -, -, -, -, -, -, -, cr_NG_Request_Type('001'B)); } []t_Wait.timeout { f_NR_RRCRelease(nr_Cell12); //@siclog "Step 6Ab3" siclog@ //Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command. f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue); //@siclog "Step 6Ab4" siclog@ //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data within 20s? t_Wait.start(20.0); v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData); if (v_ReceiveRRCSetupRequest) { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 8"); } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 8"); } //@siclog "Step 6Ab5-6Ab10" siclog@ //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed. f_NR_RRCSetup_Def (nr_Cell12); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell12, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def, ?,- , *), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0000'B)) { // @sic R5s200654 R5s210307 sic@ f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell12, v_ReceivedMsg); v_SRB:= tsc_NR_RbId_SRB2; //@siclog "Steps 15 - 16" siclog@ //The UE transmits a PDU SESSION ESTABLISHMENT REQUEST and the DNN in UL NAS TRANSPORT message is set to the same value with the DNN value of Operator-defined access category definitions defined in Table 11.3.9.3.3-1 //The SS transmits PDU SESSION ESTABLISHMENT ACCEPT message. f_NR5GC_PDUSessionEstablishment (nr_Cell12, 1, v_SRB, -, - , v_DNN, -, v_DNN); } } }

## MCC160 Implementation

## function fl_TC_11_3_9_TestBody() runs on NR5GC_PTC

## {

## var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

## var NR_RadioBearerId_Type v_SRB:= tsc_NR_RbId_SRB2;

## var NR_SRB_COMMON_IND v_ReceivedAsp;

## var template (value) NR_CellPowerList_Type v_CellPowerList_T1;

## var template (omit) DNN v_DNN;

## var boolean v_ReceiveRRCSetupRequest;

## timer t_Wait := 11.2; // @sic R5-213507 sic@

## timer t_WaitTime := 5.0; // @sic R5-214783 sic@

## ...

## // @siclog "Step 6" siclog@

## //The mobility registration updating procedure described in TS 38.508-1 [4] Table 4.9.5.2.2-1 is performed on NR Cell 12 and release RRC connection.

## f_NR5GC_MobilityRegistration(nr_Cell12);

## // @sic R5-214783 sic@

## // @siclog "Steps 6A - 6Bb1" siclog@

## t_WaitTime.start;

## alt{

## [] SRB.receive(car_NR_SRB0_RrcPdu_IND(nr_Cell12, cr_38508_RRCSetupRequest(cr_NR_EstablishmentCause_MoData, ?)))

## {

## t_WaitTime.stop;

## f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Ba1");

## }

## [] t_WaitTime.timeout

## {

## //@siclog "Step 6Bb2" siclog@

## // Make the UE attempt to establish a PDU session for the DNN defined in Table 11.3.9.3.3-1. This can be done by an AT/MMI command.

## f_UT_RequestPDUSession(UT, f_NR5GC_MobileInfo_GetNoOfSessions()+1, tsc_DNNValue);

## //@siclog "Step 6Bb3" siclog@

## //Check: Does the UE transmit a RRCSetupRequest message and establishmentCause is set to mo-Data?

## v_ReceiveRRCSetupRequest := f_NR_RRC_SetupRequest_Common( nr_Cell12, t_Wait, cr_NR_EstablishmentCause_MoData);

## if (v_ReceiveRRCSetupRequest) {

## f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 6Bb3");

## } else {

## f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 6Bb3");

## }

## }

## }

## //@siclog "Steps 9 - 14" siclog@

## //Steps 3-8 of the generic procedure for NR RRC_Connected specified in TS 38.508-1 Table 4.5.4.2-3 are performed.

## f_NR_RRCSetup_Def (nr_Cell12);

## …

## }