3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210654

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4620 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to templates using IntegrityCheckInfo as parameter |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI8_Test |  |  |  |  |  | Date: |  |  | 2021-06-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. Please see below templates affected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may FAIL the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.1, 8.1.3.7, 8.1.5.7, 8.4.2.2, 9.2.3.2.1.6, 9.2.3.2.1a, 9.2.3.2.1c, 11.2.10 and 13.1.4. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	3

1.1	Correction to function fl_UTRAN_RRC_ConnRel_DCH	3

1.2	Correction to function f_Get_cs_108_RB_SetUpSpeech	3

1.3	Correction to template cs_RadioBearerSetup_r8	4

1.4	Correction to template cs_108_RB_SetUpSpeech_FDD	4

1.5	Correction to template cs_PhysicalChannelReconfiguration	5

1.6	Correction to template cs_RadioBearerSetup_r9	6

1.7	Correction to template cs_108_RB_SetUpSpeech_r9_FDD	6

1.8	Correction to template cs_108_RB_SetUpSpeech_TDD	7

1.9	Correction to template cs_RadioBearerSetup_r10	8

1.10	Correction to template cs_108_RB_SetUpSpeech_r10_FDD	8

1.11	Correction to template cs_108_RB_SetUpSpeech_r10_TDD	9

1.12	Correction to template cs_RadioBearerSetup_r11	10

1.13	Correction to template cs_108_RB_SetUpSpeech_r11_FDD	11

1.14	Correction to template cs_108_RB_SetUpSpeech_r11_TDD	11

1.15	Correction to template cs_RadioBearerSetup_r12	12

1.16	Correction to template cs_108_RB_SetUpSpeech_r12_FDD	13

1.17	Correction to template cs_108_RB_SetUpSpeech_r12_TDD	14

1.18	Correction to template cs_RadioBearerSetup_r13	14

1.19	Correction to template cs_108_RB_SetUpSpeech_r13_FDD	15

1.20	Correction to template cs_108_RB_SetUpSpeech_r13_TDD	16

1.21	Correction to template cs_UE_CapabilityInfoCnfAM	17

1.22	Correction to template cs_RRCConnReject	18

1.23	Correction to template cs_108_RRC_ConnRelDCCH_r4	18

1.24	Correction to template cs_MeasurementControl_InterRATMeasurement	19

1.25	Correction to template cs_MeasurementControl_InterRATMeasurement_BandExt	20

1.26	Correction to template cs_MeasurementControl_InterRATMeasurement_NoCompMode	22

1.27	Correction to template cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt	23

1.28	Correction to template cs_MeasurementControl_r8	24

1.29	Correction to template cs_MeasurementControl_r8_ext	25

1.30	Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList	26

1.31	Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_BandExt	27

1.32	Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode	29

1.33	Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode_BandExt	30

1.34	Correction to template cs_UE_CapabilityEnqAll	31

1.35	Correction to template cs_108_UE_CapabilityEnq	32

1.36	Correction to template cs_MeasurementControl_InterRATMeasurement_TDD_r9	33

1.37	Correction to template cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt	34

1.38	Correction to template cs_MeasurementControl_TDD_r9	35

1.39	Correction to template cs_MeasurementControl_TDD_r9_ext	36

1.40	Correction to template cds_UE_CapabilityEnqNoTDD	37

# Corrections required

## Correction to function fl_UTRAN_RRC_ConnRel_DCH

| Function name | fl_UTRAN_RRC_ConnRel_DCH |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_IRAT_CommonFunctions.ttcn |
| MCC160 Comment | Alternative solution implemented: integrity will be set to omit if a new parameter p_USIM is false This new parameter is set to false in f_TC_11_2_10_UTRAN and step 14 of f_TC_9_2_3_2_1a_UTRAN |

Before Change:

<<SKIPPED CODE>> function fl_UTRAN_RRC_ConnRel_DCH(IntegrityCheckInfo p_IntegrityCheckInfo, integer p_N308) runs on UTRAN_PTC { <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_UTRAN_RRC_ConnRel_DCH(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, integer p_N308) runs on UTRAN_PTC { <<SKIPPED CODE>>

MCC160 Implementation:

function f_UTRAN_RRC_ConnRel(UTRAN_CellId_Type p_CellId,

RRC_Rel_Status    p_RRC_RelStatus,

boolean p_ConnEstFollows := false,

boolean           p_USIM := true) runs on UTRAN_PTC

{ /* @sic R5s120049: new parameter p_ConnEstFollows; change of default behaviour in case an RRC connection establishment is expected immediately after this function sic@ */

var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get();

var integer v_N308 := 2;     // Maximum number of retransmissions of the RRC CONNECTION RELEASE COMPLETE message.

var template (omit) IntegrityCheckInfo v_Integrityinfo := omit;

f_Delay(1.0);

if (p_USIM) {

v_Integrityinfo := v_SecurityInfo.dl_IntegrityCheckInfo;

}

if (p_RRC_RelStatus == cell_Dch) {

fl_UTRAN_RRC_ConnRel_DCH(v_Integrityinfo, v_N308);

} else if (p_RRC_RelStatus == cell_Fach_Dcch) {

U_UM.send(cas_RRC_ConnRelDCCH(utran_CellDedicated,

tsc_RB1,

cs_108_RRC_ConnRelDCCH_r4(v_Integrityinfo, tsc_RRC_TI)));

U_AM.receive(car_RLC_Data_Ind(utran_CellDedicated,

tsc_RB2,

cr_108_RRC_ConnRelCmpl(tsc_RRC_TI)));

f_UTRAN_PreliminaryPass(__FILE__, __LINE__, "");

} else if (p_RRC_RelStatus == cell_Fach_Ccch) {

U_UM.send(cas_RRC_ConnRelCCCH(p_CellId,

tsc_RB0,

cs_108_RRC_ConnRelCCCH_r5(cs_U_RNTI_Def, tsc_RRC_TI)));

} else {

FatalError(__FILE__, __LINE__, "invalid configuration");

}

function fl_UTRAN_RRC_ConnRel_DCH(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo,   // @sic R5s210654 sic@

integer            p_N308) runs on UTRAN_PTC

{

var integer v_K := 1;

timer t_WaitForRrcConRel := f_UG_SetTimerToleranceMax(5.0);

U_UM.send(cas_RRC_ConnRelDCCH(utran_CellDedicated,

tsc_RB1,

cs_108_RRC_ConnRelDCCH_r4(p_IntegrityCheckInfo, tsc_RRC_TI, p_N308)));

U_UM.receive(car_RRC_ConnRelCmplUM(utran_CellDedicated,

tsc_RB1,

cr_108_RRC_ConnRelCmpl(tsc_RRC_TI)));

f_UTRAN_PreliminaryPass(__FILE__, __LINE__, "");

//UE sends RRC Connection Release Complete for N308 times

do {

t_WaitForRrcConRel.start;

alt {

[] t_WaitForRrcConRel.timeout {

f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Insufficient RRC Connection Release");

return;

}

[] U_UM.receive(car_RRC_ConnRelCmplUM(utran_CellDedicated,

tsc_RB1,

cr_108_RRC_ConnRelCmpl(tsc_RRC_TI))) {

t_WaitForRrcConRel.stop;

v_K := v_K + 1;

}

}

} while (not (v_K == (p_N308 + 1)));

}

function fl_UTRAN_RRC_ConnRel_DCH(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo,

integer            p_N308) runs on UTRAN_PTC

{

var integer v_K := 1;

timer t_WaitForRrcConRel := f_UG_SetTimerToleranceMax(5.0);

U_UM.send(cas_RRC_ConnRelDCCH(utran_CellDedicated,

tsc_RB1,

cs_108_RRC_ConnRelDCCH_r4(p_IntegrityCheckInfo, tsc_RRC_TI, p_N308)));

U_UM.receive(car_RRC_ConnRelCmplUM(utran_CellDedicated,

tsc_RB1,

cr_108_RRC_ConnRelCmpl(tsc_RRC_TI)));

f_UTRAN_PreliminaryPass(__FILE__, __LINE__, "");

//UE sends RRC Connection Release Complete for N308 times

do {

t_WaitForRrcConRel.start;

alt {

[] t_WaitForRrcConRel.timeout {

f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Insufficient RRC Connection Release");

return;

}

[] U_UM.receive(car_RRC_ConnRelCmplUM(utran_CellDedicated,

tsc_RB1,

cr_108_RRC_ConnRelCmpl(tsc_RRC_TI))) {

t_WaitForRrcConRel.stop;

v_K := v_K + 1;

}

}

} while (not (v_K == (p_N308 + 1)));

}

## Correction to function f_Get_cs_108_RB_SetUpSpeech

| Function name | f_Get_cs_108_RB_SetUpSpeech |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RAB_Functions.ttcn |
| MCC160 Comment | Alternative solution implemented: integrity will be set to omit if a new parameter p_USIM is false |

Before Change:

<<SKIPPED CODE>> function f_Get_cs_108_RB_SetUpSpeech (UTRAN_CellId_Type p_CellId, IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activationtime) runs on UTRAN_PTC return template (value) DL_DCCH_Message { <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function f_Get_cs_108_RB_SetUpSpeech (UTRAN_CellId_Type p_CellId, template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activationtime) runs on UTRAN_PTC return template (value) DL_DCCH_Message { /* @sic R5-123750 sic@ */ <<SKIPPED CODE>>

MCC160 Implementation:

function f_UTRAN_RB_SetUp_Speech(UTRAN_CellId_Type p_CellId,

boolean           p_USIM := true) //@sic R5s110210 sic@

runs on UTRAN_PTC

{ /* @sic R5s120139 - Additional Changes sic@ */

var UTRAN_CFN_Info_Type v_CfnInfo := f_CalculateActTime(p_CellId);

var ActivationTime v_Activationtime := v_CfnInfo.actTime;

var template (value) SS_ActivationTime v_SS_ActivationTime := cs_ActivateCFN(v_Activationtime);

var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get();

var template (value) DL_DCCH_Message v_RB_SetUp := f_Get_cs_108_RB_SetUpSpeech(p_CellId, , v_Activationtime, p_USIM);

f_UTRAN_RB_SetUp_Send(v_RB_SetUp); //@sic R5s110210 Additional MCC160 changes sic@

f_SS_Config_ConvSpeech_RAB(p_CellId, v_SS_ActivationTime);  /* @sic R5s150684 sic@ */

f_UTRAN_RRC_ReceiveRB_SetupCmpl_Speech(p_CellId, v_Activationtime, p_USIM);

}

function f_Get_cs_108_RB_SetUpSpeech (UTRAN_CellId_Type  p_CellId,

,

ActivationTime     p_Activationtime,

boolean            p_USIM := true)

runs on UTRAN_PTC return template (value) DL_DCCH_Message

{ /* @sic R5-123750 sic@ */

/* @sic R5s150684: restructuring (nested functions/templates) sic@ */

var UTRAN_FddTddMessageClass_Type v_FddTddMessageClass := f_UTRAN_ConfigurationInfo_TestCaseRelease_Get();  // @sic R5s130195 Utran Release sic@

var UTRAN_FDD_TDD_Type v_FDD_TDD_Mode := f_UTRAN_CellInfo_GetFDD_TDD(p_CellId);

var template (value) DL_DCCH_Message v_RadioBearerSetup;

var B8_Type v_RAB_ID := tsc_RAB_DefCS;

var UL_ScramblingCode v_UL_ScramblingCode;

var PrimaryScramblingCode v_PrimaryScramblingCode;

var CellParametersID v_CellParametersID;

var UTRAN_SecurityInfo_Type v_SecurityInfo := f_UTRAN_Security_Get();

var template (omit) IntegrityCheckInfo v_Integrityinfo := omit;

if (p_USIM) {

v_Integrityinfo := v_SecurityInfo.dl_IntegrityCheckInfo;

}

select (v_FDD_TDD_Mode) {

case (UTRAN_FDD) {

v_UL_ScramblingCode := f_UTRAN_CellInfo_GetULScrmCode_FDD(p_CellId);

v_PrimaryScramblingCode := f_UTRAN_CellInfo_GetPriScrmCode_FDD(p_CellId);

select (v_FddTddMessageClass) {

case (FDDr8) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case (FDDr9) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r9_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case (FDDr10) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r10_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case (FDDr11) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r11_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case (FDDr12) { /* @sic R5s150329 BASELINE MOVING 2015 sic@  */

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r12_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case (FDDr14) { /* @sic R5-176156 sic@  */

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r13_FDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_PrimaryScramblingCode, v_UL_ScramblingCode);

}

case else {

FatalError(__FILE__, __LINE__, "FDD release not supported");

}

}

}

case (UTRAN_TDD) {

v_CellParametersID := f_UTRAN_CellInfo_GetCellParametersID_TDD(p_CellId);  // @sic R5s120719 sic@

select (v_FddTddMessageClass) {

case (TDDr9) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_TDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_CellParametersID);

}

case (TDDr10) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r10_TDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_CellParametersID);

}

case (TDDr11) {

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r11_TDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_CellParametersID);

}

case (TDDr12) { /* @sic R5s150329 BASELINE MOVING 2015 sic@  */

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r12_TDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_CellParametersID);

}

case (TDDr14) { /* @sic R5-176156 sic@  */

v_RadioBearerSetup := cs_108_RB_SetUpSpeech_r13_TDD(v_Integrityinfo, p_Activationtime, v_RAB_ID, v_CellParametersID);

}

case else {

FatalError(__FILE__, __LINE__, "TDD release not supported");

}

}

}

}

return v_RadioBearerSetup;

}

function f_UTRAN_MO_SpeechCall_Steps11_16(UTRAN_CellId_Type p_CellId,

TI                 p_TI_R,

boolean            p_PS_RB_Already_Est := false,

boolean            p_StopAtU4State := false ,

boolean            p_USIM := true) runs on UTRAN_PTC

{

var TI v_TI_S := p_TI_R;

v_TI_S.tiFlag := '1'B;

//Step 11  -  Send Call PROCEEDING

U_Dc.send(cas_DataReq_CS(utran_CellDedicated, tsc_RB3, cs_U_CallProc(v_TI_S)));

//Step 12-13 - Radio Bearer Establishment

if (p_PS_RB_Already_Est){

f_UTRAN_Setup_RAB_PS_and_ConvSpeech(p_CellId);

}

else {

f_UTRAN_RB_SetUp_Speech(p_CellId, p_USIM);  /* @sic R5s120139 - Additional Changes sic@ */

}

//Step 14  -  Send Alerting

U_Dc.send(cas_DataReq_CS(utran_CellDedicated, tsc_RB3, cs_U_Alert(v_TI_S)));

if (not p_StopAtU4State) { // @sic R5-150590 sic@

//Step 15  -  Send Connect

U_Dc.send(cas_DataReq_CS(utran_CellDedicated, tsc_RB3, cs_U_Connect(v_TI_S)));

//Step 16  -  Receive CONNECT ACKOWLEDGE

U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated, tsc_RB3, cr_U_ConnAck(p_TI_R)));

}

}

function f_TC_11_2_10_UTRAN() runs on UTRAN_IRAT_PTC

{

var U_RLC_TM_IND v_RLC_TR_DATA_IND; // @sic R5-150124 sic@

var InitialUE_Identity v_InitialUE_Id;

var TI v_TI_R;

var RRC_DATA_IND v_RRC_DataInd;

var template (present) EstablishmentCause v_EstCause := emergencyCall;

var KeySeq v_KeySeq := '111'B; //cipher key sequence number IE: no key is available

var float v_Wait := 60.0; // 60 seconds to allow UE to complete the procedure on UTRAN cell

var RB_ConfigType v_CellConfig;

timer t_Wait := v_Wait ; // Local wait Timer

//Initialise test case

f_UTRAN_Init(EUTRA_UTRAN);

//Create UTRAN cell5

f_UTRAN_SS_CreateCellDCH(utran_Cell5); // @sic R5s140846 sic@

//Start sending System Information

f_UTRAN_SendDefSysInfo ( utran_Cell5 );  // @sic R5s140731 R5-144741 sic@

//First Trigger, sent to UTRA after step 2 having been performed in EUTRA

f_IRAT_WaitForCoOrd_Trigger(EUTRA);

// Cell 5 powered after step 2

f_UTRAN_SetCellPower(utran_Cell5, -70, -70); // @sic R5s140731 R5-144741 R5s140961 R5s160755 sic@

f_UTRAN_TestBody_Set(true);

// Wait for 60s for the UE to complete cell selection. UTRA cell is less powerful than EUTRA cell

//@siclog "Step 2B" siclog@

f_Delay(60.0); // @sic R5s160755 sic@ @sic R5s160947 sic@

// Cause the UE to originate Emergency call. The request is assumed to be triggered by AT command D.

//@siclog "Step 3" siclog@

f_UT_RequestCSCall(UT, px_EmergencyCallNumber);

t_Wait.start;

alt {

//@siclog "Step 4" siclog@

//Check: Does the UE transmit an RRC CONNECTION REQUEST message on Cell 5 with Establishment cause: Emergency Call in the next [60] seconds?

[] U_TM.receive(car_RRC_ConnReq(utran_Cell5,

tsc_RB0,

cr_108_RRC_ConnReq_r8(v_EstCause))) -> value v_RLC_TR_DATA_IND {

f_UTRAN_PreliminaryPass(__FILE__, __LINE__, "Step 4");

v_InitialUE_Id := v_RLC_TR_DATA_IND.data.tM_message.uL_CCCH_Message.message_.rrcConnectionRequest.initialUE_Identity;  // @sic R5-150124 sic@

f_UTRAN_CheckReleaseIndicator(v_RLC_TR_DATA_IND.data.tM_message.uL_CCCH_Message.message_.rrcConnectionRequest.v3d0NonCriticalExtensions.v4b0NonCriticalExtensions.rrcConnectionRequest_v4b0ext.accessStratumReleaseIndicator); // @sic R5-150124 sic@

t_Wait.stop; // @sic R5s150887 sic@

}

[] t_Wait.timeout {

f_UTRAN_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4: RRC Connection Request not received within wait time");

}

}

//@siclog "Step 5" siclog@

// The SS transmits an RRC CONNECTION SETUP message on Cell 5.

//@siclog "Step 6" siclog@

// The UE transmits an RRC CONNECTION SETUP COMPLETE message on Cell 5.

f_UTRAN_RRC_ConnEst_WithoutRRCConnectionReq(utran_Cell5,v_InitialUE_Id);

//@siclog "Step 7" siclog@

// The UE transmits an INITIAL DIRECT TRANSFER message on Cell 5. This message includes a CM SERVICE REQUEST message.

U_Dc.receive(car_InitDirectTransfer(utran_CellDedicated,

tsc_RB3,

cr_U_ServReq_MO_Emergency(v_KeySeq, ?))); // UE id to be clarified

//@siclog "Step 8" siclog@

// The SS transmits a DOWNLINK DIRECT TRANSFER message on Cell 5. This message includes a CM SERVICE ACCEPT message.

U_Dc.send(cas_DataReq_CS(utran_CellDedicated,

tsc_RB3,

cs_U_CM_ServAcc)); // @sic R5s140731 R5-144741 sic@

//@siclog "Step 9" siclog@

// The UE transmits an UPLINK DIRECT TRANSFER message on Cell 5. This message includes an EMERGENCY SETUP message.

U_Dc.receive(car_UplinkDirectTransfer(utran_CellDedicated,

tsc_RB3,

cr_U_ESetup)) -> value v_RRC_DataInd;

//@siclog "Steps 10-15" siclog@

// steps 11-16 of 34.108 cl. 7.2.3.2.3

v_TI_R := v_RRC_DataInd.msg.esetup.ti;

f_UTRAN_MO_SpeechCall_Steps11_16(utran_Cell5, v_TI_R, -, -, false);

f_UTRAN_TestBody_Set(false);

// send coordination message for end of test case

f_IRAT_SendCoOrd(EUTRA, cms_IRAT_Trigger);

// Switch off/power off and release cells   @sic R5s140731 R5-144741 sic@

//Disconnect the active CC call

f_UTRAN_CC_Disconnect (cs_TI_MO );

//Release RRC connection

f_UTRAN_RRC_ConnRel( utran_Cell5, cell_Dch, -, false );

//SwitchPower off UE

f_UT_SwitchOffUE (UT, true, CNF_REQUIRED);

v_CellConfig := f_UTRAN_CellInfo_GetConfigType(utran_Cell5);

//Release the UTRAN cell

f_UTRAN_ReleaseCell(utran_Cell5, v_CellConfig);

}  // f_TC_11_2_10_UTRAN

## Correction to template cs_RadioBearerSetup_r8

| Template name | cs_RadioBearerSetup_r8 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r8(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r8_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r8(template (omit) IntegrityCheckInfo p_Integrityinfo,//WI768506 11.2.10 template (value) RadioBearerSetup_r8_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_FDD

| Template name | cs_108_RB_SetUpSpeech_FDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_FDD ( IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode ) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r8(p_Integrityinfo, cds_RadioBearerSetup_r8_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_FDD ( template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode ) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r8(p_Integrityinfo, cds_RadioBearerSetup_r8_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

## Correction to template cs_PhysicalChannelReconfiguration

| Template name | cs_PhysicalChannelReconfiguration |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a UE Physical Channel Reconfiguration message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_PhysicalChannelReconfiguration(IntegrityCheckInfo p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_PhysicalChannelReconfiguration(template (omit) IntegrityCheckInfo p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_RadioBearerSetup_r9

| Template name | cs_RadioBearerSetup_r9 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/ UTRAN_RRC_RAB_Templates_r9.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r9(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r9_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r9(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r9_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r9_FDD

| Template name | cs_108_RB_SetUpSpeech_r9_FDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/ UTRAN_RRC_RAB_Templates_r9.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r9_FDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r9(p_Integrityinfo, cds_RadioBearerSetup_r9_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r9_FDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r9(p_Integrityinfo, cds_RadioBearerSetup_r9_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_TDD

| Template name | cs_108_RB_SetUpSpeech_TDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/ UTRAN_RRC_RAB_Templates_r9.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_TDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r9(p_Integrityinfo, cds_RadioBearerSetup_r9_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_TDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r9(p_Integrityinfo, cds_RadioBearerSetup_r9_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

## Correction to template cs_RadioBearerSetup_r10

| Template name | cs_RadioBearerSetup_r10 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r10.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r10(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r10_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r10(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r10_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r10_FDD

| Template name | cs_108_RB_SetUpSpeech_r10_FDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r10.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r10_FDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r10(p_Integrityinfo, cds_RadioBearerSetup_r10_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r10_FDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r10(p_Integrityinfo, cds_RadioBearerSetup_r10_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r10_TDD

| Template name | cs_108_RB_SetUpSpeech_r10_TDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r10.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r10_TDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r10(p_Integrityinfo, cds_RadioBearerSetup_r10_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r10_TDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r10(p_Integrityinfo, cds_RadioBearerSetup_r10_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

## Correction to template cs_RadioBearerSetup_r11

| Template name | cs_RadioBearerSetup_r11 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r11.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r11(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r11_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r11(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r11_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r11_FDD

| Template name | cs_RadioBearerSetup_r11 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r11.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r11_FDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r11(p_Integrityinfo, cds_RadioBearerSetup_r11_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r11_FDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r11(p_Integrityinfo, cds_RadioBearerSetup_r11_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, p_PrimScramblingCode)); <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r11_TDD

| Template name | cs_108_RB_SetUpSpeech_r11_TDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r11.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r11_TDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r11(p_Integrityinfo, cds_RadioBearerSetup_r11_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r11_TDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r11(p_Integrityinfo, cds_RadioBearerSetup_r11_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

## Correction to template cs_RadioBearerSetup_r12

| Template name | cs_RadioBearerSetup_r12 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r12(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r12_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r12(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r12_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r12_FDD

| Template name | cs_108_RB_SetUpSpeech_r12_FDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r12_FDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r12(p_Integrityinfo, cds_RadioBearerSetup_r12_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, cs_DL_InformationPerRL_FDD_r12(p_PrimScramblingCode, tsc_DL_DPCH1_ChC_Speech, tsc_DL_DPCH1_2ndScrC))); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r12_FDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r12(p_Integrityinfo, cds_RadioBearerSetup_r12_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, cs_DL_InformationPerRL_FDD_r12(p_PrimScramblingCode, tsc_DL_DPCH1_ChC_Speech, tsc_DL_DPCH1_2ndScrC))); <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r12_TDD

| Template name | cs_RadioBearerSetup_r12 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r12_TDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r12(p_Integrityinfo, cds_RadioBearerSetup_r12_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r12_TDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r12(p_Integrityinfo, cds_RadioBearerSetup_r12_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

## Correction to template cs_RadioBearerSetup_r13

| Template name | cs_RadioBearerSetup_r13 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r13(IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r13_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_RadioBearerSetup_r13(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) RadioBearerSetup_r13_IEs p_RadioBearerSetup) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r13_FDD

| Template name | cs_108_RB_SetUpSpeech_r13_FDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r13_FDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r13(p_Integrityinfo, cds_RadioBearerSetup_r13_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, cs_DL_InformationPerRL_FDD_r13(p_PrimScramblingCode, tsc_DL_DPCH1_ChC_Speech, tsc_DL_DPCH1_2ndScrC))); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r13_FDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, PrimaryScramblingCode p_PrimScramblingCode, UL_ScramblingCode p_UL_ScramblingCode) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r13(p_Integrityinfo, cds_RadioBearerSetup_r13_IEs_Speech_FDD(p_Activetime, p_RAB_Id, p_UL_ScramblingCode, cs_DL_InformationPerRL_FDD_r13(p_PrimScramblingCode, tsc_DL_DPCH1_ChC_Speech, tsc_DL_DPCH1_2ndScrC))); <<SKIPPED CODE>>

## Correction to template cs_108_RB_SetUpSpeech_r13_TDD

| Template name | cs_108_RB_SetUpSpeech_r13_TDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_RAB_Templates_r12.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r13_TDD(IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r13(p_Integrityinfo, cds_RadioBearerSetup_r13_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RB_SetUpSpeech_r13_TDD(template (omit) IntegrityCheckInfo p_Integrityinfo, ActivationTime p_Activetime, bitstring p_RAB_Id, CellParametersID p_CellParametersID) := /* @status APPROVED (IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ) */ cs_RadioBearerSetup_r13(p_Integrityinfo, cds_RadioBearerSetup_r13_IEs_Speech_TDD(p_Activetime, p_RAB_Id, p_CellParametersID)); <<SKIPPED CODE>>

## Correction to template cs_UE_CapabilityInfoCnfAM

| Template name | cs_UE_CapabilityInfoCnfAM |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a UE Capability Information Confirm message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_UE_CapabilityInfoCnfAM(IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI) := { /* @status APPROVED (LTE_IRAT, UTRAN) */ integrityCheckInfo := p_IntegrityCheckInfo, message_ := { ueCapabilityInformationConfirm := { r3 := { ueCapabilityInformationConfirm_r3 := { rrc_TransactionIdentifier := p_RRC_TI }, laterNonCriticalExtensions := omit } } } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_UE_CapabilityInfoCnfAM(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI) := { /* @status APPROVED (LTE_IRAT, UTRAN) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

## Correction to template cs_RRCConnReject

| Template name | cs_RRCConnReject |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Alternative solution implemented: The integrityCheckInfo field of cs_RRCConnReject hard coded to omit and the parameter p_IntegrityCheckInfo removed as this field is set to omit in all instances. |

Before Change:

<<SKIPPED CODE>> template (value) DL_CCCH_Message cs_RRCConnReject(IntegrityCheckInfo p_IntegrityCheckInfo, InitialUE_Identity p_InitialUEId, integer p_WaitTime := 0, RRC_TransactionIdentifier p_RRC_Ti := tsc_RRC_TI, template (omit) RRCConnectionReject_r3_laterNonCriticalExtensions_Type p_LaterNonCriticalExtensions := omit) := { /* @status APPROVED (LTE_IRAT) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_CCCH_Message cs_RRCConnReject(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, InitialUE_Identity p_InitialUEId, integer p_WaitTime := 0, RRC_TransactionIdentifier p_RRC_Ti := tsc_RRC_TI, template (omit) RRCConnectionReject_r3_laterNonCriticalExtensions_Type p_LaterNonCriticalExtensions := omit) := { /* @status APPROVED (LTE_IRAT) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

MCC160 Implementation:

template (value) DL_CCCH_Message cs_RRCConnReject(

InitialUE_Identity  p_InitialUEId,

integer            p_WaitTime := 0,

RRC_TransactionIdentifier p_RRC_Ti := tsc_RRC_TI,

template (omit) RRCConnectionReject_r3_laterNonCriticalExtensions_Type p_LaterNonCriticalExtensions := omit) :=

{ /* @status    APPROVED (LTE_IRAT) */

integrityCheckInfo := omit,

## Correction to template cs_108_RRC_ConnRelDCCH_r4

| Template name | cs_108_RRC_ConnRelDCCH_r4 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RRC_ConnRelDCCH_r4(IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_Ti, template (omit) integer p_N308 := omit, template (omit) ReleaseCause p_ReleaseCause := normalEvent, template (omit) RRCConnectionRelease_v4d0NonCriticalExtensions_Type p_NonCriticalExtensions := omit) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_RRC_ConnRelDCCH_r4(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_Ti, template (omit) integer p_N308 := omit, template (omit) ReleaseCause p_ReleaseCause := normalEvent, template (omit) RRCConnectionRelease_v4d0NonCriticalExtensions_Type p_NonCriticalExtensions := omit) := { /* @status APPROVED (IMS, IMS_IRAT, LTE_A_IRAT, LTE_IRAT, POS, SSNITZ, UTRAN) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement

| Template name | cs_MeasurementControl_InterRATMeasurement |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); // @sic R5-122105 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement(template (omit) IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); // @sic R5-122105 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_BandExt

| Template name | cs_MeasurementControl_InterRATMeasurement_BandExt |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_BandExt(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, p_EUTRA_FrequencyInfoList_r11, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); // @sic R5s180225, R5-122105 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_BandExt(template (omit) IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, p_EUTRA_FrequencyInfoList_r11, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); // @sic R5s180225, R5-122105 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_NoCompMode

| Template name | cs_MeasurementControl_InterRATMeasurement_NoCompMode |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_NoCompMode(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, omit, // @sic R5s140149 sic@ p_RRC_TI); // @sic R5-122105 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_NoCompMode(template (omit) IntegrityCheckInfo p_Integrityinfo,//WI768506 8.4.2.2 RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, omit, // @sic R5s140149 sic@ p_RRC_TI); // @sic R5-122105 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt

| Template name | cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, p_EUTRA_FrequencyInfoList_r11, omit, // @sic R5s140149 sic@ p_RRC_TI); // @sic R5s180225, R5-122105 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_NoCompMode_BandExt(template (omit) IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth) := // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(cpich_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, {cs_InterRATEvent3a_Def}), -, p_EUTRA_FrequencyInfoList_r11, omit, // @sic R5s140149 sic@ p_RRC_TI); // @sic R5s180225, R5-122105 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_r8

| Template name | cs_MeasurementControl_r8 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_r8(IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r8 p_MeasurementCommand, template (value) MeasurementReportingMode p_MeasurementReportingMode := cs_MeasurementReportingMode, template (omit) DPCH_CompressedModeStatusInfo p_DPCH_CompressedModeStatusInfo := omit, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (LTE_A_IRAT, LTE_IRAT, POS) */ integrityCheckInfo := p_Integrityinfo, CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_r8(template (omit) IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r8 p_MeasurementCommand, template (value) MeasurementReportingMode p_MeasurementReportingMode := cs_MeasurementReportingMode, template (omit) DPCH_CompressedModeStatusInfo p_DPCH_CompressedModeStatusInfo := omit, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (LTE_A_IRAT, LTE_IRAT, POS) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_r8_ext

| Template name | cs_MeasurementControl_r8_ext |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_r8_ext(IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r8 p_MeasurementCommand, template (value) MeasurementReportingMode p_MeasurementReportingMode := cs_MeasurementReportingMode, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, template (omit) DPCH_CompressedModeStatusInfo p_DPCH_CompressedModeStatusInfo := omit, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_r8_ext(template (omit) IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r8 p_MeasurementCommand, template (value) MeasurementReportingMode p_MeasurementReportingMode := cs_MeasurementReportingMode, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, template (omit) DPCH_CompressedModeStatusInfo p_DPCH_CompressedModeStatusInfo := omit, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ integrityCheckInfo := p_Integrityinfo, <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList

| Template name | cs_MeasurementControl_InterRATMeasurement_InterRATEventList |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList(IntegrityCheckInfo p_Integrityinfo, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList(template (omit) IntegrityCheckInfo p_Integrityinfo, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_BandExt

| Template name | cs_MeasurementControl_InterRATMeasurement_InterRATEventList_BandExt |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_BandExt(IntegrityCheckInfo p_Integrityinfo, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, p_EUTRA_FrequencyInfoList_r11, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); //@sic R5s180225 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_BandExt(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, TGPS_Reconfiguration_CFN p_Tgps_Reconfiguration_CFN, TGCFN p_TGCFN, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, p_EUTRA_FrequencyInfoList_r11, cs_DPCH_CompressedModeStatusInfo(p_Tgps_Reconfiguration_CFN, p_TGCFN), p_RRC_TI); //@sic R5s180225 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode

| Template name | cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode(IntegrityCheckInfo p_Integrityinfo, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, omit, p_RRC_TI); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode(template (omit) IntegrityCheckInfo p_Integrityinfo, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, omit, p_RRC_TI); <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode_BandExt

| Template name | cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode_BandExt |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode_BandExt(IntegrityCheckInfo p_Integrityinfo, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, p_EUTRA_FrequencyInfoList_r11, omit, p_RRC_TI); //@sic R5s180225 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_InterRATEventList_NoCompMode_BandExt(template (omit) IntegrityCheckInfo p_Integrityinfo, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (value) InterRATEventList p_InterRATEventList, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := /* @status APPROVED (LTE_IRAT) */ cs_MeasurementControl_r8_ext(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r8(cs_EUTRA_FrequencyInfoList_1Entry(tsc_EUTRA_MaxEARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_r8(-, rsrp), // @sic R5s120565 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), -, p_EUTRA_FrequencyInfoList_r11, omit, p_RRC_TI); //@sic R5s180225 sic@ <<SKIPPED CODE>>

## Correction to template cs_UE_CapabilityEnqAll

| Template name | cs_UE_CapabilityEnqAll |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a UE Capability Enquiry message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_UE_CapabilityEnqAll(IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (UTRAN) */ integrityCheckInfo := p_IntegrityCheckInfo, <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_UE_CapabilityEnqAll(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := { /* @status APPROVED (UTRAN) */ integrityCheckInfo := p_IntegrityCheckInf <<SKIPPED CODE>>

## Correction to template cs_108_UE_CapabilityEnq

| Template name | cs_108_UE_CapabilityEnq |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a UE Capability Enquiry message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_UE_CapabilityEnq(IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI, template (omit) UECapabilityEnquiry.r3.laterNonCriticalExtensions p_NonCriticalExtensions := omit) := { /* @status APPROVED (LTE_IRAT) */ integrityCheckInfo := p_IntegrityCheckInfo, message_ := { ueCapabilityEnquiry := { r3 := { ueCapabilityEnquiry_r3 := { rrc_TransactionIdentifier := p_RRC_TI, capabilityUpdateRequirement := { ue_RadioCapabilityFDDUpdateRequirement := true, ue_RadioCapabilityTDDUpdateRequirement := false, systemSpecificCapUpdateReqList := omit } }, laterNonCriticalExtensions := p_NonCriticalExtensions } } } }; <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_108_UE_CapabilityEnq(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI, template (omit) UECapabilityEnquiry.r3.laterNonCriticalExtensions p_NonCriticalExtensions := omit) := { /* @status APPROVED (LTE_IRAT) */ integrityCheckInfo := p_IntegrityCheckInfo, message_ := { ueCapabilityEnquiry := { r3 := { ueCapabilityEnquiry_r3 := { rrc_TransactionIdentifier := p_RRC_TI, capabilityUpdateRequirement := { ue_RadioCapabilityFDDUpdateRequirement := true, ue_RadioCapabilityTDDUpdateRequirement := false, systemSpecificCapUpdateReqList := omit } }, laterNonCriticalExtensions := p_NonCriticalExtensions } } } }; <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_TDD_r9

| Template name | cs_MeasurementControl_InterRATMeasurement_TDD_r9 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates_r9.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_TDD_r9(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, template (value) InterRATEventList p_InterRATEventList := {cs_InterRATEvent3a_Def}) := // @sic R5s140120 sic@ // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_TDD_r9(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r9(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_TDD_r8(primaryCCPCH_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), // @sic R5s140120 sic@ p_CellDCHMeasOccasionInfo_TDD128, // @sic R5s130342 sic@ p_RRC_TI); <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_TDD_r9(template (omit) IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, template (value) InterRATEventList p_InterRATEventList := {cs_InterRATEvent3a_Def}) := // @sic R5s140120 sic@ // Default values according to 36.508 Table 4.7B.1-3: MEASUREMENT CONTROL for Inter-RAT and special settings @sic R5s120686 sic@ /* @status APPROVED (LTE_A_IRAT, LTE_IRAT) */ cs_MeasurementControl_TDD_r9(p_Integrityinfo, 3, cs_MeasurementCommand_SetupInterRatEutra_r9(cs_EUTRA_FrequencyInfoList_1Entry(p_EutraCell_ARFCN, p_EutraCell_MeasBandwidth), cs_InterRATMeasQuantity_TDD_r8(primaryCCPCH_RSCP, rsrp), // @sic R5-122105 sic@ @sic R5s130195 BASELINE MOVING 2013 sic@ cs_InterRATReportingQuantity_r8, p_InterRATEventList), // @sic R5s140120 sic@ p_CellDCHMeasOccasionInfo_TDD128, // @sic R5s130342 sic@ p_RRC_TI); <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt

| Template name | cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates_r9.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(IntegrityCheckInfo p_Integrityinfo, RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, template (value) InterRATEventList p_InterRATEventList := {cs_InterRATEvent3a_Def}) := // @sic R5s140120 sic@ <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_InterRATMeasurement_TDD_r9_BandExt(template (omit) IntegrityCheckInfo p_Integrityinfo,//WI768506 8.1.3.7 RRC_TransactionIdentifier p_RRC_TI, integer p_EutraCell_ARFCN, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, EUTRA_MeasurementBandwidth p_EutraCell_MeasBandwidth, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, template (value) InterRATEventList p_InterRATEventList := {cs_InterRATEvent3a_Def}) := // @sic R5s140120 sic@ <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_TDD_r9

| Template name | cs_MeasurementControl_TDD_r9 |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates_r9.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_TDD_r9(IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r9 p_MeasurementCommand, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_TDD_r9(template (omit) IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r9 p_MeasurementCommand, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := <<SKIPPED CODE>>

## Correction to template cs_MeasurementControl_TDD_r9_ext

| Template name | cs_MeasurementControl_TDD_r9_ext |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/UTRAN_RRC_Templates_r9.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a Measurement Control message is sent. |

Before Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_TDD_r9_ext(IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r9 p_MeasurementCommand, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) := <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template (value) DL_DCCH_Message cs_MeasurementControl_TDD_r9_ext(template (omit) IntegrityCheckInfo p_Integrityinfo, MeasurementIdentity p_MeasurementIdentity, template (value) MeasurementCommand_r9 p_MeasurementCommand, template (value) EUTRA_FrequencyInfoList_r11 p_EUTRA_FrequencyInfoList_r11, template (omit) CellDCHMeasOccasionInfo_TDD128_r9 p_CellDCHMeasOccasionInfo_TDD128, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) :=<<SKIPPED CODE>>

## Correction to template cds_UE_CapabilityEnqNoTDD

| Template name | cds_UE_CapabilityEnqNoTDD |
| --- | --- |
| Reason for change | In certain test cases RRC message are sent with IntegrityCheckInfo set to omit as RRC security is not activated, the current code results in a runtime exception as the template is expecting a value for integrity check info. |
| Summary of change | Changed the definition of different UTRAN teamplates making the paramter p_Integrityinfo as optional. |
| TTCN module | Common/UTRAN/8_1/UTRAN34_UE_CapabilityInfo.ttcn |
| MCC160 Comment | Rejected. According to 33.102 cl.6.4.5, integrity shall be started before a UE Capability Enquiry message is sent. |

Before Change:

<<SKIPPED CODE>> template(value) DL_DCCH_Message cds_UE_CapabilityEnqNoTDD(IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) modifies cs_UE_CapabilityEnqAll := <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> template(value) DL_DCCH_Message cds_UE_CapabilityEnqNoTDD(template (omit) IntegrityCheckInfo p_IntegrityCheckInfo, RRC_TransactionIdentifier p_RRC_TI := tsc_RRC_TI) modifies cs_UE_CapabilityEnqAll := <<SKIPPED CODE>>