3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210657

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-3 | CR | 4621 | rev | - | Current version: | 16.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to CIoT TC 23.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13_Test, LTE_MTCe2_L1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - In the case of CATM1 UEs, during the execution of TC 23.1.1 with pc_APN_RateControl set to true , with the existing TTCN code,TC fails as the “for loop condition” and t_PLMNRateControl at step 17a6 and v_ProtocolContainerList at step 18a1 are not correct. - Incase of f_EUTRA_SetTimerToleranceMax, at step 17a6, test fails upon transmission of 2nd ESM_DATA if the UE sends immediately after the expiry of PLMNrateControlTimer. UE : <1st_ESM_DATA><T_plmnRateControl_1min><2ndESM_DATA>< T_plmnRateControl_1min> SS : <1st_ESM_DATA><T_plmnRateControl_1min+MaxTolerance>< Ready for 2ndESM_DATA> Therefore, it is proposed to used f_EUTRA_SetTimerToleranceMin so that SS can be ready to receive the 2ndESM_DATA with the below time SS : <1st_ESM_DATA><T_plmnRateControl_1min-MinTolerance>< Ready for 2ndESM_DATA |  |  |  |  |  |  |  |  |
| Summary of change: |  | - for loop condition is modified and and v_ProtocolContainerList at step 18a1 need correction from v_ProtocolContainerIPv4 to v_ProtocolContainerAPN - function f_EUTRA_SetTimerToleranceMin () used instead of f_EUTRA_SetTimerToleranceMax |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant CATM1 UE will fail if the TC 23.1.1 executed with pc_APN_RateControl set to true. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 23.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	2

2	Corrections required	2

2.1	Change 1	2

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## Change 1

| Function name | f_TC_23_1_1_EUTRA() |
| --- | --- |
| Reason for change | - In the case of CATM1 UEs, during the execution of TC 23.1.1 with pc_APN_RateControl set to true , with the existing TTCN code,TC fails as the “for loop condition” and t_PLMNRateControl at step 17a6 and v_ProtocolContainerList at step 18a1 are not correct. - Incase of f_EUTRA_SetTimerToleranceMax, at step 17a6, test fails upon transmission of 2nd ESM_DATA if the UE sends immediately after the expiry of PLMNrateControlTimer. UE : <1st_ESM_DATA><T_plmnRateControl_1min><2ndESM_DATA>< T_plmnRateControl_1min> SS : <1st_ESM_DATA><T_plmnRateControl_1min+MaxTolerance>< Ready for 2ndESM_DATA> Therefore, it is proposed to used f_EUTRA_SetTimerToleranceMin so that SS can be ready to receive the 2ndESM_DATA with the below time SS : <1st_ESM_DATA><T_plmnRateControl_1min-MinTolerance>< Ready for 2ndESM_DATA> |
| Summary of change | - for loop condition is modified and and v_ProtocolContainerList at step 18a1 need correction from v_ProtocolContainerIPv4 to v_ProtocolContainerAPN - function f_EUTRA_SetTimerToleranceMin () used instead of f_EUTRA_SetTimerToleranceMax() |
| TTCN module | \LTE_A_PRO\23\ CIoT_Optimization_CP.ttcn |
| MCC160 Comment | Accepted |

Before change:

<<SKIPPED CODE>>

//@siclog "Step 17a5" siclog@

//SS transmits an ESM DATA TRANSPORT message containing 320 Octets of downlink user data.

SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1,

tsc_SRB1,         //@sic R5s180379 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, v_PTI, cs_UserDataContainer(v_DataStep17a5)))));

v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd ('00'B) ifpresent;

for (i:=0; i==1; i:=i+1) {  //Steps 17a6-17a8 are repeated  2 times.

//@siclog "Step 17a6" siclog@

//SS starts timer 1 min APN Rate control

t_PLMNRateControl.start( f_EUTRA_SetTimerToleranceMax (eutra_Cell1, nasTimer, 60.0));

//@siclog "Step 17a7" siclog@

//Check: Does the UE send an ESM DATA TRANSPORT message respecting the APN set in step 17a1: Max size of message 256 Octets?

//@sic R5s180379 sic@

SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_ESM_DATA_TRANSPORT(tsc_EpsDefaultBearerId, cr_UserDataContainer(int2oct(lengthof(v_DataStep17a5),2),v_DataStep17a5), v_ReleaseAssistanceInd))));

//@siclog "Step 17a8" siclog@

//Wait until timer 1 min APN Rate control expires.

t_PLMNRateControl.timeout;

v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd (('00'B, '01'B)) ifpresent;

}

}

//Steps 18a1-18a6 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports MTU parameters

if (pc_NonIP_Link_MTU_Parameter or pc_IPv4_Link_MTU_Parameter) {

if (pc_IPv4_Link_MTU_Parameter) {

v_ProtocolContainerIPv4 := cs_ProtocolContainer ('0010'O, int2oct(128, 2));

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4;

v_PCOCount := v_PCOCount + 1 ;

}

if (pc_NonIP_Link_MTU_Parameter) {

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4;

v_PCOCount := v_PCOCount + 1 ;

}

if (pc_APN_RateControl) {

v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, '00'O & 'FFFFFF'O);

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4;

}

v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX (v_ProtocolContainerList);

//@siclog "Step 18a1" siclog@

<<SKIPPED CODE>>

After change:

<<SKIPPED CODE>>

//@siclog "Step 17a5" siclog@

//SS transmits an ESM DATA TRANSPORT message containing 320 Octets of downlink user data.

SRB.send(cas_SRB_NasPdu_REQ(eutra_Cell1,

tsc_SRB1,         //@sic R5s180379 sic@

cs_TimingInfo_Now,

cs_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered,

cs_ESM_DATA_TRANSPORT (tsc_EpsDefaultBearerId, v_PTI, cs_UserDataContainer(v_DataStep17a5)))));

v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd ('00'B) ifpresent;

for (i:=0; i<2; i:=i+1) {  //Steps 17a6-17a8 are repeated  2 times.

//@siclog "Step 17a6" siclog@

//SS starts timer 1 min APN Rate control

t_PLMNRateControl.start( f_EUTRA_SetTimerToleranceMin (eutra_Cell1, nasTimer, 60.0));

//@siclog "Step 17a7" siclog@

//Check: Does the UE send an ESM DATA TRANSPORT message respecting the APN set in step 17a1: Max size of message 256 Octets?

//@sic R5s180379 sic@

SRB.receive(car_SRB1_NasPdu_IND(eutra_Cell1,

cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered,

cr_ESM_DATA_TRANSPORT(tsc_EpsDefaultBearerId, cr_UserDataContainer(int2oct(lengthof(v_DataStep17a5),2),v_DataStep17a5), v_ReleaseAssistanceInd))));

//@siclog "Step 17a8" siclog@

//Wait until timer 1 min APN Rate control expires.

t_PLMNRateControl.timeout;

v_ReleaseAssistanceInd := cr_ReleaseAssistanceInd (('00'B, '01'B)) ifpresent;

}

}

//Steps 18a1-18a6 describe behaviour that depends on UE capabilities; the "lower case letter" identifies a step sequence that take place depending on whether the UE supports MTU parameters

if (pc_NonIP_Link_MTU_Parameter or pc_IPv4_Link_MTU_Parameter) {

if (pc_IPv4_Link_MTU_Parameter) {

v_ProtocolContainerIPv4 := cs_ProtocolContainer ('0010'O, int2oct(128, 2));

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4;

v_PCOCount := v_PCOCount + 1 ;

}

if (pc_NonIP_Link_MTU_Parameter) {

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerIPv4;

v_PCOCount := v_PCOCount + 1 ;

}

if (pc_APN_RateControl) {

v_ProtocolContainerAPN := cs_ProtocolContainer ('0016'O, '00'O & 'FFFFFF'O);

v_ProtocolContainerList[v_PCOCount] := v_ProtocolContainerAPN;

}

v_NewPco := f_NAS_ExtdProtocolConfigOptionsTX (v_ProtocolContainerList);

//@siclog "Step 18a1" siclog@		 */<<SKIPPED CODE>>