3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210658

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1767 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR5GC SOR TCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | With the implementation of Prose CR R5-211378, procedure changes to clear UE’s Last Registered PLMN (RPLMN) were implemented. Registration Reject procedure defined in TS 38.508-1 Subclause 4.9.8, which was previously used to clear UE’s RPLMN list was replaced with AT Command sequence. With this change, UE can send initial registration request with integrity protection. Thus, the initial Registration_Request should now be handled in both cases (with or without Integrity protection), depending on UE’s current status of NAS Security Context. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Allow initial Registration_Request to be sent with or without Integrity protection, depending on UE’s current status of NAS Security Context. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.NR5GC, 6.3.1.2.NR5GC, 6.3.1.3.NR5GC, 6.3.1.4.NR5GC, 6.3.1.8.NR5GC, 6.3.1.9.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

- Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk12 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

- Corrections required

## 2.1 Change 1

| Function name | fl_TC_6_3_1_1_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@

var O16_Type v_Mac_UE;

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

timer t_Wait:= 10.0;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header);

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B),v_Mac_UE)))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed");

}

}

After change:

function fl_TC_6_3_1_1_TestBody() runs on NR5GC_PTC

{

var NR_SRB_COMMON_IND v_ReceivedAsp;

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202596 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202596 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202596 sic@

var O16_Type v_Mac_UE;

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200663 sic@

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

timer t_Wait:= 10.0;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

v_Mac_UE:=f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message carrying acknowledgement of successful reception of the steering of roaming information

SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell13, tsc_NR_RbId_SRB1, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp;

if (not match (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, cr_NG_REGISTRATION_COMPLETE(cr_NG_SORTransparentContainer(crs_NG_SOR_Header('0'B,'0'B,'0'B,'1'B),v_Mac_UE)))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Complete Message Failed");

}

}

## 2.2 Change 2

| Function name | fl_TC_6_3_1_2_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before change:

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202598 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202598 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var NAS_PlmnId v_PlmnID_13 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_14 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13));//@sic R5-202598 sic@

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200672 sic@

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId & v_PlmnID_13 & v_AccessTechId & v_PlmnID_14 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('0'B,'1'B,'1'B,'0'B);

timer t_Tmax;

timer t_Wait:= 10.0;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header);

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

}

After change:

function fl_TC_6_3_1_2_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202598 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202598 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var NAS_PlmnId v_PlmnID_13 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-202598 sic@

var NAS_PlmnId v_PlmnID_14 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell13));//@sic R5-202598 sic@

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200672 sic@

var octetstring v_Data := v_PlmnID_2 & v_AccessTechId & v_PlmnID_13 & v_AccessTechId & v_PlmnID_14 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('0'B,'1'B,'1'B,'0'B);

timer t_Tmax;

timer t_Wait:= 10.0;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,-,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

}

## 2.3 Change 3

| Function name | fl_TC_6_3_1_3_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@

var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId;

var octetstring v_Data_2:=v_PlmnID_3 & v_AccessTechId;//@sic R5-204543 sic@

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13siclog "Steps 2-15" siclog@

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,true);

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

}

After change:

function fl_TC_6_3_1_3_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;//@sic R5-202599 sic@

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;//@sic R5-202599 sic@

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202599 sic@

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var NAS_PlmnId v_PlmnID_3 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell12));//@sic R5-204543 sic@

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200680 sic@

var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId;

var octetstring v_Data_2:=v_PlmnID_3 & v_AccessTechId;//@sic R5-204543 sic@

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2-14" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13siclog "Steps 2-15" siclog@

//@siclog "Step 15" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

f_NR5GC_SorRegistration(nr_Cell13,v_Data,v_Header,true,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

//@siclog "Step 16" siclog@

//SS starts timer of tmax =(6 minutes + cell selection time) (Note 1, 2 and 3)

t_Tmax.start(f_NR_SetTimerToleranceMax(nr_Cell13, nonProtocolTimer, 660.0));

//@siclog "Step 17" siclog@

//The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell13);

}

## 2.4 Change 4

| Function name | fl_TC_6_3_1_4_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before change:

function fl_TC_6_3_1_4_TestBody() runs on NR5GC_PTC

{

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200682 sic@

var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Step 2" siclog@

//PLMN3 is manually selected

f_UT_ManualPLMN_Select(UT,

f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell12)));//@sic R5s200682 sic@

//@siclog "Steps 3-15" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 16" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception

f_NR5GC_SorRegistration(nr_Cell12,v_Data,v_Header,true);//@sic R5s200682 sic@

//@siclog "Step 17" siclog@

////The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell12);

}

After change:

function fl_TC_6_3_1_4_TestBody() runs on NR5GC_PTC

{

var NAS_PlmnId v_PlmnID_2 := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11));

var O2_Type v_AccessTechId :='0800'O;//@sic R5s200682 sic@

var octetstring v_Data:=v_PlmnID_2 & v_AccessTechId;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Step 2" siclog@

//PLMN3 is manually selected

f_UT_ManualPLMN_Select(UT,

f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell12)));//@sic R5s200682 sic@

//@siclog "Steps 3-15" siclog@

//Steps 1 to 13 of the registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13

//@siclog "Step 16" siclog@

//The SS transmits an DLInformationTransfer message and an REGISTRATION ACCEPT message containing

//steering of roaming information, includes unmatched SOR-MAC-IAUSF that will result in unsuccessful security check while on UE reception

f_NR5GC_SorRegistration(nr_Cell12,v_Data,v_Header,true,-,(tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));//@sic R5s200682 sic@

//@siclog "Step 17" siclog@

////The UE transmits an ULInformationTransfer message and REGISTRATION COMPLETE message without Steering of Roaming Transparent container.

f_NR5GC_Registration_Complete(nr_Cell12);

}

## 2.5 Change 5

| Function name | fl_TC_6_3_1_8_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before change:

function fl_TC_6_3_1_8_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var octetstring  v_NAS_TxMsgContainer_SOR;

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202604 sic@

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); //@sic R5s200739 sic@

var O2_Type v_AccessTechId :='0800'O; //@sic R5s200739 sic@

var O2_Type v_Counter :='0001'O;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2" siclog@

//whole registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13 with"connected without release"

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell13, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell13, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s200739 sic@

//@siclog "Step 3" siclog@

//The SS transmits an DLInformationTransfer message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

}

After change:

function fl_TC_6_3_1_8_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var octetstring  v_NAS_TxMsgContainer_SOR;

var TypeOfRegistration_Type v_TypeOfRegistration;//@sic R5-202604 sic@

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); //@sic R5s200739 sic@

var O2_Type v_AccessTechId :='0800'O; //@sic R5s200739 sic@

var O2_Type v_Counter :='0001'O;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

timer t_Tmax;

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2" siclog@

//whole registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 13 with"connected without release"

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell13, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell13, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s200739 sic@

//@siclog "Step 3" siclog@

//The SS transmits an DLInformationTransfer message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

}

## 2.6 Change 6

| Function name | fl_TC_6_3_1_9_TestBody |
| --- | --- |
| Reason for change | Support the handling of Registration_Request in cases where UE retains and deletes NAS Security Context. |
| Summary of change | Modify call to function f_NR5GC_SorRegisration to support handling of Registration_Request with and without Integrity Protection. |
| TTCN module | SOR_NR5GC |
| MCC160 Comment | Accepted. |

Before change:

function fl_TC_6_3_1_9_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring  v_NAS_TxMsgContainer_SOR;

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); // @sic R5s200745 sic@

var O2_Type v_AccessTechId :='0800'O;// @sic R5s200745 sic@

var O2_Type v_Counter :='0001'O;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2" siclog@

//PLMN3 is manually selected

f_UT_ManualPLMN_Select(UT,

f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell12)));//@sic R5s200682 sic@

//@siclog "Steps 3" siclog@

//whole registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 12 with"connected without release"

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, tsc_SHT_NoSecurityProtection);

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s200745 sic@

//@siclog "Step 4" siclog@

//The SS transmits an DLInformationTransfer message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

}

After change:

function fl_TC_6_3_1_9_TestBody() runs on NR5GC_PTC

{

var GMM_MobilityInfo_Type v_GMM_MobilityInfo;

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var octetstring  v_NAS_TxMsgContainer_SOR;

var template (value) NG_NAS_DL_Pdu_Type v_PduToSend;

var O16_Type v_Mac;

var B128_Type v_Mac_A17;

var NG_NAS_SecurityParams_Type v_NAS_SecurityParams;

var octetstring v_Data;

var NAS_PlmnId v_PlmnID := f_NR_Asn2Nas_PlmnId(f_NR_CellInfo_GetPLMN (nr_Cell11)); // @sic R5s200745 sic@

var O2_Type v_AccessTechId :='0800'O;// @sic R5s200745 sic@

var O2_Type v_Counter :='0001'O;

var template (value) SOR_Header v_Header := crs_NG_SOR_Header('1'B,'1'B,'1'B,'0'B);

//@siclog "Step 1" siclog@

//Power on the UE

f_UT_SwitchOnUE(UT);

//@siclog "Steps 2" siclog@

//PLMN3 is manually selected

f_UT_ManualPLMN_Select(UT,

f_NR_Asn2Nas_PlmnId (f_NR_CellInfo_GetPLMN (nr_Cell12)));//@sic R5s200682 sic@

//@siclog "Steps 3" siclog@

//whole registration procedure described in TS 38.508 subclause 4.5.2.2-2 are performed on NR Cell 12 with"connected without release"

v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell12, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected));

if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity)) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed");

}

f_NR5GC_RRC_Idle_Steps5_20(v_GMM_MobilityInfo, nr_Cell12, v_ReceivedMsg, TESTMode_OFF, noRrcConnectionRelease, Initial_NoSecurity); // @sic R5s200745 sic@

//@siclog "Step 4" siclog@

//The SS transmits an DLInformationTransfer message containing

//steering of roaming information indicating list of preferred PLMN/access technology combination provided with acknowledgment requested from the UE for successful reception

}