3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210660

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1768 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR5GC test case 11.1.5 in FR1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Datang LinkTester |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-22 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add Rel-15 NR5GC test case 11.1.5 to the NR5GC ATS |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to NR5GC test case 11.1.5 required for approval. See detailed change description for further information. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.5.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	7

2.3	Change 3	10

2.4	Change 4	11

2.5	Change 5	13

2.6	Change 6	16

3	Branches executed	16

4	Execution Log Files	16

4.1	HiSilicon Balong 5000	16

5	References	17

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 11.1.5 which is part of the NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Pengchong Shen
	shenpengchong@datangmobile.cn

## Verification Test Summary

Test Case: 	11.1.5

ATS Version:	iwd-TTCN3-B2020-09_D21wk12

System Simulator used:	Datang LinkTester 5G Protocol Conformance Test System ECT9610

UE used:	HiSilicon Balong 5000

Verification Status:	PASS

# Corrections required

## Change 1

| Function name | f_TC_11_1_5_NR5GC |
| --- | --- |
| Reason for change | - NR side should to use f_NR5GC_IRAT_UECapabilityTransfer to complete LTE registration process at preamble. - It should to set cell power on and UE power on command before NR registration. |
| Summary of change | - Add f_NR5GC_IRAT_UECapabilityTransfer after NR cell setup. - Instead NR preamble to f_NR5GC_Preamble_Steps1_13. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function f_TC_11_1_5_NR5GC() runs on NR5GC_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NwkSupport := cds_NG_NetworkFeatureSupport_NoN26; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR // Steps 2 - 4 // RRC Connection Setup v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, ?, (tsc_SHT_NoSecurityProtection, tsc_SHT_IntegrityProtected)); if (not f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Initial_NoSecurity, -, -, -, -, cdr_NG_GMM_Cap_S1)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Registration Request Message Failed"); } v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NwkSupport.emc := '00'B; // prose hardcodes these to different values from the default // Steps 5 - 13 f_NR5GC_RRC_Idle_Steps5_13 (v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Initial_NoSecurity); // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, v_NwkSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

After Change

function f_TC_11_1_5_NR5GC() runs on NR5GC_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var GMM_MobilityInfo_Type v_GMM_MobilityInfo; var template (value) NG_NAS_DL_Message_Type v_MsgToSend; var PLMN_Identity v_ASN_PLMN; var NAS_PlmnId v_PLMN; var NG_NAS_GutiParameters_Type v_GutiParams; var template (value) NG_MobileIdentity v_GUTIToSend; var TrackingAreaCode v_TAC; var template (value) NG_TrackingAreaIdList v_TAIList; var template (value) NG_NetworkFeatureSupport v_NwkSupport := cds_NG_NetworkFeatureSupport_NoN26; f_NR5GC_IRAT_Init(NR_6); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); // Wait until UE has attached on EUTRA f_EUTRA_NR_WaitForCoOrd_Trigger (EUTRA, "Preamble"); // Now register on NR v_GMM_MobilityInfo := f_NR5GC_Preamble_Steps1_13 (nr_Cell1); v_ASN_PLMN := f_NR_CellInfo_GetPLMN (nr_Cell1); v_PLMN := f_NR_Asn2Nas_PlmnId(v_ASN_PLMN); v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1); v_GUTIToSend := f_NG_GutiParameters2MobileIdentity (v_PLMN, v_GutiParams); v_TAC := f_NR_CellInfo_GetTAC (nr_Cell1); v_TAIList := cs_NG_TAIListNonConsecutive(v_PLMN, {bit2oct(v_TAC)}); v_NwkSupport.emc := '00'B; // prose hardcodes these to different values from the default // Step 14 // Registration Accept v_MsgToSend := f_Get_NG_RegistrationAcceptMsg (Initial_Secure, v_GMM_MobilityInfo, v_GUTIToSend, v_TAIList, omit, -, -, -, v_NwkSupport); SRB.send(cas_NR_SRB_NasPdu_REQ(nr_Cell1, tsc_NR_RbId_SRB1, -, cs_NG_NAS_Request(tsc_SHT_IntegrityProtected_Ciphered, v_MsgToSend))); // Steps 15 - 20 f_NR5GC_RRC_Idle_Steps15_20(nr_Cell1, TESTMode_OFF); // Now tell EUTRA preamble finished f_EUTRA_NR_SendCoOrd (EUTRA, cms_EUTRA_NR_Trigger("Preamble")); f_NR_TestBody_Set(true); fl_TC_11_1_5_NR5GC_TestBody(); f_NR_TestBody_Set(false); // UE now on EUTRA f_NR_ReleaseAllCells(); }

## Change 2

| Function name | fl_TC_11_1_5_NR5GC_TestBody |
| --- | --- |
| Reason for change | - The cause of service triggered by pagging should no be mo-data. - In order to cooperate with the fl_TAUReject_AttachWithoutN26 function on the LTE side, only GutiParameters, PduSessionInfo and UECapability should be sent. - Modify with change point 4.3. |
| Summary of change | - The second and third parameters of f_NR5GC_RRCIdleToConnected_Steps2_4 uses the default value. - Use f_NR5GC_Send_GutiParametersToEUTRA, f_NR5GC_SendPduSessionInfoListToEUTRA and f_NR5GC_IRAT_UECapabilityTransfer instead of f_NR5GC_TrackingAreaUpdateFromN1. - Add v_CoOrd_SysInfo_cell1 to receive Arfcn of eutra_cell1. |
| TTCN module | EPS_Fallback_NR5GC_NR.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function fl_TC_11_1_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1, '0001'B, mo_Data); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 11" siclog@ //Step 1 of TS 34.229-5 Annex A.9.1, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_TrackingAreaUpdateFromN1(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

After Change

function fl_TC_11_1_5_NR5GC_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var template (value) RedirectedCarrierInfo v_RedirectedCarrierInfo; var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); var EUTRA_NR_CoOrd_SysInfo_Type v_CoOrd_SysInfo_cell1 := f_EUTRA_NR_WaitForCoOrd_SysInfo(EUTRA); // @siclog "Step 1" siclog@ f_NR_SetCellPower (nr_Cell1, tsc_NR_ServingCellSSS_EPRE_FR1, tsc_NR_ServingCellSSS_EPRE_FR2); //@sic R5-210133 sic@ //@siclog "Step 2" siclog@ f_NR_Paging_Def(nr_Cell1, RRC_IDLE); //@siclog "Steps 3 - 9" siclog@ v_ReceivedMsg := f_NR5GC_RRCIdleToConnected_Steps2_4(nr_Cell1); f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); //@siclog "Step 10" siclog@ f_UT_RequestIMSCall(UT, px_IMS_CalleeUri); //@siclog "Step 11" siclog@ //Step 1 of TS 34.229-5 Annex A.9.1, done on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); //@siclog "Step 13" siclog@ //The SS transmits an RRCRelease message (IE redirectedCarrierInfo including EUTRA cells). v_RedirectedCarrierInfo := cs_NR_RedirectedCarrierInfoToEUTRA(v_CoOrd_SysInfo.Eutra[0].Arfcn); SRB.send(cas_NR_SRB1_RrcPdu_REQ(nr_Cell1, -, cs_38508_RRCRelease(tsc_NR_RRC_TI_Def, -, v_RedirectedCarrierInfo))); f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1)); // Send both authentication and NR sysinfo paramters to EUTRA f_NR5GC_Send_GutiParametersToEUTRA(nr_Cell1); // send details of existing PDU sessions f_NR5GC_SendPduSessionInfoListToEUTRA(); f_NR5GC_IRAT_UECapabilityTransfer(nr_Cell1); f_EUTRA_NR_WaitForCoOrd_Trigger(EUTRA, "Postamble"); }

## Change 3

| Function name | f_TC_11_1_5_NR5GC_EUTRA |
| --- | --- |
| Reason for change | - The MaxReferencePower of cell 3 should be set to -73. - Acoording to R5-213496, Cell2 need to be replaced by cell3. |
| Summary of change | - Add f_EUTRA_CellInfo_InitMaxReferencePower. - Instead eutra_Cell2 to eutra_Cell3. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function f_TC_11_1_5_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ////MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success f_EUTRA38_IRAT_Init(c31); //Create Eutra Cell 2 and set it as reserved for operator use and modify Sysinfo updated f_EUTRA_CellInfo_SetSysInfo_CellReservedForOpUse (eutra_Cell2, reserved); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell2); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_5_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

After Change

function f_TC_11_1_5_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { ////MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode without N26 interface / E-UTRAN cell reselection using cell status reservation / Success f_EUTRA38_IRAT_Init(c31); //Create Eutra Cell 2 and set it as reserved for operator use and modify Sysinfo updated f_EUTRA_CellInfo_SetSysInfo_CellReservedForOpUse (eutra_Cell3, reserved); f_EUTRA_CellInfo_InitMaxReferencePower (eutra_Cell3, -73); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); f_EUTRA_TestBody_Set(true); fl_TC_11_1_5_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); }

## Change 4

| Function name | fl_TC_11_1_5_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | - Modify with change point 3.2.. - According to 38.523-1 table 11.1.5 3.2-2. It should to add PDN CONNECTIVITY REQUEST process. - Since the arfcn of eutra_cell3 should be carried in the redirection message, the step of sending the arfcn of eutra_cell3 to NR side is added. |
| Summary of change | - Instead eutra_Cell2 to eutra_Cell3. - Add PDN CONNECTIVITY REQUEST process after step 27. - Add f_EUTRA_NR_SendSysinfoCoOrd to send arfcn of cell3. |
| TTCN module | EPS_Fallback_NR5GC_EUTRA.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell2, tsc_CellRS_EPRE_73, tsc_CellRS_EPRE_73); // @sic R5-210133 sic@ // @siclog "Step 14 - 27" siclog@ fl_TAUReject_AttachWithoutN26 (); // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

After Change

function fl_TC_11_1_5_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var EPS_BearerIdentity v_EpsBearerId; var SRB_COMMON_IND v_ReceivedAsp; var timer t_wait := 5; // @siclog "Step 0" siclog@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_CellRS_EPRE_73, tsc_CellRS_EPRE_73); // @sic R5-210133 sic@ f_EUTRA_NR_SendSysinfoCoOrd(NR, f_EUTRA38_IRAT_ListOfFrequencies({eutra_Cell3})); // @siclog "Step 14 - 27" siclog@ fl_TAUReject_AttachWithoutN26 (); alt { [] SRB.receive(car_SRB2_NasPdu_IND(eutra_Cell1, cr_NAS_Indication(tsc_SHT_IntegrityProtected_Ciphered, cr_PDN_CONNECTIVITY_REQUEST_N1ToS1(tsc_PdnRequest_Handover, ?)))) -> value v_ReceivedAsp { v_EpsBearerId := f_EUTRA38_MultiPDN_EstablishAdditionalPDN_Step2 (eutra_Cell1, v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg.pDN_CONNECTIVITY_REQUEST); a_EUTRA_IdleUpdated_ConnectToAdditionalPDN_Step3(eutra_Cell1); a_EUTRA38_MultiPDN_ConnectToAdditionalPDN_Step4(eutra_Cell1, v_EpsBearerId); } [] t_wait.timeout { } } // @siclog "Step 30 -32" siclog@ f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA38_MultiPDN_ActivateDedicatedEpsBearer_Speech(eutra_Cell1); // Test body finishes on IMS PTC f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS1]); f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); }

## Change 5

| Function name | f_TC_11_1_5_NR5GC_IMS1 |
| --- | --- |
| Reason for change | - After INVITE, it should to send CoOrdMsg trigger RRCRelease message. - Missing Response183 sending steps. - The IMS Deregistration process is happen on the LTE side. |
| Summary of change | - Add f_IMS_IPCAN_SendCoOrdMsg after step 11. - Add f_IMS_MOCallSetup_AnnexC21_Step4 in step 28-29. - Set the first parameter to OtherIPCAN |
| TTCN module | EPS_Fallback_NR5GC_IMS.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function f_TC_11_1_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader_Response183; var default v_MyDefaultVar := null; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 11" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(-, -); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); //@siclog "Step 28 - 29" siclog@ v_MyDefaultVar := activate (a_C46_ParallelBehavior ()); //Steps 28 - 29 Map to 36.508 Table 4.5A.6.3-1 Steps 9a1- 11a1 which maps to 34.229-1 C.21 Steps 3 -4 f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); v_MessageHeader_Response183 := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri, omit, -, NORMAL_CALL); deactivate (v_MyDefaultVar); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); //Carry on with the C.21 Steps 5-13 f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, valueof(v_MessageHeader_Response183)); // @siclog "Step 36" siclog@ f_Delay(1.0); // @siclog "Step 37" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration(); }

After Change

function f_TC_11_1_5_NR5GC_IMS1()runs on IMS_PTC { var IMS_IPCAN_Coordination_MSG v_IMS_IPCAN_CoordMsg; var IMS_InviteRequestWithSdp_Type v_InviteRequestWithSdp; var template (value) MessageHeader v_MessageHeader_Response183; var default v_MyDefaultVar := null; fl_IMSFallback_PreambleRegistrations(); // @siclog "Step 11" siclog@ v_InviteRequestWithSdp := f_IMS_MOCallSetup_ReceiveINVITE_Common(-, -); f_IMS_IPCAN_SendCoOrdMsg(IPCAN); // Wait for indication RAT has changed to EUTRA OtherIPCAN.receive(cmr_IPCAN_IMS_Response) -> value v_IMS_IPCAN_CoordMsg; f_IMS_PTC_SetRanType(v_IMS_IPCAN_CoordMsg.IPCAN_IMS_Response.IpcanInfo.RanType); //@siclog "Step 28 - 29" siclog@ v_MyDefaultVar := activate (a_C46_ParallelBehavior ()); //Steps 28 - 29 Map to 36.508 Table 4.5A.6.3-1 Steps 9a1- 11a1 which maps to 34.229-1 C.21 Steps 3 -4 f_IMS_MOCallSetup_Send100Trying(v_InviteRequestWithSdp); v_MessageHeader_Response183 := f_IMS_InviteResponse_183_MessageHeaderTX(v_InviteRequestWithSdp.Invite, px_IMS_CalleeContactUri, omit, -, NORMAL_CALL); f_IMS_MOCallSetup_AnnexC21_Step4(v_InviteRequestWithSdp, v_MessageHeader_Response183); deactivate (v_MyDefaultVar); // Trigger EUTRA for Dedicated Bearer Setup f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); //Carry on with the C.21 Steps 5-13 f_IMS_MOCallSetup_AnnexC21_Steps5_13(v_InviteRequestWithSdp, valueof(v_MessageHeader_Response183)); // @siclog "Step 36" siclog@ f_Delay(1.0); // @siclog "Step 37" siclog@ f_IMS_CallReleaseMO(v_InviteRequestWithSdp.Invite); f_IMS_IPCAN_SendCoOrdMsg(OtherIPCAN); f_IMS_IpcanReleaseWithOptionalDeregistration (OtherIPCAN); }

## Change 6

| Function name | f_IMS_IpcanReleaseWithOptionalDeregistration |
| --- | --- |
| Reason for change | - Modify with change point 5. |
| Summary of change | - Add the parameter p_Port. |
| TTCN module | IMS_Procedures_Registration.ttcn |
| MCC160 Comment | See section 4.1. |

Before Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(boolean p_AllowRetransmission := false) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(IPCAN, t_Dummy, -, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

After Change

function f_IMS_IpcanReleaseWithOptionalDeregistration(IMS_IPCAN_CO_ORD_PORT p_Port, boolean p_AllowRetransmission := false) runs on IMS_PTC { // @sic R5s170198 sic@ timer t_Dummy; f_IMS_OptionalDeregistrationOrC24Reinvite(p_Port, t_Dummy, -, p_AllowRetransmission); // @sic R5-154047, R5s170028, R5s170198, R5s170541 sic@ }

# Branches executed

NR Band 41, nea2, Nia2.

# Execution Log Files

## HiSilicon Balong 5000

The HiSilicon Balong 5000 passed this test case on Datang LinkTester 5G Protocol Conformance Test System ECT9610. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test Case Execution log file:
\ TC_11_1_5_NR5GC_N41-FDD3.log

- PIXCIT settings used:
\ Pics(Hisilicon-IRAT).xml

\ Pixit(TC_11_1_5_NR5GC).xml

| MCC160 Comment | Rejected. During SS log check, it was discovered that several TTCN changes were made but not listed in section 2. |
| --- | --- |

# References

| [1] | R5s210661: Supporting information for agreement of NR5GC test case 11.1.5 in FR1 |
| --- | --- |