3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210671

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1773 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to function fl_NR5GC_QoSFlowsLength |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the function f_NR5GC_ModifyPDUSession_RemoveIMSVoice parameter cs_QoSFlowDescr({cs_QoS_DeleteFlow('000111'B)}) is passed to function f_Get_NG_PDUSessionModificationCommand. This results in the function fl_NR5GC_QoSFlowsLength being called, and the parameter being passed is {cs_QoS_DeleteFlow}. Within cs_QoS_DeleteFlow the IE parameterlist is set to OMIT. This causes a runtime error when the check isvalue(p_FlowList[j].parameterList) is performed within fl_NR5GC_QoSFlowsLength. This needs to be addressed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added isvalue check on parameterList content before applying lengthof operation. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case(s) may end with a runttime error. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | NR IMS Voice call TCs. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	4

1.1	Correction to function fl_NR5GC_QoSFlowsLength	4

# Corrections required

## Correction to function fl_NR5GC_QoSFlowsLength

| Function name | fl_NR5GC_QoSFlowsLength |
| --- | --- |
| Reason for change | In the function f_NR5GC_ModifyPDUSession_RemoveIMSVoice parameter cs_QoSFlowDescr({cs_QoS_DeleteFlow('000111'B)}) is passed to function f_Get_NG_PDUSessionModificationCommand. This results in the function fl_NR5GC_QoSFlowsLength being called, and the parameter being passed is {cs_QoS_DeleteFlow}. Within cs_QoS_DeleteFlow the IE parameterlist is set to OMIT. This causes a runtime error when the check isvalue(p_FlowList[j].parameterList) is performed within fl_NR5GC_QoSFlowsLength. This needs to be addressed. |
| Summary of change | Added isvalue check on parameterList content before applying lengthof operation. |
| TTCN module | Common\NG_NAS\NG_NAS_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change:

<<SKIPPED CODE>> function fl_NR5GC_QoSFlowsLength (template (value) QoS_FlowList p_FlowList) return Type6Length_Type { var integer i; var integer j; var integer v_Length:=0; for (j:= 0; j < lengthof(p_FlowList); j:= j+1) { for (i:= 0; i < lengthof(p_FlowList[j].parameterList); i:= i+1) { v_Length := v_Length + oct2int(valueof(p_FlowList[j].parameterList[i].lenOfContent)); v_Length := v_Length + 2; // Parameter identifier & length @sic R5s190856 sic@ } v_Length := v_Length + 3; // QFI + Operation Code + Num of Parameters } return int2oct(v_Length, 2); } <<SKIPPED CODE>>

After Change:

<<SKIPPED CODE>> function fl_NR5GC_QoSFlowsLength (template (value) QoS_FlowList p_FlowList) return Type6Length_Type { var integer i; var integer j; var integer v_Length:=0; for (j:= 0; j < lengthof(p_FlowList); j:= j+1) { if(isvalue(p_FlowList[j].parameterList) ){ for (i:= 0; i < lengthof(p_FlowList[j].parameterList); i:= i+1) { v_Length := v_Length + oct2int(valueof(p_FlowList[j].parameterList[i].lenOfContent)); v_Length := v_Length + 2; // Parameter identifier & length @sic R5s190856 sic@ } } v_Length := v_Length + 3; // QFI + Operation Code + Num of Parameters } return int2oct(v_Length, 2); } <<SKIPPED CODE>>