3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210673

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1775 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR test cases 7.1.3.5.1 and 7.1.3.5.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current TTCN implementation - For 7.1.3.5.1 and 7.1.3.5.4 , the usage of NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot - For 7.1.3.5.1 at step 7 of the test case , p_NR_PDCP_State.RX_NEXT is set wrongly - For 7.1.3.5.4 , at step 14-15, for the PDU List containing multiple PDU in the same slot , the expected combination is populated wrongly |  |  |  |  |  |  |  |  |
| Summary of change: |  | - For the test cases 7.1.3.5.1 and 7.1.3.5.4 , it is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs - At step 7 of the test case 7.1.3.5.1 , p_NR_PDCP_State.RX_NEXT has to be set with p_NR_PDCP_State.TX_NEXT – 2 instead of having “0” value - At step 14-15 of the test case 7.1.3.5.4 , the expected combination for multiple PDUs in the same slot should be set with {v_ExpectedPdu1,v_ExpectedPdu2} |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.5.1.ENDC, 7.1.3.5.1.NR5GC, 7.1.3.5.4.ENDC and 7.1.3.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	3

1	Corrections required	3

1.1	Correction to function f_TC_7_1_3_5_1_NR_TestBody	3

1.2	Correction to function f_TC_7_1_3_5_4_NR_Step1to17	7

# Corrections required

## Correction to function f_TC_7_1_3_5_1_NR_TestBody

| Function name | f_TC_7_1_3_5_1_NR_TestBody |
| --- | --- |
| Reason for change | - Usage of NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot - At step 7 of the test case , p_NR_PDCP_State.RX_NEXT is set wrongly |
| Summary of change | - It is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs - At step 7 of the test case 7.1.3.5.1 , p_NR_PDCP_State.RX_NEXT has to be set with p_NR_PDCP_State.TX_NEXT – 2 instead of having “0” value |
| TTCN module | NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_3_5_1_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfok; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var float v_T_reordering; var integer k; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Stop UL Grants f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 1" siclog@ All PDCP PDUs use same SDU v_IPData v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 78, 94, 110}); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); //@siclog "Step 3" siclog@ for (k:= 0; k <= 2; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[k], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 20*(k+2)); } //@siclog "Step 4" siclog@ Wait Discard timer to expire. @sic R5s190225, R5s190770 sic@ v_T_reordering := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, 500.0); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfok, float2int(v_T_reordering) - 20); //@siclog "Step 5" SS transmits a PDCP Data PDU containing one PDCP SDU @sic R5s190225 sic@ v_TimingInfok := v_TimingInfo2; for (k:= 0; k <= 1; k:= k+1) { f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[k+3], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,20*(k+1)); } v_TimingPreviousSlot:= f_NR_GetCurrentTiming(nr_Cell1); //@sic R5-213462 sic@ //@siclog "Step 6" The SS resumes normal UL grant allocation. v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfok, 0); // @sic R5s190590 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL)); //@siclog "Step 7" siclog@ Does UE transmit a PDCP Data PDU # 5 of size 94 bytes? (Note 2) //@sic R5-213462 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= (v_ExpectedPdu1,v_ExpectedPdu2); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } } }

After Change:

function f_TC_7_1_3_5_1_NR_TestBody(NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfok; var SubFrameTiming_Type v_TimingUL; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var NR_PDCP_SDUList_Type v_SduList; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var float v_T_reordering; var integer k; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc; //Stop UL Grants f_NR_ULGrantConfiguration_Stop(nr_Cell1, -, cs_NR_UplinkTimeAlignment_Keep); //@siclog "Step 1" siclog@ All PDCP PDUs use same SDU v_IPData v_SduList := f_GenerateSDU (p_NR_PDCP_State, {46, 62, 78, 94, 110}); v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); //@siclog "Step 3" siclog@ for (k:= 0; k <= 2; k:= k+1) { //SS transmits a PDCP Data PDU containing one PDCP SDU f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[k], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1, 20*(k+2)); } //@siclog "Step 4" siclog@ Wait Discard timer to expire. @sic R5s190225, R5s190770 sic@ v_T_reordering := f_NR_SetTimerToleranceMax(nr_Cell1,l2Timer, 500.0); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfok, float2int(v_T_reordering) - 20); //@siclog "Step 5" SS transmits a PDCP Data PDU containing one PDCP SDU @sic R5s190225 sic@ v_TimingInfok := v_TimingInfo2; for (k:= 0; k <= 1; k:= k+1) { f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1, v_SduList[k+3], v_TimingInfok); v_TimingInfok := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,20*(k+1)); } v_TimingPreviousSlot:= f_NR_GetCurrentTiming(nr_Cell1); //@sic R5-213462 sic@ //@siclog "Step 6" The SS resumes normal UL grant allocation. v_TimingUL := f_NR_GetNextSendOccasion_ULafterDL(nr_Cell1, v_TimingInfok, 0); // @sic R5s190590 sic@ f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_NR(v_TimingUL)); //@siclog "Step 7" siclog@ Does UE transmit a PDCP Data PDU # 5 of size 94 bytes? (Note 2) //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT := p_NR_PDCP_State.TX_NEXT - 2; v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {v_ExpectedPdu1,v_ExpectedPdu2}; alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 7-8"); } } }

## Correction to function f_TC_7_1_3_5_4_NR_Step1to17

| Function name | f_TC_7_1_3_5_4_NR_Step1to17 |
| --- | --- |
| Reason for change | In the current TTCN implementation - Usage of NR_PDCP_PDUList_Type for the single PDU is leading to mismatch while receiving the PDCP PDU List containing multiple PDUs coming in the same slot - At step 14-15, for the PDU List containing multiple PDU in the same slot , the expected combination is populated wrongly |
| Summary of change | - It is proposed to use the data type NR_PDCP_PDU_Type for the expected individual PDUs - At step 14-15 of the test case 7.1.3.5.4 , the expected combination for multiple PDUs in the same slot should be set with {v_ExpectedPdu1,v_ExpectedPdu2} |
| TTCN module | \ttcn\develop\NR_TC_Common\7_1_3\PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted |

Before Change:

function f_TC_7_1_3_5_4_NR_Step1to17(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_SubFrameTimingT1; var SubFrameTiming_Type v_SubFrameTimingT2; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var float v_T_reordering; var TimingInfo_Type v_TimingInfo; var integer v_Duration; timer t_Wait := 1.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc ; //@siclog "Step 1" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU #131072 p_NR_PDCP_State.TX_NEXT := 131072; //@sic R5s190227 sic@ f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData); //@siclog "Step 2" siclog@ //Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_Wait.timeout{} } //@siclog "Step 3" siclog@ //The SS sends the PDCP SDU #1 p_NR_PDCP_State.TX_NEXT := 1; v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,120); f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo1); //@siclog "Step 4" siclog@ //The SS sends the PDCP SDU #2 f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo2); //@siclog "Step 5" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo3 - v_TimingInfo2 //@siclog "Step 6" siclog@ The SS sends the PDCP SDU #0 p_NR_PDCP_State.TX_NEXT := 0; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo3); //@sic R5-213462 sic@ //@siclog "Step 7-9" siclog@ //UE transmits the PDCP SDU #0 via the AM RLC entity #0,#1,#2 (Note 4). @sic R5s190227 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= (v_ExpectedPdu1, v_ExpectedPdu2); v_ExpectedPdu5:= (v_ExpectedPdu2, v_ExpectedPdu3); v_ExpectedPdu6:= (v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } //@siclog "Step 10" siclog@ //The SS sends the PDCP SDU #4 v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); //t-Reordering starts v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,100); //Next PDU v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,120); //Wait for PDU v_SubFrameTimingT1 := v_TimingInfo3; p_NR_PDCP_State.TX_NEXT := 4; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo1); //@siclog "Step 11" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo2 - v_TimingInfo1 //@siclog "Step 12" siclog@ The SS sends the PDCP SDU #7 p_NR_PDCP_State.TX_NEXT := 7; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[0],v_TimingInfo2); //@siclog "Step 13" siclog@ The SS sends the PDCP SDU #3 p_NR_PDCP_State.TX_NEXT := 3; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo3); //@siclog "Step 14-15" siclog@ UE transmits a PDCP SDU #3, #4 (Note 5). @sic R5s190227 sic@ //@sic R5-213462 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= (v_ExpectedPdu5,v_ExpectedPdu6); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } } //@siclog "Step 16" siclog@ UE transmits a PDCP SDU #7 p_NR_PDCP_State.RX_NEXT := 5; //@sic R5-194837 sic@ v_TimingInfo := f_NR_PDCP_Recv(p_NR_PDCP_State,nr_Cell1,v_SduList[0], cr_RlcBearerRouting_NR(nr_Cell1)); v_SubFrameTimingT2 := v_TimingInfo.SubFrame; // Check 1: Is (T2 - T1) > t-Reordering? v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingT1, v_SubFrameTimingT2); v_T_reordering := f_NR_SetTimerToleranceMin(nr_Cell1,l2Timer,300.0); if (v_Duration > float2int(v_T_reordering)){ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } //@siclog "Step 17" siclog@ The SS sends the PDCP SDU #9 p_NR_PDCP_State.TX_NEXT := 9; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[1]); }

After Change:

function f_TC_7_1_3_5_4_NR_Step1to17(inout NR_PDCP_SS_State_Type p_NR_PDCP_State) runs on NR_BASE_PTC { var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE()); var NR_PDCP_SDUList_Type v_SduList := f_GenerateSDU (p_NR_PDCP_State, {64, 164}); var NR_SecurityParams_Type v_NR_Security:= f_NR_Security_Get(); var SubFrameTiming_Type v_TimingInfo1; var SubFrameTiming_Type v_TimingInfo2; var SubFrameTiming_Type v_TimingInfo3; var SubFrameTiming_Type v_SubFrameTimingT1; var SubFrameTiming_Type v_SubFrameTimingT2; var SubFrameTiming_Type v_TimingCurrentSlot; var SubFrameTiming_Type v_TimingPreviousSlot; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; var NR_DRB_COMMON_IND v_ReceivedPdu1; var NR_DRB_COMMON_IND v_ReceivedPdu2; var NR_DRB_COMMON_IND v_ReceivedPdu3; var NR_DRB_COMMON_IND v_ReceivedPdu4; var NR_DRB_COMMON_IND v_ReceivedPdu5; var NR_DRB_COMMON_IND v_ReceivedPdu6; var float v_T_reordering; var TimingInfo_Type v_TimingInfo; var integer v_Duration; timer t_Wait := 1.0; p_NR_PDCP_State.KUPenc := v_NR_Security.AS_Ciphering.KUPenc ; //@siclog "Step 1" siclog@ //SS transmits a PDCP Data PDU containing one PDCP SDU #131072 p_NR_PDCP_State.TX_NEXT := 131072; //@sic R5s190227 sic@ f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData); //@siclog "Step 2" siclog@ //Does the UE transmit a PDCP SDU via the AM RLC entity in the next 1s? t_Wait.start; alt { [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList(nr_Cell1, p_NR_PDCP_State.DRB_Id)) { t_Wait.stop; f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Step 4"); } [] t_Wait.timeout{} } //@siclog "Step 3" siclog@ //The SS sends the PDCP SDU #1 p_NR_PDCP_State.TX_NEXT := 1; v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,20); v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo2,120); f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo1); //@siclog "Step 4" siclog@ //The SS sends the PDCP SDU #2 f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo2); //@siclog "Step 5" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo3 - v_TimingInfo2 //@siclog "Step 6" siclog@ The SS sends the PDCP SDU #0 p_NR_PDCP_State.TX_NEXT := 0; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo3); //@sic R5-213462 sic@ //@siclog "Step 7-9" siclog@ //UE transmits the PDCP SDU #0 via the AM RLC entity #0,#1,#2 (Note 4). @sic R5s190227 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= {v_ExpectedPdu1, v_ExpectedPdu2}; v_ExpectedPdu5:= {v_ExpectedPdu2, v_ExpectedPdu3}; v_ExpectedPdu6:= {v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3}; alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } // receive PDU1, PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 7-9"); } } //@siclog "Step 10" siclog@ //The SS sends the PDCP SDU #4 v_TimingInfo1 := f_NR_GetNextSendOccasion_DL(nr_Cell1); //t-Reordering starts v_TimingInfo2 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,100); //Next PDU v_TimingInfo3 := f_SubFrameTiming_AddMilliSeconds(v_TimingInfo1,120); //Wait for PDU v_SubFrameTimingT1 := v_TimingInfo3; p_NR_PDCP_State.TX_NEXT := 4; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo1); //@siclog "Step 11" siclog@ Wait for 100ms (< configured t-Reordering) //v_TimingInfo2 - v_TimingInfo1 //@siclog "Step 12" siclog@ The SS sends the PDCP SDU #7 p_NR_PDCP_State.TX_NEXT := 7; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[0],v_TimingInfo2); //@siclog "Step 13" siclog@ The SS sends the PDCP SDU #3 p_NR_PDCP_State.TX_NEXT := 3; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_IPData,v_TimingInfo3); //@siclog "Step 14-15" siclog@ UE transmits a PDCP SDU #3, #4 (Note 5). @sic R5s190227 sic@ //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= {v_ExpectedPdu1,v_ExpectedPdu2}; alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4))-> value v_ReceivedPdu3 { f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 14-15"); } } //@siclog "Step 16" siclog@ UE transmits a PDCP SDU #7 p_NR_PDCP_State.RX_NEXT := 5; //@sic R5-194837 sic@ v_TimingInfo := f_NR_PDCP_Recv(p_NR_PDCP_State,nr_Cell1,v_SduList[0], cr_RlcBearerRouting_NR(nr_Cell1)); v_SubFrameTimingT2 := v_TimingInfo.SubFrame; // Check 1: Is (T2 - T1) > t-Reordering? v_Duration := f_SubFrameTiming_Duration(v_SubFrameTimingT1, v_SubFrameTimingT2); v_T_reordering := f_NR_SetTimerToleranceMin(nr_Cell1,l2Timer,300.0); if (v_Duration > float2int(v_T_reordering)){ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 16"); } //@siclog "Step 17" siclog@ The SS sends the PDCP SDU #9 p_NR_PDCP_State.TX_NEXT := 9; f_NR_PDCP_Send(p_NR_PDCP_State,nr_Cell1,v_SduList[1]); }