3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210682

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1779 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR idle mode test case 6.1.2.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | After the implementation of R5s210524, it is observed that configuration and activation of VNG is done for NR CELL11, when NR CELL11 is off (Power level setting v_CellPowerList_AtT1 is in effect) . This results in a situation when the noise is being generated in a cell that is OFF. Any cell that is set to power level OFF is basically just noise, and applying NoC for such a cell does not make sense. Prose CR R5-206318 was raised for a similar issue and was agreed. It is proposed to move the configuration and activation of VNG for cell11 once the power level is applied as per v_CellPowerList_AtT2 in step 4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Moved the configuration and activation of VNG after applying v_CellPowerList_AtT2 which powers up Nr cell 11 in step4 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE may fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Corrections required	2

1.1	Correction to function f_TC_6_1_2_2_NR5GC_TestBody	2

# Corrections required

## Correction to function f_TC_6_1_2_2_NR5GC_TestBody

| Function name | f_TC_6_1_2_2_NR5GC_TestBody |
| --- | --- |
| Reason for change | After the implementation of R5s210524, it is observed that configuration and activation of VNG is done for NR CELL11, when NR CELL11 is off (Power level setting v_CellPowerList_AtT1 is in effect) . This results in a situation when the noise is being generated in a cell that is OFF. Any cell that is set to power level OFF is basically just noise, and applying NoC for such a cell does not make sense. Prose CR R5-206318 was raised for a similar issue and was agreed. It is proposed to move the configuration and activation of VNG for cell11 once the power level is applied as per v_CellPowerList_AtT2 in step 4. |
| Summary of change | Moved the configuration and activation of VNG after applying v_CellPowerList_AtT2 which powers up Nr cell 11 in step4 |
| TTCN module | Idle_CellSelection_NR5GC.ttcn |
| MCC160 Comment | Rejected, not needed following R5-216163 agreed at RAN5#92-e |

Before Change:

function f_TC_6_1_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-206318 sic@ var integer v_T1_T3_Cell1_RS_EPRE_FR1 := -91; var integer v_T1_T3_Cell1_RS_EPRE_FR2 := -91; var integer v_T1_Cell11_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T1_Cell11_RS_EPRE_FR2 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell1_RS_EPRE_FR1 := -78; var integer v_T2_Cell1_RS_EPRE_FR2 := -78; var integer v_T2_Cell11_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T2_Cell11_RS_EPRE_FR2 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T3_Cell11_RS_EPRE_FR1 := -78; var integer v_T3_Cell11_RS_EPRE_FR2 := -78; var integer v_Noc := -94; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3; var boolean v_IsFR1; var Q_RxLevMin v_Q_RxLevMin := -55; var DeltaValues_Type v_DeltaPrimaryBand; var float v_Wait60s := 60.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_T3_Cell1_RS_EPRE_FR1, v_T1_T3_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T1_Cell11_RS_EPRE_FR1, v_T1_Cell11_RS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T2_Cell11_RS_EPRE_FR1, v_T2_Cell11_RS_EPRE_FR2) }; v_CellPowerList_AtT3:={ cs_NR_CellPower(nr_Cell1, v_T1_T3_Cell1_RS_EPRE_FR1, v_T1_T3_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T3_Cell11_RS_EPRE_FR1, v_T3_Cell11_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS adjusts the SS/PBCH EPRE level of NR Cell 1 according to row "T1" in table 6.1.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // @siclog "Step 2" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait60s, -, "Step3: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); //@sic R5s201387 sic@ v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); // @siclog "Step 4" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 level according to row "T2" in table 6.1.2.2.3.2-1/2. if (not v_IsFR1) {// FR2: q-QualMin on SIB1 and SIB2 are FFS for FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); // @sic R5s200969 sic@ v_Q_RxLevMin := f_NR_CalculateQ_RxLevMin(-55.0, v_DeltaPrimaryBand.DeltaNRf1); } f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell11, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-12)); f_NR_CellInfo_SetSIB2_Q_QualMin(nr_Cell11, -12); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell11, v_Noc)}); f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210524 sic@ // @siclog "Step 5" siclog@ <<< SKIPPED CODE >>>

After Change:

function f_TC_6_1_2_2_NR5GC_TestBody() runs on NR5GC_PTC { // @sic R5-206318 sic@ var integer v_T1_T3_Cell1_RS_EPRE_FR1 := -91; var integer v_T1_T3_Cell1_RS_EPRE_FR2 := -91; var integer v_T1_Cell11_RS_EPRE_FR1 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T1_Cell11_RS_EPRE_FR2 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T2_Cell1_RS_EPRE_FR1 := -78; var integer v_T2_Cell1_RS_EPRE_FR2 := -78; var integer v_T2_Cell11_RS_EPRE_FR1 := tsc_NR_NonSuitableCellSSS_EPRE_FR1; var integer v_T2_Cell11_RS_EPRE_FR2 := tsc_NR_NonSuitableOffCellSSS_EPRE; var integer v_T3_Cell11_RS_EPRE_FR1 := -78; var integer v_T3_Cell11_RS_EPRE_FR2 := -78; var integer v_Noc := -94; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT1; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2; var template(value) NR_CellPowerList_Type v_CellPowerList_AtT3; var boolean v_IsFR1; var Q_RxLevMin v_Q_RxLevMin := -55; var DeltaValues_Type v_DeltaPrimaryBand; var float v_Wait60s := 60.0; v_CellPowerList_AtT1:={ cs_NR_CellPower(nr_Cell1, v_T1_T3_Cell1_RS_EPRE_FR1, v_T1_T3_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T1_Cell11_RS_EPRE_FR1, v_T1_Cell11_RS_EPRE_FR2) }; v_CellPowerList_AtT2:={ cs_NR_CellPower(nr_Cell1, v_T2_Cell1_RS_EPRE_FR1, v_T2_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T2_Cell11_RS_EPRE_FR1, v_T2_Cell11_RS_EPRE_FR2) }; v_CellPowerList_AtT3:={ cs_NR_CellPower(nr_Cell1, v_T1_T3_Cell1_RS_EPRE_FR1, v_T1_T3_Cell1_RS_EPRE_FR2), cs_NR_CellPower(nr_Cell11, v_T3_Cell11_RS_EPRE_FR1, v_T3_Cell11_RS_EPRE_FR2) }; // @siclog "Step 1" siclog@ // SS adjusts the SS/PBCH EPRE level of NR Cell 1 according to row "T1" in table 6.1.2.2.3.2-1/2. f_NR_SetCellPowerList(v_CellPowerList_AtT1); f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell1, v_Noc)}); // @siclog "Step 2" siclog@ // The UE is switched on. f_UT_SwitchOnUE(UT); // @siclog "Step 3" siclog@ // Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 60 s? f_NR_CheckNoRRCSetupReq(nr_Cell1, v_Wait60s, -, "Step3: UE sent an RRCSetupRequest on NR Cell 1 within 60 s"); //@sic R5s201387 sic@ v_IsFR1 := f_NR_CellInfo_GetIsFR1(nr_Cell1); // @siclog "Step 4" siclog@ // SS re-adjusts the SS/PBCH EPRE level of NR Cell 1 level according to row "T2" in table 6.1.2.2.3.2-1/2. if (not v_IsFR1) {// FR2: q-QualMin on SIB1 and SIB2 are FFS for FR2 v_DeltaPrimaryBand := f_NR_MobileInfo_GetPrimaryDeltaValue(); // @sic R5s200969 sic@ v_Q_RxLevMin := f_NR_CalculateQ_RxLevMin(-55.0, v_DeltaPrimaryBand.DeltaNRf1); } f_NR_CellInfo_SetSIB1_CellSelectionInfo(nr_Cell11, cs_38508_SIB1_CellSelectionInfo(v_Q_RxLevMin,-,-12)); f_NR_CellInfo_SetSIB2_Q_QualMin(nr_Cell11, -12); f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210524 sic@ f_NR_ConfigureAndActivateVNG_MultipleCells({cs_CellNoiseLevel(nr_Cell11, v_Noc)}); //f_NR_SetCellPowerList(v_CellPowerList_AtT2); //@sic R5s210524 sic@ // @siclog "Step 5" siclog@ << SKIPPED CODE >> }