3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210694

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1784 | rev | - | Current version: | 16.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for NR MAC testcase 7.1.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change 1 - If TA=0 sent at step 1 and Step18, then at step 17 and step24(UL MAC PDU) will be DTX or CRC-NG due to UL timing expectation difference between UE and SS. UE sends with TA=31(no change in UL timing) while SS expects with TA=0(change in UL timing). Therefore, TA=31 at Step 1 and step 18 shall be applied like in preambles so that UL timing will not be changed for step 17. Change 2 - 17-17B and 21-23A are C-RNTI based RACH procedure. TTCN uses same Rach configuration Therefore removed one of the configuration (cs_NR_RachProcedureConfig_71112_2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change 1 1. Set the TA value as 31 at step 1 and step18 Change 2 - Removed Rach Configuration at step 21-23A |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE may fail the test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.1.1.2.NR5GC, 7.1.1.1.2.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

# Overview

This document lists all the changes needed to correct issues in the ATS iwd-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Kalahasti, Narendra
	Narendra.Kalahasti@anritsu.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_7_1_1_1_2_NR_TestBody1 |
| --- | --- |
| Reason for change | - If TA=0 sent at step 1 and Step18, then at step 17 and step24(UL MAC PDU) will be DTX or CRC-NG due to UL timing expectation difference between UE and SS. UE sends with TA=31(no change in UL timing) while SS expects with TA=0(change in UL timing). Therefore, TA=31 at Step 1 and step 18 shall be applied like in preambles so that UL timing will not be changed for step 17. |
| Summary of change | Set SS configuration TA = 31 at step1 |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

var NR_SYSTEM_IND v_Received_NR_SysInd;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var SubFrameTiming_Type v_TimingUL2;

var SubFrameTiming_Type v_TimingUL3;

…

// Enable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

// configure RACH procedure suitable for steps 4 to 17.

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// @siclog "Step 1" siclog@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL

// Configure SS  transmission of Timing advance

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE));

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop);

...

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config

};

After change:

function f_TC_7_1_1_1_2_NR_TestBody1( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

var NR_SYSTEM_IND v_Received_NR_SysInd;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var SubFrameTiming_Type v_TimingUL2;

var SubFrameTiming_Type v_TimingUL3;

...

// Enable SR reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(enable));

// Enable PRACH reception indication

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(enable));

// configure RACH procedure suitable for steps 4 to 17.

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// @siclog "Step 1" siclog@

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,200); // Timing of DL

// Configure SS  transmission of Timing advance

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B)));

// Stop UL Grant configuration

f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop);

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

f_NR_ULGrantConfiguration_Start(nr_Cell1); // Def UL Grant Config

};

## 2.2 Change 2

| Function name | f_TC_7_1_1_1_2_NR_TestBody2 |
| --- | --- |
| Reason for change | 1. If TA=0 sent at step 1 and Step18, then at step 17 and step24(UL MAC PDU) will be DTX or CRC-NG due to UL timing expectation difference between UE and SS. UE sends with TA=31(no change in UL timing) while SS expects with TA=0(change in UL timing). Therefore, TA=31 at Step 1 and step 18 shall be applied like in preambles so that UL timing will not be changed for step 17. 2. 17-17B and 21-23A are C-RNTI based RACH procedure. TTCN uses same Rach configuration Therefore removed one of the configuration (cs_NR_RachProcedureConfig_71112_2). |
| Summary of change | 1. Set SS configuration with TA = 31 at step18 2. Removed duplicate Rach Configuration at step 21-23A |
| TTCN module | MAC_TC_Common_NR |
| MCC160 Comment | Accepted as improvement |

Before change:

function f_TC_7_1_1_1_2_NR_TestBody2( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

var SubFrameTiming_Type v_TimingDL;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,400); // Timing of DL

...

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE));

f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop);

// configure RACH procedure suitable for steps 21 to 24.

f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// @siclog "Steps 19" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 300); //@sic R5s20046 sic@

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header  =44B=352 bits >208 bits

...

// Default grant transmission, disable PRACH/SR reception indication

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now);

//@sic R5s190837 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

};

After change:

function f_TC_7_1_1_1_2_NR_TestBody2( DRB_Identity p_NR_DRB_Id) runs on NR_BASE_PTC

{

var SubFrameTiming_Type v_TimingDL;

var NR_PhysicalParameters_Type v_NR_PhysicalParameters := f_NR_CellInfo_GetPhysicalParameters(nr_Cell1);

v_TimingDL := f_NR_GetNextSendOccasion_DL(nr_Cell1,400); // Timing of DL

...

f_NR_SS_CommonCellConfig(nr_Cell1, cas_NR_MacCommandTrigger_REQ(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_MAC_TimingAdvance_CE(-,'011111'B)));

f_NR_ULGrantConfiguration_Stop(nr_Cell1, cs_TimingInfo_NR(v_TimingDL), cs_NR_UplinkTimeAlignment_Stop);

// configure RACH procedure suitable for steps 21 to 24.

// Removed f_NR_SS_RachProcedureConfig(nr_Cell1, v_NR_RachProcedure);

// @siclog "Steps 19" siclog@

v_TimingDL := f_SubFrameTiming_AddMilliSeconds(v_TimingDL, 300); //@sic R5s20046 sic@

// PDCP SDU =36B +3B PDCP Headed+ 3B RLC Header + 2B MAC Header  =44B=352 bits >208 bits

...

// Default grant transmission, disable PRACH/SR reception indication

f_NR_ULGrantConfiguration_Start(nr_Cell1, cs_TimingInfo_Now);

//@sic R5s190837 sic@

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_RachPreamble(disable));

f_NR_SS_SystemIndCtrlConfig(nr_Cell1, cds_NR_SystemIndCtrl_SchedReq(disable));

};