3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210719

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1792 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR PDCP test case 7.1.3.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-14-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Following the implementation of R5-213462, the handling of multiple PDUs in a single ASP is incorrect. e.g. current implementation of v_ExpectedPdu3 allows either v_ExpectedPdu1 or v_ExpectedPdu2 to be received, but does not allow for a list containing both ( “()” brackets are used instead of “{}”) |  |  |  |  |  |  |  |  |
| Summary of change: |  | Redefine expected single PDUs to be NR_PDCP_PDU_Type, then combine them into lists of type NR_PDCP_PDUList_Type, to allow all allowed PDUs to be received, in any order. Note : Changes are given for 7.1.3.4.1. Similar changes for 7.1.3.5.1 have been documented in R5s210787. Simialr changes will also be required for steps 7-9 of 7.1.3.5.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail these test cases |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.1.3.4.1.ENDC, 7.1.3.4.1.NR5GC, 7.1.3.5.4.ENDC, 7.1.3.5.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 1	8

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_7_1_3_4_1_NR_Step10to23 |
| --- | --- |
| Reason for change | Following the implementation of R5-213462, the current implementation of v_ExpectedPdu4 at step 15 allows either v_ExpectedPdu1 or v_ExpectedPdu2 to be received, but does not allow for a list containing both ( “()” brackets are used instead of “{}”). Similarly for v_ExpectedPdu5 and v_ExpectedPdu6. |
| Summary of change | Redefine v_ExpectedPdu1, v_ExpectedPdu2 and v_ExpectedPdu3 to be single PDUs (NR_PDCP_PDU_Type). Then combine them into lists in v_ExpectedPdu4, v_ExpectedPdu5 and v_ExpectedPdu6, to be allow all specified PDUs entries to be recieved, in any order. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted partially: Accepted: Redefine v_ExpectedPdu1, v_ExpectedPdu2 and v_ExpectedPdu3 to be single PDUs (NR_PDCP_PDU_Type). Rejected: Then combine them into lists in v_ExpectedPdu4, v_ExpectedPdu5 and v_ExpectedPdu6, to allow all specified PDUs entries to be recieved, in any order. This not per prose |

Before Change

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { ………………………………… var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; ……………………………………………… //@siclog "Step 15" siclog@ //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= (v_ExpectedPdu1, v_ExpectedPdu2); v_ExpectedPdu5:= (v_ExpectedPdu2, v_ExpectedPdu3); v_ExpectedPdu6:= (v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3 v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu3)) -> value v_ReceivedPdu3; …………………………… }

After Change

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { ………………………………… var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu3; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6; ………………………… //@siclog "Step 15" siclog@ //@sic R5-213462 sic@ p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu4:= ({v_ExpectedPdu1, v_ExpectedPdu2},{v_ExpectedPdu2, v_ExpectedPdu1}); v_ExpectedPdu5:= ({v_ExpectedPdu2, v_ExpectedPdu3}, {v_ExpectedPdu3, v_ExpectedPdu2}); v_ExpectedPdu6:= ({v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3}, {v_ExpectedPdu1, v_ExpectedPdu3, v_ExpectedPdu2}, {v_ExpectedPdu2, v_ExpectedPdu1, v_ExpectedPdu3}, {v_ExpectedPdu2, v_ExpectedPdu3, v_ExpectedPdu1}, {v_ExpectedPdu3, v_ExpectedPdu1, v_ExpectedPdu2}, {v_ExpectedPdu3, v_ExpectedPdu2, v_ExpectedPdu1}); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; alt { // receive PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 { v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); v_TimingPreviousSlot:= v_TimingCurrentSlot; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3 v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } // receive PDU2 and PDU3 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5 { v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame; f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot); f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15"); } } } // receive PDU1 and PDU2 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4 { v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame; // receive PDU3 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell2, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell2), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3; …………………………… }

MCC160 Proposal

function f_TC_7_1_3_4_1_NR_Step10to23(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var SubFrameTiming_Type v_TimingCurrentSlot;

var SubFrameTiming_Type v_TimingPreviousSlot;

// var TimingInfo_Type v_TimingInfo;

var template (present) NR_PDCP_PDU_Type  v_ExpectedPdu1;

var template (present) NR_PDCP_PDU_Type  v_ExpectedPdu2;

var template (present) NR_PDCP_PDU_Type  v_ExpectedPdu3;

var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu4;

var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu5;

var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu6;

var NR_DRB_COMMON_IND v_ReceivedPdu1;

var NR_DRB_COMMON_IND v_ReceivedPdu2;

var NR_DRB_COMMON_IND v_ReceivedPdu3;

var NR_DRB_COMMON_IND v_ReceivedPdu4;

var NR_DRB_COMMON_IND v_ReceivedPdu5;

var NR_DRB_COMMON_IND v_ReceivedPdu6;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR1 := -82;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell1_FR2 := -82;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR1 := -88;

var NR_AbsoluteCellPower_Type v_CellPowerAtT2Cell2_FR2 := -91;

var template(value) NR_CellPowerList_Type v_CellPowerList_AtT2 :=  {

cs_NR_CellPower(nr_Cell1, v_CellPowerAtT2Cell1_FR1, v_CellPowerAtT2Cell1_FR2),

cs_NR_CellPower(nr_Cell2, v_CellPowerAtT2Cell2_FR1, v_CellPowerAtT2Cell2_FR2)

}; //@sic R5-195929 sic@

timer t_Wait:= 5.0;

….

//@sic R5-213462 sic@

p_NR_PDCP_State.RX_NEXT:= 2;//@sic R5-196261 sic@

v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

v_ExpectedPdu3:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

v_ExpectedPdu4:= {v_ExpectedPdu1, v_ExpectedPdu2};

v_ExpectedPdu5:= {v_ExpectedPdu2, v_ExpectedPdu3};

v_ExpectedPdu6:= {v_ExpectedPdu1, v_ExpectedPdu2, v_ExpectedPdu3};

alt {

// receive PDU1

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 // @sic R5s210719 sic@

{

v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame;

alt {

// receive PDU2

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2 // @sic R5s210719 sic@

{

v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

v_TimingPreviousSlot:= v_TimingCurrentSlot;

}

// receive PDU3

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3 // @sic R5s210719 sic@

{

v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

}

// receive PDU2 and PDU3

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu5)) -> value v_ReceivedPdu5

{

v_TimingCurrentSlot := v_ReceivedPdu5.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

}

}

}

// receive PDU1 and PDU2

[] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu4)) -> value v_ReceivedPdu4

{

v_TimingPreviousSlot := v_ReceivedPdu4.Common.TimingInfo.SubFrame;

// receive PDU3

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu3})) -> value v_ReceivedPdu3;

v_TimingCurrentSlot := v_ReceivedPdu3.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

}

// receive PDU1, PDU2 and PDU3

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu6)) -> value v_ReceivedPdu6

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 15");

}

}

…

}

## Change 2

| Function name | f_TC_7_1_3_4_1_NR_Step25to30 |
| --- | --- |
| Reason for change | Following the implementation of R5-213462, the current implementation of v_ExpectedPdu3 at steps 30-31 allows either v_ExpectedPdu1 or v_ExpectedPdu2 to be received, but does not allow for a list containing both ( “()” brackets are used instead of “{}”). |
| Summary of change | Redefine v_ExpectedPdu1 and v_ExpectedPdu2 to be single PDUs (NR_PDCP_PDU_Type). Then combine them into a lists in v_ExpectedPdu3, to be allow both PDUs entries to be recieved, in any order. |
| TTCN module | PDCP_TC_Common_NR.ttcn |
| MCC160 Comment | Accepted partially: Accepted: Redefine v_ExpectedPdu1 and v_ExpectedPdu2 to be single PDUs (NR_PDCP_PDU_Type). Rejected: Then combine them into a lists in v_ExpectedPdu3, to allow both PDUs entries to be recieved, in any order. This not per prose |

Before Change

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { ……………………………………… var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; ……………………………………………… //@siclog "Steps 30 - 31" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#6.(Note 9) //Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#7(Note 9). //@sic R5-213462 sic@ v_ExpectedPdu1:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= {f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT)}; f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= (v_ExpectedPdu1,v_ExpectedPdu2); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu1)) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu2)) -> value v_ReceivedPdu2; ………………………………………… }

After Change

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State, boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC { ………………………………………… var template (present) NR_PDCP_PDU_Type v_ExpectedPdu1; var template (present) NR_PDCP_PDU_Type v_ExpectedPdu2; var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3; ………………………………………… //@siclog "Steps 30 - 31" siclog@ //Check: Does the UE transmit a PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=6 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#6.(Note 9) //Check: Does the UE transmit PDCP Data PDU via RLC-AM RB with the following content back to the SS? D/C field = 1 (PDCP Data PDU) and PDCP SN=7 on NR Cell 1. //Note: Data is previously received packet in PDCP Data PDU#7(Note 9). //@sic R5-213462 sic@ v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT); f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1); v_ExpectedPdu3:= ({v_ExpectedPdu1,v_ExpectedPdu2},{v_ExpectedPdu2,v_ExpectedPdu1}); alt { // receive PDU1 [] DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu1})) -> value v_ReceivedPdu1 { v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame; // receive PDU2 DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; ……………………………………………………… }

## MCC160 Proposal

function f_TC_7_1_3_4_1_NR_Step25to30(inout NR_PDCP_SS_State_Type p_NR_PDCP_State,  boolean p_WaitforRRCConCmp := false) runs on NR_BASE_PTC

{

var octetstring v_IPData := f_IPv4IPv6_IcmpEchoReply(f_LoopbackModeB_IP_Address_UE());

var NR_ResourceAllocation_Type v_NR_ResourceAllocation;

var SubFrameTiming_Type v_TimingDL;

var SubFrameTiming_Type v_TimingUL;

var SubFrameTiming_Type v_TimingCurrentSlot;

var SubFrameTiming_Type v_TimingPreviousSlot;

var template (present) NR_PDCP_PDU_Type  v_ExpectedPdu1;

var template (present) NR_PDCP_PDU_Type  v_ExpectedPdu2;

var template (present) NR_PDCP_PDUList_Type v_ExpectedPdu3;

var NR_DRB_COMMON_IND v_ReceivedPdu1;

var NR_DRB_COMMON_IND v_ReceivedPdu2;

var NR_DRB_COMMON_IND v_ReceivedPdu3;

var NR_UplinkBWP_Type v_NR_UplinkBWP := f_NR_CellInfo_GetUplinkBWP(nr_Cell1, tsc_NR_BWP_Id);

timer t_Wait:= 5.0;

…

//@sic R5-213462  sic@

v_ExpectedPdu1:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

v_ExpectedPdu2:= f_NR_PDCP_PDU_Get(p_NR_PDCP_State.PDCP_SN_Size, p_NR_PDCP_State.RX_NEXT);

f_NR_PDCP_Increment_NEXT_RX(p_NR_PDCP_State,1);

v_ExpectedPdu3:= {v_ExpectedPdu1,v_ExpectedPdu2};

alt {

// receive PDU1

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -,{v_ExpectedPdu1})) -> value v_ReceivedPdu1; // @sic R5s210719 sic@

{

v_TimingPreviousSlot := v_ReceivedPdu1.Common.TimingInfo.SubFrame;

// receive PDU2

DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, {v_ExpectedPdu2})) -> value v_ReceivedPdu2; // @sic R5s210719 sic@

v_TimingCurrentSlot := v_ReceivedPdu2.Common.TimingInfo.SubFrame;

f_NR_SubFrameTiming_DifferentSlots (v_TimingCurrentSlot, v_TimingPreviousSlot);

f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31");

}

// receive PDU1 and PDU2

[]  DRB.receive(car_NR_DRB_COMMON_IND_PDCP_PDUList_DC(nr_Cell1, p_NR_PDCP_State.DRB_Id, cr_RlcBearerRouting_NR(nr_Cell1), -, v_ExpectedPdu3))-> value v_ReceivedPdu3

{

f_NR_PreliminaryPass(__FILE__, __LINE__, "Steps 30-31");

}

}

}