3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210720

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-3 | CR | 0726 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of IMS 5GS registration test case 6.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-06-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add IMS 5GS registration test case 6.2 to the IMS/NR5GC ATS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This document lists all changes applied to IMS 5GS registration test case 6.2 required for approval. See detailed change description for further information |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case will not be added to ATS |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

1.1	Verification Test Summary	3

2	Corrections required	3

2.1 Change 1	3

3	Branches executed	6

4	Execution Log Files	7

4.1	Qualcomm SM8350+SDX60	7

5	References	7

# Overview

This document lists all the essential changes needed to correct problems in the TTCN implementation of test case 6.2 which is part of the IMS NR5GC test suite.

With these changes applied the test case can be demonstrated to run with one or more UEs (see section 4). Execution log files are provided as evidence.

Contact:	Shaun Harry
	shaun.harry@keysight.com

## Verification Test Summary

Test Case: 	6.2.NR5GC

ATS Version:	iwd-TTCN3-B2020-09_D12wk24

System Simulator used:	Keysight S8704A Protocol Conformance Toolset

UE used:	Qualcomm SM8350+SDX60

Verification Status:	PASS

# Corrections required

## 2.1 Change 1

| Function name | f_TC_6_2_NR5GC_IMS1 |
| --- | --- |
| Reason for change | 1. Second P-CSCF address allocation was missing. 2. Step 6 timer needs to consider RRC Release timer of 1.2 second. |
| Summary of change | In f_IMS_CC_Preamble function call configured p_NumOfIPAddressesToAllocate to 2 instead of 1. 2. Corrected Step 6 timer as 8.8 second. |
| TTCN module | IMS_NR5GC_RegistrationTestcases.ttcn |
| MCC160 Comment | - Accepted - Accepted in principle: Instead of calculation of the timer value depending a delay which is specified elsewhere, the timer shall be started before RRC connection release |

Before Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2) var charstring v_RegisterExpiration; var integer v_Cseq; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address2_NW, px_IPv6_Address2_NW); /* @sic R5-213534: support of 2nd P-CSCF sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ f_Delay(f_IMS_SetTimerToleranceMin(10.0)); v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

After Change:

function f_TC_6_2_NR5GC_IMS1() runs on IMS_PTC { var IMS_DATA_REQ v_IMS_REGISTER_REQ; var REGISTER_Request v_RegisterReq; var template (value) IMS_RoutingInfo_Type v_RoutingInfoDL; var template (omit) RetryAfter v_RetryAfterForREGISTER; var integer v_RetryAfterForPublish := 60; // as per "503 Service Unavailable" default message contents (34.229-1 A.4.2) var charstring v_RegisterExpiration; var integer v_Cseq; f_IMS_CC_Preamble(IPCAN_SignallingOnly, IMS_NULL, IMS_Security, -,-,2); f_IMS_TestBody_Set(true); // @siclog "Step 1" siclog@ f_IMS_CC_StartSignalling(IPCAN_InitialRegistration); // @siclog "Step 2" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); // @siclog "Step 3" siclog@ v_RetryAfterForREGISTER := omit; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_InitialRegistration); f_IMS_RestartPCSCF(PDN_1, px_IPv4_Address2_NW, px_IPv6_Address2_NW); /* @sic R5-213534: support of 2nd P-CSCF sic@ NOTE: IP addresses need to be consistent with what is used as 2nd P-CSCF at NAS signalling */ f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 4" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 4"); // @siclog "Step 5" siclog@ v_RetryAfterForREGISTER := cs_RetryAfter(10); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine503, f_IMS_CommonResponse_503_MessageHeaderTX(v_RegisterReq.msgHeader, v_RetryAfterForREGISTER)))); f_IMS_CC_ReleaseConnection(IPCAN_MO_IMS_Signalling); f_IMS_CC_StartSignalling(IPCAN_MO_IMS_Signalling); // @siclog "Step 6" siclog@ f_Delay(f_IMS_SetTimerToleranceMin(8.8)); v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; v_Cseq := fl_TC_6_2_CheckGetCseq(v_RegisterReq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 6"); // @siclog "Step 7" siclog@ v_RegisterExpiration := "800000"; IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine423, f_IMS_RegisterResponse_423_MessageHeaderTX(v_RegisterReq, v_RegisterExpiration)))); // @siclog "Step 8" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_InitialRequest(-, -, -, v_RegisterExpiration); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; fl_TC_6_2_CheckGetCseq(v_RegisterReq, v_Cseq); f_IMS_PreliminaryPass(__FILE__, __LINE__, "Step 8"); // @siclog "Step 9" siclog@ f_IMS_Register_SecurityInit(v_RegisterReq); IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine401, f_IMS_RegisterResponse_401_MessageHeaderTX(v_RegisterReq)))); // @siclog "Step 10" siclog@ v_IMS_REGISTER_REQ := f_IMS_REGISTER_SubsequentRequest(-, -, -, -, v_RegisterExpiration); v_RoutingInfoDL := f_IMS_RoutingInfo_ULtoDL(v_IMS_REGISTER_REQ.RoutingInfo); v_RegisterReq := v_IMS_REGISTER_REQ.Request.Register; // @siclog "Step 11" siclog@ IMS_Server.send(cas_IMS_DATA_RSP(v_RoutingInfoDL, cs_Response(c_statusLine200, f_IMS_RegisterResponse_200_MessageHeaderTX(v_RegisterReq, -, v_RegisterExpiration)))); // @siclog "Step 12 - 15" siclog@ f_IMS_Register_SubscribeNotify(v_IMS_REGISTER_REQ, tsc_IMS_PublishDuringSubscribeForRegEvent, v_RetryAfterForPublish); f_IMS_TestBody_Set(false); f_IMS_CC_Postamble(IPCAN_MO_IMS_Signalling); }

# Branches executed

The test case was executed on NR bands n78

# Execution Log Files

## Qualcomm SM8350+SDX60

The Qualcomm SM8350+SDX60  UE passed this test case on Keysight S8704A Protocol Conformance Toolset. The documentation below is enclosed as evidence of the successful test case run [1]:

- Test case execution log file:

Keysight/TC_6_2_LOG.html

- PICS/PIXIT parameter file:

Keysight/TC_6_2_PIXIT.xml

# References

| [1] | R5s210685: Supporting information for addition of IMS 5GS registration test case 6.2 |
| --- | --- |