3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210722

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1794 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC mulitlayer test case 11.1.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Change 1.2 and 2 in R5s210478 was accepted but not implemented in 21wk24 TTCN. Without such change, the SI change may not take effect in time before step 2 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implemented Change 1.2 and 2 in R5s210478. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.1.4.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.523-1 CR xxxx |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

2.2	Change 2	4

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_11_1_4_NR5GC_EUTRA |
| --- | --- |
| Reason for change | Change 1.2 in R5s210478, accepted but not implemented |
| Summary of change | Implement change 1.2 of R5s210478 |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment | Accepted, subject to Prose CR agreement. |

Before Change

function f_TC_11_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell reselection using cell status barred / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-206360 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); // @sic R5-206360 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_1_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

After Change

function f_TC_11_1_4_NR5GC_EUTRA() runs on EUTRA_Multi5GS_PTC { //MO MMTEL voice call setup from NR RRC_CONNECTED / EPS Fallback with redirection / Single registration mode with N26 interface / E-UTRAN cell reselection using cell status barred / Success f_EUTRA38_IRAT_Init(c31, HANDLE_UM_DATA); // @sic R5s210414 sic@ f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); //Create and configure cells f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellConfig_Def(eutra_Cell3); // @sic R5-206360 sic@ f_EUTRA38_NR5GC_PreambleAndSwitchOff (eutra_Cell1); // @sic R5-206360 sic@ f_EUTRA_TestBody_Set(true); fl_TC_11_1_4_NR5GC_EUTRA_TestBody (); f_EUTRA_TestBody_Set(false); f_EUTRA38_MultiPDN_Postamble(eutra_Cell1, E2_CONNECTED); // @sic R5-204399 sic@ }

## Change 2

| Function name | fl_TC_11_1_4_NR5GC_EUTRA_TestBody |
| --- | --- |
| Reason for change | Change 2 in R5s210478, accepted but not implemented |
| Summary of change | Implement change 2 of R5s210478 |
| TTCN module | EPS_Fallback_NR5GC_EUTRA |
| MCC160 Comment | Accepted, subject to Prose CR agreement. |

Before Change

function fl_TC_11_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; // @sic R5s210304 sic@ //Eutra Cell1 is set as barred and modified Sysinfo updated, setting the Cell power to suitable neighbour cell FFS : Prose CR ?? f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); // @sic R5-206360 sic@ f_EUTRA_ModifySysinfo(eutra_Cell3); // @sic R5-206360 sic@ // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE); // @sic R5-206360 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @sic R5-206360 sic@ ..... }

After Change

function fl_TC_11_1_4_NR5GC_EUTRA_TestBody() runs on EUTRA_Multi5GS_PTC { var NasCount_Type v_NasCount; // @sic R5s210304 sic@ //Eutra Cell1 is set as barred and modified Sysinfo updated, setting the Cell power to suitable neighbour cell FFS : Prose CR ?? // REMOVE f_EUTRA_CellInfo_SetSysInfo_CellBarred(eutra_Cell3, barred); // @sic R5-206360 sic@ // REMOVE f_EUTRA_ModifySysinfo(eutra_Cell3); // @sic R5-206360 sic@ // @siclog "Step 1" siclog@ f_EUTRA38_SetCellPower(eutra_Cell1, tsc_SuitableCellRS_EPRE, tsc_SuitableCellRS_EPRE); // @sic R5-206360 sic@ f_EUTRA38_SetCellPower(eutra_Cell3, tsc_ServingCellRS_EPRE, tsc_ServingCellRS_EPRE); // @sic R5-206360 sic@