3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210731

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1802 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction for NR5GC multilayer test case 11.3.7 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | At step8a1/8b1, the UE may send SERVICE REQ, or REGISTRATION REQ as per test spec. the TTCN implementation calling f_Check_NG_ServiceReqMsg to check the received message gainst SERVICE REQ, and if UE sends REGISTRATION REQ (and not SERVICE REQ), the TTCN fails the testcase with FatalError on clearText IE error, rather than proceed to check REGISTRATION REQ |  |  |  |  |  |  |  |  |
| Summary of change: |  | TTCN has been modified to handle the SERVICE REQ/REG REQ by first checking the message type what has been received from UE so that respective messages can be compared correctly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.7.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

- Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

- Corrections required

## 2.1 Change 1

| Function name | fl_TC_11_3_7_TestBody() |
| --- | --- |
| Reason for change | At step8a1/8b1, the UE may send SERVICE REQ, or REGISTRATION REQ as per test spec. the TTCN implementation calling f_Check_NG_ServiceReqMsg to check the received message gainst SERVICE REQ, and if UE sends REGISTRATION REQ (and not SERVICE REQ), the TTCN fails the testcase with FatalError on clearText IE error, rather than proceed to check REGISTRATION REQ |
| Summary of change | TTCN has been modified to handle the SERVICE REQ/REG REQ by first checking the message type what has been received from UE so that respective messages can be compared correctly. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted. |

Before Change:

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { … … //@siclog "Step 7" siclog@ //@sic R5-213504 sic@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell1); //@sic R5-213504 sic@ //EXCEPTION: Steps 8a1 to 8b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //@siclog "Step 8a1" siclog@ //The UE transmits an RRCSetupComplete message AND a SERVICE REQUEST message with 'Service type' set to 'high priority access'. if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0101'B)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Service Request"); //@siclog "Step 8a2" siclog@ //Steps 4-8 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.9.11. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } //@siclog "Step 8b1" siclog@ //The UE transmits an RRCSetupComplete message AND a REGISTRATION REQUEST message with 'registration type' set to 'emergency'. (Note 6) //Note 6: UE may try to find service to place the emergency call (step 1) on other RATs since emergency access is barred on NR Cell 1. In this case UE will send a REGISTRATION REQUEST instead of a SERVICE REQUEST. else if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //@siclog "Step 8b2" siclog@ //Steps 5-15 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.5.2.2-2 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Emergency); // @sic R5s210424 sic@ //a Prose CR is needed for RAN5 92-e meeting SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message or Registration Request Message Failed"); } //@siclog "Step 9" siclog@ //@sic R5-211410, R5-213504 sic@ //Void. … … }

After Change:

function fl_TC_11_3_7_TestBody() runs on NR5GC_PTC { … … //@siclog "Step 7" siclog@ //@sic R5-213504 sic@ //The SS transmits an RRCSetup message. f_NR_RRCSetup_Def (nr_Cell1); //@sic R5-213504 sic@ //EXCEPTION: Steps 8a1 to 8b1 describe behaviour that depends on the UE implementation; the "lower case letter" identifies a step sequence that take place if a particular implementation is under test. v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); //Service Request if (match (v_ReceivedMsg.Pdu.Msg, cr_NG_SERVICE_REQUEST (?, ?, ?, omit, omit, omit, *))) { //@siclog "Step 8a1" siclog@ //The UE transmits an RRCSetupComplete message AND a SERVICE REQUEST message with 'Service type' set to 'high priority access'. if (f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get(), tsc_NasKsi_NativeSecurityContext), f_NG_S_TMSI2MobileIdentity(omit, f_NR5GC_CellInfo_GetGutiParameters (nr_Cell1)),'0101'B)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Service Request"); //@siclog "Step 8a2" siclog@ //Steps 4-8 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.9.11. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } } //Registration Req else if ( match (v_ReceivedMsg.Pdu.Msg, cr_NG_REGISTRATION_REQUEST(?, // @sic R5-196637 sic@ ?, ?, ?, omit, omit, *, omit, omit, omit, omit, omit, omit, *, // default * *, // default * omit, omit, omit, *, // default * omit, omit, omit, omit, omit, omit, omit, // @sic R5s201387 Baseline Moving sic@ omit, // @sic R5s201387 Baseline Moving sic@ *, omit, // @sic R5s201387 Baseline Moving sic@ omit, omit, omit, omit, omit, omit, omit))) { //@siclog "Step 8b1" siclog@ //The UE transmits an RRCSetupComplete message AND a REGISTRATION REQUEST message with 'registration type' set to 'emergency'. (Note 6) //Note 6: UE may try to find service to place the emergency call (step 1) on other RATs since emergency access is barred on NR Cell 1. In this case UE will send a REGISTRATION REQUEST instead of a SERVICE REQUEST. if (f_Check_NG_RegistrationReqMsg (v_GMM_MobilityInfo, v_ReceivedMsg.Pdu, Emergency)) { f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Establishment: Registration Request"); // Store GMM Mobility Info now f_NR5GC_MobileInfo_SetGMM_MobilityInfo(v_GMM_MobilityInfo); //@siclog "Step 8b2" siclog@ //Steps 5-15 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.5.2.2-2 f_NR5GC_RRC_Idle_Steps5_15(v_GMM_MobilityInfo, nr_Cell1, v_ReceivedMsg, v_PLMN, TESTMode_OFF, Emergency); // @sic R5s210424 sic@ //a Prose CR is needed for RAN5 92-e meeting SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } } } else { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message or Registration Request Message Failed"); } //@siclog "Step 9" siclog@ //@sic R5-211410, R5-213504 sic@ //Void. … … }