3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210757

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1815 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to NR5GC multilayer test case 11.3.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-14-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1. Following switch on at step2A5, the SS side routing table is not cleared which can cause issues during next DRB activation. 2. The t_WaitTime timer started after step 16 is currently never stopped which can cause test case failure when it eventually expires 3. The service request procedure at step 21 shall be handled but not completed since UE may not support emergency call over NR and the TC will become complicated if both IMS emergency call over NR and EPS fallback cases are handled. In this case UE will start T3580 after the transmission of PDU SESSION ESTABLISHMENT REQUEST message for emergency call. While T3580 is running, the postamble procedure can be performed successfully without any interruption. 4. If the SS does not respond to PDU SESSION ESTABLISHMENT REQUEST for emergency session, as per 24.501 cl 4.32, this may become a case for ‘IMS voice not available’ and UE may disable N1 mode capability, UEs implemented in this way will implicitly de-register. 5. The bearer Id is incorrectly set to a hardcoded value of ‘50’O in the QoS parameters when emergency bearer is established. This may lead a conformant UE to reject the PDU SESSION ESTABLISHMENT ACCEPT due to a mismatch in configured EPS bearer IDs 6. Due to the RRCReject sent at step 16, this test case should use early contention resolution |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Used function f_NR5GC_SwitchOnAndResetIMS() at step-2A5 instead of f_UT_SwitchOnUE() to clear and restore the IP Routing table. - Replaced the common function f_NR_RRC_ConnEst_DefWithNas for RRC connection with individual functions so that the t_WaitTime timer can be passed into f_NR_RRC_SetupRequest_Common() for stopping - Deleted the steps for emergency PDU Session Establishment - Added steps so that UEs may implicitly de-register. Removed RRCRelease step as it is now handled in the steps for postamble. - Removed the hardcoded value for bearer Id, and used passed value p_EPSBearerID - Added the test case name to the function for early contention resolution attribute, so the SS is not configured with default rrcSetup for msg4. Note: A Prose CR on 38.523-1 associatedf with changes 3 and 4 will be raised at RAN5#92 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.3.2.NR5GC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | 38.523-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	3

1	Overview	4

2	Corrections required	4

2.1	Change 1	4

2.2	Change 2	5

2.3	Change 3	9

2.4	Change 4	10

2.5	Change 5	11

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## Change 1

| Function name | f_TC_11_3_2_NR5GC |
| --- | --- |
| Reason for change | Since the postamble steps to handle the optional DEREGISTRATION REQUEST are covered in the test body (following change 2), the postamble function is no longer needed. |
| Summary of change | The postamble function is removed and replaced with functions for local resource release. |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_11_3_2_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 0 / 0% access probability / Paging for MT Access / Emergency call f_NR5GC_Init(NR_1); // Table 11.3.2.3.3-1: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(2,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_3_2_TestBody(); f_NR_TestBody_Set(false); f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); }

After Change

function f_TC_11_3_2_NR5GC() runs on NR5GC_PTC { // UAC / Access Identity 0 / 0% access probability / Paging for MT Access / Emergency call f_NR5GC_Init(NR_1); // Table 11.3.2.3.3-1: SIB1 for NR Cell 1 f_NR_CellInfo_SetSIB1_UacBarringInfo(nr_Cell1, cs_SIB1_UacBarringInfo(cs_UAC_BarringPerCatList_1Entry(2,1), -, cs_UAC_BarringInfoSetList_1Entry(p00, s4, '0000000'B))); //Create and configure cell f_NR_CellConfig_Def(nr_Cell1); f_NR5GC_Preamble(nr_Cell1, STATE_IDLE_1A); f_NR_TestBody_Set(true); fl_TC_11_3_2_TestBody(); f_NR_TestBody_Set(false); //REMOVED f_NR_Postamble(nr_Cell1, STATE_IDLE_1A); f_NR5GC_MobileInfo_Init(); f_NR_ReleaseAllCells(); }

## Change 2

| Function name | fl_TC_11_3_2_TestBody |
| --- | --- |
| Reason for change | - Following switch on at step2A5, the SS side routing table is not cleared which can cause issues during next DRB activation. - The t_WaitTime timer started after step 16 is currently never stopped which can cause test case failure when it eventually expires - The service request procedure at step 21 shall be handled but not completed since UE may not support emergency call over NR and the TC will become complicated if both IMS emergency call over NR and EPS fallback cases are handled. In this case UE will start T3580 after the transmission of PDU SESSION ESTABLISHMENT REQUEST message for emergency call. While T3580 is running, the postamble procedure can be performed successfully without any interruption. - If the SS does not respond to PDU SESSION ESTABLISHMENT REQUEST for emergency session, as per 24.501 cl 4.32, this may become a case for ‘IMS voice not available’ and UE may disable N1 mode capability, UEs implemented in this way will implicitly de-register. |
| Summary of change | - Used function f_NR5GC_SwitchOnAndResetIMS() at step-2A5 instead of f_UT_SwitchOnUE() to clear and restore the IP Routing table. - Replaced the common function f_NR_RRC_ConnEst_DefWithNas for RRC connection with individual functions so that the t_WaitTime timer can be passed into f_NR_RRC_SetupRequest_Common() for stopping - Deleted the steps for emergency PDU Session Establishmen - Added steps so that UEs may implicitly de-register. Removed RRCRelease step as it is now handled in the steps for postamble. Note: A Prose CR on 38.523-1 associatedf with changes 3 and 4 will be raised at RAN5#92 |
| TTCN module | UAC_NR5GC.ttcn |
| MCC160 Comment | - Accepted - Accepted - Rejected. Emergency PDU Session will be established if pc_NR_5GC_EmergencyServices supported. Implementation according to R5-216191 - Accepted and implemented according to R5-216191 |

Before Change

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; … … //@siclog "Step 2A5" siclog@ //Switch on the UE f_UT_SwitchOnUE() ; … … //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); //@sic R5-213505 sic@ //@siclog "Step 18 - 27" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? //Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); // Steps 8-13 f_NR5GC_EmergencyCallPDUSession(nr_Cell1); f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); //@siclog "Step 33" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. f_NR_RRCRelease(nr_Cell1); }

After Change

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC { var NG_NAS_MSG_Indication_Type v_ReceivedMsg; var NR_SRB_COMMON_IND v_ReceivedAsp; var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1); var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams); var RejectWaitTime v_RejectWaitTime := 16; var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime)); var float v_TimerValue := 30.0; timer t_WaitTime; var GSM_MobilityInfo_Type v_GSM_MobilityInfo; … … //@siclog "Step 2A5" siclog@ //Switch on the UE f_NR5GC_SwitchOnAndResetIMS(); … … //@siclog "Step 17" siclog@ // Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1) //NOTE 1: This could be done by e.g. MMI or AT command. f_UT_RequestIMSEmergencyCall (UT); //@sic R5-213505 sic@ //@siclog "Step 18 - 27" siclog@ //Check: Does the UE transmit an RRCSetupRequest message with establishmentCause set to 'emergency' (Note 2)? //Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1. //REMOVED v_ReceivedMsg := f_NR_RRC_ConnEst_DefWithNas (nr_Cell1, tsc_NR_RRC_TI_Def, emergency); f_NR_RRC_SetupRequest_Common(nr_Cell1, t_WaitTime, cr_NR_EstablishmentCause_emergency); f_NR_RRCSetup_Def(nr_Cell1); v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1, cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def), tsc_SHT_IntegrityProtected); if (not f_Check_NG_ServiceReqMsg (v_ReceivedMsg.Pdu, cs_NAS_KeySetIdentifier_lv(f_NR_SecurityKSIamf_Get (), tsc_NasKsi_NativeSecurityContext), v_S_TMSI, '0011'B)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Service Request Message Failed"); } else{ f_NR_PreliminaryPass(__FILE__, __LINE__, "Step 18"); } //Steps 4-8 are performed from the generic test procedure as specified in TS 38.508-1 [4] subclause 4.9.11. f_NR5GC_RRC_ConnectedState3N_Steps5_8(nr_Cell1, v_ReceivedMsg); SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, ?, cr_NG_NAS_IndWithPiggybacking(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp; if (not f_Check_NG_PDUSessionEstablishmentReq(v_GSM_MobilityInfo, v_ReceivedAsp.Signalling.Nas[0].Pdu, -, -, -, -, crs_SSC_Mode('A'H, '001'B), -, -, -, -, -, -, cr_NG_Request_Type('011'B),//initial emergency request omit, omit)) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "PDU Session Establishment Message Failed"); } // Steps 8-13 /* REMOVED f_NR5GC_EmergencyCallPDUSession(nr_Cell1); //f_IMS_IPCAN_WaitForTrigger(IMS[tsc_Index_IMS2]); //f_NR_PreliminaryPass(__FILE__, __LINE__, "IMS Emergency Call Established"); */ if(pc_SwitchOnOff){ f_UT_SwitchOffUE(UT, true); //@siclog "Step 28a2" siclog@ //SS starts timer1 = 5 sec to see if UE performs optional Step 10a3a1 (Note 5) t_WaitTime.start(5.0); //@siclog "Steps 28a3a1-28a3a2" siclog@ //The UE transmits a DEREGISTRATION REQUEST message. //The SS transmits an RRCRelease message IF step 10a3a1 was performed ELSE SS implicitly releases the RRC connection. alt { [] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1, tsc_NR_RbId_SRB2, cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{ if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) { f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed"); } f_NR_RRCRelease(nr_Cell1); } [] t_WaitTime.timeout { f_NR_RRCRelease_Local(nr_Cell1, f_NR_GetNextSendOccasion_DL(nr_Cell1), tsc_CnfReq); } } } //@siclog "Step 28b1" siclog@ //ELSE power off UE else { f_UT_PowerOffUE (UT); } //@siclog "Step 29-32 VOID" siclog@ //@siclog "Step 33 VOID" siclog@ //SS transmits an RRCRelease message to release RRC connection and move the UE to RRC_IDLE. //REMOVED f_NR_RRCRelease(nr_Cell1); }

MCC160 Implementation

function fl_TC_11_3_2_TestBody() runs on NR5GC_PTC

{

var NG_NAS_MSG_Indication_Type v_ReceivedMsg;

var NR_SRB_COMMON_IND v_ReceivedAsp;

var NG_NAS_GutiParameters_Type v_GutiParams := f_NR5GC_CellInfo_GetGutiParameters(nr_Cell1);

var template (present) NG_MobileIdentity v_S_TMSI := f_NG_S_TMSI2MobileIdentity(omit, v_GutiParams);

var RejectWaitTime v_RejectWaitTime := 16;

var float v_WaitForConnectionMin := f_NR_SetTimerToleranceMin(nr_Cell1, rrcTimer, int2float(v_RejectWaitTime));

var float v_TimerValue := 30.0;

timer t_WaitTime;

timer t_Wait:= 15.0;

//@siclog "Step 1" siclog@

// Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1)

f_UT_RequestIMSEmergencyCall (UT);

…

//@siclog "Step 17" siclog@

// Make the UE attempt an emergency call dialling a number which is stored on the ME (e.g. 112 or 911). (Note 1)

//NOTE 1: This could be done by e.g. MMI or AT command.

f_UT_RequestIMSEmergencyCall (UT);

t_WaitTime.stop; //@sic R5s210804 sic@

//@sic R5-216191 sic@

//EXCEPTION : Steps 21a1 – 21b6 describe the UE behaviour that depends on UE capability; the “lower case letter” identifies a step sequence that takes place if a capability is supported

//@siclog "Steps 21a1 - 21a12" siclog@

//IF pc_NR_5GC_EmergencyServices  THEN Step 2-13 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1.

if (pc_NR_5GC_EmergencyServices){

f_NR5GC_EmergencyCallWithRegistration(nr_Cell1);

//@siclog "Step 21a13" siclog@

//Make the UE release the emergency call.

f_NR5GC_EmergencyCallRelease(nr_Cell1);

f_NR_RRCRelease(nr_Cell1);

f_IP_Handling_StopPDN(IP, cs_IP_StopPDN(PDN_2));

}

else{

//@siclog "Steps 21b1- 21b4" siclog@

//ELSE Step 2- 5 for IMS Emergency call establishment are performed as specified in TS 38.508-1 [4], subclause 4.9.11 on NR Cell 1.

f_NR_RRC_SetupRequest_Common(nr_Cell1, t_Wait, cr_NR_EstablishmentCause_emergency);

f_NR_RRCSetup_Def(nr_Cell1);

v_ReceivedMsg := f_NR_RRCSetupComplete_Def(nr_Cell1,

cr_38508_RRCSetupComplete(tsc_NR_RRC_TI_Def),

tsc_SHT_IntegrityProtected);

f_NR_RRC_ActivateSecurity (nr_Cell1, valueof(v_ReceivedMsg.SecurityProtection.NasCount));

//@siclog "Steps 21b5" siclog@

//IF pc_SwitchOnOff THEN switch off UE, IF pc_USIM_Removal THEN remove the USIM.

if(pc_SwitchOnOff){

//@siclog "Steps 21b6" siclog@

t_WaitTime.start(15.0);

alt{

//@siclog "Steps 21b6a1" siclog@

[] SRB.receive(car_NR_SRB_NasPdu_IND(nr_Cell1,

tsc_NR_RbId_SRB1,

cr_NG_NAS_Ind(tsc_SHT_IntegrityProtected_Ciphered))) -> value v_ReceivedAsp{

t_WaitTime.stop;

if (not f_Check_NG_DeregistrationReqMOMsg (v_ReceivedAsp.Signalling.Nas[0].Pdu.Msg, crs_DeregisterType('1'B, '0'B, '01'B))) {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration Request Message Failed");

}

//@siclog "Steps 21b6a2" siclog@

f_NR_RRCRelease(nr_Cell1);

}

[] t_WaitTime.timeout {

f_NR_SetVerdictFailOrInconc(__FILE__, __LINE__, "Deregistration failed");

}

}

}

else{

//@siclog "Steps 21b6b1" siclog@

f_UT_PowerOffUE (UT);

}

}

}

## Change 3

| Function name | TC_11_3_2_NR5GC |
| --- | --- |
| Reason for change | Since the emergency call is no longer fully established (see change 2), the IMS signalling for the emergency call is no longer needed. |
| Summary of change | Remove the IMS2 PTC for this TC. |
| TTCN module | NR5GC_Testsuite.ttcn |
| MCC160 Comment | Rejected. Emergency call will be stablished when pc_NR_5GC_EmergencyServices is supported |

Before Change

testcase TC_11_3_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_2_NR5GC()); v_IMS2.start(f_TC_11_3_2_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

After Change

testcase TC_11_3_2_NR5GC() runs on MTC_NR5GC system SYSTEM_NR5GC { // @purpose // UAC / Access Identity 0 / 0% access probability / Paging for MT Access/Emergency Call var NR5GC_PTC v_NR5GC := null; var IMS_PTC v_IMS1 := null; var IMS_PTC v_IMS2 := null; timer t_GuardTimer := int2float(300); v_NR5GC := NR5GC_PTC.create("NR5GC") alive; v_IMS2 := IMS_PTC.create("IMS2") alive; f_MTC_ConnectPTCs_NR5GC(system, v_NR5GC, v_IMS1, v_IMS2); v_NR5GC.start(f_TC_11_3_2_NR5GC()); //REMOVED v_IMS2.start(f_TC_11_3_2_NR5GC_IMS2()); t_GuardTimer.start; f_MTC_MainLoop_NR5GC(t_GuardTimer); }

## Change 4

| Function name | f_BuildDefaultQosFlow |
| --- | --- |
| Reason for change | The bearer Id is incorrectly set to a hardcoded value of ‘50’O in the QoS parameters when emergency bearer is established. This may lead a conformant UE to reject the PDU SESSION ESTABLISHMENT ACCEPT due to a mismatch in configured EPS bearer IDs |
| Summary of change | Removed the hardcoded value for bearer Id, and used passed value p_EPSBearerID |
| TTCN module | NG_NAS_Templates.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, '50'O); } } … … }

After Change

function f_BuildDefaultQosFlow (charstring p_FlowNumber, boolean p_InterworkWithEPS := false, O1_Type p_EPSBearerId := '00'O) return template (value) QoS_Flow { var template (value) QoSParametersList v_QoSParametersList; var B6_Type v_QFI; select (p_FlowNumber) { case ("1") { v_QFI := '000001'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '09'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); // @sic R5-204400 sic@ } } case ("2") { v_QFI := '000010'B; v_QoSParametersList := { cs_QoSParameters(tsc_QoSFlowParameter_5QI, '01'O, '05'O)}; if (p_InterworkWithEPS) { v_QoSParametersList[1] := cs_QoSParameters(tsc_QoSFlowParameter_EPSBearerId, '01'O, p_EPSBearerId); } } … … }

## Change 5

| Function name | f_GetTestcaseAttrib_EarlyContentionResolution |
| --- | --- |
| Reason for change | Due to the RRCReject sent at step 16, this test case should use early contention resolution |
| Summary of change | Added the TC to the function for early contention resolution, so the SS is not configured with default rrcSetup for msg4. |
| TTCN module | TestcaseProperties.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } } return false; }

After Change

function f_GetTestcaseAttrib_EarlyContentionResolution(charstring p_Testcase) return boolean { select (p_Testcase) { case ("TC_11_3_2_NR5GC") { return true; } case ("TC_11_3_6_NR5GC") { return true; } case ("TC_6_4_1_1_NR5GC") { return true; } case ("TC_6_4_2_2_NR5GC") { return true; } case ("TC_7_1_1_1_3_NR5GC") { return true; } case ("TC_7_1_1_1_5_NR5GC") { return true; } case ("TC_8_1_1_1_1_NR5GC") { return true; } case ("TC_8_1_1_2_1_NR5GC") { return true; } case ("TC_8_1_1_2_3_NR5GC") { return true; } case ("TC_8_1_1_4_1_NR5GC") { return true; } case ("TC_8_1_1_4_2_NR5GC") { return true; } case ("TC_8_1_2_1_4_NR5GC") { return true; } case ("TC_8_1_3_1_23_NR5GC") { return true; } case ("TC_8_1_4_1_5_NR5GC") { return true; } case ("TC_8_1_4_1_6_NR5GC") { return true; } case ("TC_8_1_5_6_1_NR5GC") { return true; } case ("TC_8_1_5_8_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_1_NR5GC") { return true; } case ("TC_8_1_5_8_2_2_NR5GC") { return true; } case ("TC_8_1_5_8_2_3_NR5GC") { return true; } case ("TC_8_1_6_1_3_3_NR5GC") { return true; } case ("TC_8_1_6_1_4_1_NR5GC") { return true; } case ("TC_8_1_6_1_4_2_NR5GC") { return true; } case ("TC_8_1_6_1_4_4_NR5GC") { return true; } } return false; }