A3GPP TSG-RAN5 Meeting #2021-TTCN email	R5s210758

# Online, , 7th Dec 2020 - 31st Dec 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-3 | CR | 1816 | rev | - | Current version: | 16.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to EN-DC RRC CA test case 8.2.4.3.1.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-07-02 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | f_TC_8_2_4_3_1_Common_ENDC_EUTRA function calls the non-CA Init function for CA test cases |  |  |  |  |  |  |  |  |
| Summary of change: |  | Replaced the call of f_EUTRA38_Init function with f_EUTRA38_Init_CA function |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant UE can fail this test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.3.1.1.ENDC, 8.2.4.3.1.3.ENDC |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Table of Contents

Table of Contents	2

1	Overview	3

2	Corrections required	3

2.1	Change 1	3

# Overview

This document lists all the changes needed to correct issues in the ATS iWD-TTCN3-B2020-09_D21wk24 related to the title of this CR.

Contact:	Shaun Harry
	shaun.harry@keysight.com

# Corrections required

## 2.1 Change 1

| Function name | f_TC_8_2_4_3_1_Common_ENDC_EUTRA |
| --- | --- |
| Reason for change | f_TC_8_2_4_3_1_Common_ENDC_EUTRA function calls the non-CA Init function for CA test cases |
| Summary of change | Replaced the call of f_EUTRA38_Init function with f_EUTRA38_Init_CA function |
| TTCN module | RRC_Measurement_CA_ENDC_EUTRA.ttcn |
| MCC160 Comment | Accepted |

Before Change

function f_TC_8_2_4_3_1_Common_ENDC_EUTRA() runs on EUTRA_5GS_PTC { //NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band Contiguous CA f_EUTRA38_Init(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -79); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1, clause 4.5.4 f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_SCG, RRC_CONNECTED); f_EUTRA_SetCellPowerList({cs_CellPower(eutra_Cell1, -79)}); //@sic R5s201309 sic@ f_AdjustCellPowerForTestingFR2(eutra_Cell1); //@sic R5s201309 sic@ f_EUTRA_TestBody_Set(true); fl_TC_8_2_4_3_1_x_TestBody(); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@sic R5s201309 sic@ f_EUTRA_TestBody_Set(false); //Postamble f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); } // End of f_TC_8_2_4_3_1_1__ENDC_EUTRA()

After Change

function f_TC_8_2_4_3_1_Common_ENDC_EUTRA() runs on EUTRA_5GS_PTC { //NR CA / SCell change / Intra-NR measurement event A6 / SRB3 / EN-DC / Intra-band Contiguous CA f_EUTRA38_Init_CA(EN_DC, c1); //Create and configure cell f_EUTRA_CellConfig_Def(eutra_Cell1); f_EUTRA_CellInfo_InitMaxReferencePower(eutra_Cell1, -79); //The UE is in state RRC_CONNECTED using generic procedure parameter Connectivity (EN-DC) and DC Bearers (MCG and SCG) on E-UTRA Cell 1 according to TS 38.508-1, clause 4.5.4 f_EUTRA38_ENDC_Preamble (eutra_Cell1, TESTMode_OFF, MCG_SCG, RRC_CONNECTED); f_EUTRA_SetCellPowerList({cs_CellPower(eutra_Cell1, -79)}); //@sic R5s201309 sic@ f_AdjustCellPowerForTestingFR2(eutra_Cell1); //@sic R5s201309 sic@ f_EUTRA_TestBody_Set(true); fl_TC_8_2_4_3_1_x_TestBody(); f_EUTRA_NR_WaitForCoOrd_Trigger(NR); //@sic R5s201309 sic@ f_EUTRA_TestBody_Set(false); //Postamble f_EUTRA38_ENDC_Postamble(eutra_Cell1, E2_CONNECTED); // If finishing in E2_CONNECTED, need to wait until connection released before taking down the NR cell! f_EUTRA_NR_SendCoOrd (NR, cms_EUTRA_NR_Trigger("Postamble")); } // End of f_TC_8_2_4_3_1_1__ENDC_EUTRA()